3GPP TSG-RAN WG4 meeting #94bis-e-Bis	R4-2004026

Electronic Meeting, 20 – 30 April 2020

Agenda Item:	6.11.3.2

Source:	Ericsson

Title:	PDSCH requirements with Rel-16 DMRS sequence

Document for:	Discussion

# 1	Introduction

RAN4#94-e discussed UE demodulation and CSI reporting requirements for NR eMIMO WI and agreed with the way forward [1]. One of the open issues in WF is whether to define PDSCH requirements based on Rel-16 DMRS sequence.

- Whether to define PDSCH requirement based on Rel-16 DMRS enhancement - Option 1: Define one DL test to verify receiver processing of Rel-16 DMRS enhancement - Option 1a: if defined, existing UE performance test cases can be reused or replaced with Rel-16 DMRS configuration without requirements and other test parameters modification - Option 1b: One new test case with test parameters modification - Option 2: Not to define any new PDSCH performance requirement of Rel-16 DMRS enhancement

# 2	Discussion

## 2.1	Rel-16 DMRS sequence

RAN1 introduced new DMRS sequence targeting low PAPR in Rel-16. The difference of Rel-16 from Rel-15 DMRS sequence is the c_init. In Rel-16, c_init depends on CDM group, λ, associated with the DMRS port number p, although Rel-15 DMRS c_init is independent of λ. If we look the c_init formula in detail, c_init formula of Rel-15 and Rel-16 are identical if CDM group λ=0, which corresponds to DRMS port indexes p=1000/1001/1004/1005 in the case of PDSCH DM-RS configuration type 1.

Moreover, if we review the existing Rel-15 PDSCH demodulation requirements, the test cases using CDM group 1 are rank 3 or 4 test cases. If we set new PDSCH demodulation requirements by replacing the DMRS sequence with the Rel-16 sequence, this is only applicable for rank 3 or 4 scenarios which mean it is only applicable UEs with 4 receive antennas.

Observation 1: Rel-15 DMRS sequence and Rel-16 DMRS sequence are identical for DMRS port index 1000/1001/1004/1005. If new UE performance tests are going to be defined by replacing the DMRS sequence, it is eventually applicable for UEs with 4 received antennas.

## 2.2	Simulation results and discussion

Figure 1 shows the PDSCH simulation results with Rel-16 DMRS sequence, where we reused Test 2 in TS38.101-4 Table 5.2.2.1.1-4, which is Rank 2 test with 64QAM 1/2. As we discussed in 2.1, the DMRS sequences are identical if the DMRS port index is 1000 or 1001. Therefore we set the DMRS port indexes to 1000/1002. We also note that we don’t consider PA non-linearity, e.g., clipping, on BS part as it is the assumption in RAN4 UE demodulation requirements. From the simulation results, we don’t see any performance difference between Rel-15 DMRS sequence and Rel-16 DMRS sequence.

Observation 2: No performance difference between Rel-15 DMRS sequence and Rel-16 DMRS sequence.

Figure 1	Comparison between Rel-15 DMRS sequence and Rel-16 DMRS sequence.

# 3	Summary

Observation 1: Rel-15 DMRS sequence and Rel-16 DMRS sequence are identical for DMRS port index 1000/1001/1004/1005. If new UE performance tests are going to be defined by replacing the DMRS sequence, it is eventually applicable for UEs with 4 received antennas.

Observation 2: No performance difference between Rel-15 DMRS sequence and Rel-16 DMRS sequence.

Based on the observations, we don’t see any motivation to define new PDSCH demodulation requirements with Rel-16 DMRS sequence.

Proposal: Not to define any new PDSCH performance requirement of Rel-16 DMRS enhancement.

# References

- R4-2002419, “Way forward on demodulation and CSI requirement for NR eMIMO​”, Samsung.

# Appendix

The UE shall assume the sequence is defined by . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231 where is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 or 1_2 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI - NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI; - NIDnSCIDλ=NIDcell otherwise; - nSCIDλ and λ are given by - if the higher-layer parameter dmrsDownlink-r16 in the DMRS-DownlinkConfig IE is provided nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1 λ=λ where λ is the CDM group defined in clause 7.4.1.1.2. - otherwise by nSCIDλ=nSCID λ=0 The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1 or 1_2 in [4, TS 38.212] is used, otherwise nSCID=0.