# 3GPP TSG-RAN WG4 Meeting #94bis-e                                                       R4-2004173

# Electronic Meeting, April 20-24, 2020

Agenda Item:	6.5.2.1.3

Source: 	Ericsson

Title:	TP to TS 38.174: Conducted RX spurious

Document for:	Approval

- Introduction

Section 2 proivde some reasoning on the text proposal and Section 3 provides text proposal on chapter 7 10 for Rx spurious for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Disucssion

The text proposal is based on the WF [1] and WF [2]. As the general description need to be modified to reflect the IAB-MT receiver, thus direct reference is not possible. Addtionaly, the Fix Scaling Factor (FSF) for the IAB-MT cell need to be defined so the same approach to scale the spurious requirement for BS 1-H type[3] could be reused. As the maximum layer/streams supported by IAB-DU and IAB-MT as a pair of the transmitter and receiver, we think it is ok to inherited the FSF of 8 for IAB-DU receiver to IAB-MU receiver.

Another point is that Ncells parameter need to be redefined to IAB-MT as IAB-DU and IAB-MT may have different number of the connected cells depending how the IAB-MT and IAB-DU is configured. Thus we propose the define the Ncell_MT for IAB-MT with modification (in red) of the Ncells for IAB-DU as below:

The minimum number of supported geographical cells (Ncells_MT) relates to the IAB-MT setting with the minimum amount of parent IAB cell splitting supported with transmission on all TAB connectors supporting the operating band, or with minimum amount of transmitted beams.

The suggested text proposal below captured the above proposals.

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[6]	3GPP TS 38.133: “NR: Requirements for support of radio resource management”

[7]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

## 7.6	Receiver spurious emissions

7.6.1 General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at at the TAB connector (for IAB type 1-H). The requirements apply to all IAB with separate RX and TX TAB connectors.

For TAB connectors supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period.

For IAB type 1-H manufacturer shall declare TAB connector RX min cell groups. Every TAB connector of IAB type 1-H supporting reception in an operating band shall map to one TAB connector RX min cell group, where mapping of TAB connectors to cells/beams is implementation dependent.

The number of active receiver units that are considered when calculating the conducted RX spurious emission limits (NRXU,counted) for IAB-DU for IAB type 1-H is calculated as follows:

NRXU,counted = min(NRXU,active , 8 × Ncells)

The number of active receiver units that are considered when calculating the conducted RX spurious emission limits (NRXU,counted) for IAB-MT for IAB type 1-H is calculated as follows:

NRXU,counted = min(NRXU,active , [8] × Ncells_MT)

NRXU,countedpercell is used for scaling of basic limits and is derived as NRXU,countedpercell = NRXU,counted / Ncells for IAB-DU and NRXU,countedpercell = NRXU,counted / Ncells_MT for IAB-MT, where Ncells is defined in TS38.104[2] in clause 6.1 and Ncells_MT is defined as below

The minimum number of supported geographical cells (Ncells_MT) relates to the IAB-MT setting with the minimum amount of parent IAB cell splitting supported with transmission on all TAB connectors supporting the operating band, or with minimum amount of transmitted beams.

NOTE:	NRXU,active is the number of actually active receiver units and is independent to the declaration of Ncells or Ncells_MT.

### 7.6.2. IAB-DU receiver spurious emissions

#### 7.6.2.1 Minimum requirement for IAB type 1-H

Minimum requirement of IAB-DU is specified as the same as BS type 1-H in TS 38.104[2], subclause 7.6.4.

### 7.6.3. IAB-MT receiver spurious emissions

.

### 7.6.3.1	Basic limits

The receiver spurious emissions basic limits are provided in table 7.6.2-1.

Table 7.6.2-1: General IAB-MT receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [7], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [7], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the IAB-MT transmitter operating band to ΔfOBUE above the highest frequency of the IAB-MT transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. |  |  |  |

## 7.6.3.2  	Minimum requirement for IAB type 1-H

The RX spurious emissions requirements for IAB type 1-H are that for each applicable basic limit specified in table 7.6.3.1-1 for each TAB connector RX min cell group, the power sum of emissions at respective TAB connectors shall not exceed the IAB limits specified as the basic limits + X, where X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.

The RX spurious emission requirements are applied per the TAB connector RX min cell group for all the configurations supported by the IAB-MT.

NOTE:	Conformance to the IAB-MT receiver spurious emissions requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the spurious emissions power measured on each TAB connector in the TAB connector RX min cell group shall be less than or equal to the IAB-MT limit above for the respective frequency span.

Or

2)	The spurious emissions power at each TAB connector shall be less than or equal to the IAB-MT limit as defined above for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector RX min cell group.

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002484, WF on TS referencing rules, Nokia

- R4-2002483, WF on IAB RX RF requirement, Samsung

- 37.843, RF requirement background for AAS BS radiated requirements.