3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004251

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	6.5.3.3

Source: 	Huawei, HiSilicon

Title: 	TP to TS 38.174 on timing advance requirement for IAB-MT

Document for:	Approval

Introduction

It was agreed to introduce transmit timing requirement for IAB MT in previous RAN4 meetings. The text proposal on timing advance requirement for IAB MT to TS 37.174 [1] is presented in this paper.

Text proposal

Text proposal to TS 37.174

<START OF TP>

12.2.2	IAB-MT Timing advance

Editor notes: The terminology of “downlink” and “uplink” could be revised and aligned with RF conclusion.

12.2.2.1	Introduction

The timing advance is initiated from gNB and IAB-DU to IAB-MT with MAC message that implies and adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [TBD].

12.2.2.2	Requirements

12.2.2.2.1	Timing Advance adjustment delay

IAB-MT shall adjust the timing of its uplink transmission timing at time slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [TBD]. The same requirement applies also when the IAB-MT is not able to transmit a configured uplink transmission due to the channel assessment procedure.

12.2.2.2.2	Timing Advance adjustment accuracy

The IAB-MT shall adjust the timing of its transmissions with a relative accuracy better than or equal to the IAB-MT Timing Advance adjustment accuracy requirement in Table 12.2.2.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [TBD].

Table 12.2.2.2.2-1: IAB-MT Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| IAB-MT Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

<END OF TP>

Reference

[1] TS 37.174