3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004314

Online, 20th – 30th April, 2020

Title: 	Simulation assumptions of PSBCH-RSRP measurement evaluation for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	6.4.5.2

Document for:	Approval

Introduction

In this contribution, we provided the simulation assumptions of PSBCH-RSRP measurements evaluation, which can be used as basis for performing PSBCH-RSRP measurement accuracy studies.

Discussion

Simulation Assumption

Table 2.1-1 shows the simulation assumptions for evaluating PSBCH-RSRP measurements performance.

Table 1: Simulation assumptions for PSBCH-RSRP measurement performance

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode | - | TDD |
| Carrier frequency | GHz | 5.9 |
| Subcarrier spacing | kHz | 15, 30, 60, |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| S-SSB bandwidth | RB | 11 |
| Utilization of non S-SSB slots for data transmission | % | 100 |
| Relative PSD on non S-SSB slots relative to SLSS slots | dB | 0 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Number of transmitted S-SSB within a S-SSB period | - | 1 |
| S-SSB period | ms | 160 |
| L1 measurement period | ms | 320ms |
| L3 filtering | - | Disabled |
| Number of Tx antennas | - | 1 |
| Number of Rx antennas |  | 2 |
| Propagation Condition |  | AWGN, TDL- C with 100ns, 300Hz TDL- C with 100ns, 150Hz |
| PSBCH_Es/Ioc | dB | -6dB, -3dB, 0dB |
| Ioc Model | - | AWGN noise |
| Note 1: AWGN is a mandatory propagation conditions. The company is also encouraged to submit the simulation with other propagation condition. |  |  |

Performance metrics

The following performance metrics can be considered

- The CDF curves (including the 95% and 5% CDF points) for delta PSBCH-RSRP.

Delta PSBCH-RSRP is defined as follows:

Delta PSBCH-RSRP = Measured PSBCH-RSRP – Ideal PSBCH-RSRP.

Ideal PSBCH-RSRP is defined as the true value assuming perfect channel estimation using the same sampling rate and sampling occasions as used in the measured PSBCH-RSRP evaluation.

Conclusions

In this paper, we provided simulation assumptions for PSBCH-RSRP measurements evaluation. Based on the simulation assumption, we encourage companies to provide simulation results in RAN4#96 meeting.

Reference

- R4-2002230, “WF on NR V2X RRM requirement”, LG Electronics, MediaTek