3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004317

Online, 20th – 30th April, 2020

Title: 	Simulation results of PSSCH-RSRP and PSCCH-RSRP measurements for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	6.4.5.3

Document for:	Discussion

Introduction

RAN4 needs to study SL L1-RSRP measurement requirements for UE sensing measurements. Based on the simulation assumptions agreed in [1], we provide the simulation results of PSSCH-RSRP and PSCCH-RSRP measurement performance in NR V2X.

Discussion

In this section, we provide the simulation results of SL L1-RSRP measurement accuracy with PSSCH-DMRS based and PSCCH-DMRS based. The results in Tables 1-2 show 5%, 50% and 95% CDF of the delta measurements of SL L1-RSRP.

Table 1: Delta measurements of measured SL L1-RSRP for 10RBs

| SCS=15kHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation condition | SINR | PSSCH-DMRS based |  |  |  |  | PSCCH-DMRS based |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | -6dB | -3.13 | -0.28 | 1.66 | 2.85 | 1.94 | -2.16 | 0.66 | 2.71 | 2.82 | 2.05 |
|  | -3dB | -1.89 | -0.21 | 1.11 | 1.68 | 1.32 | -1.27 | 0.37 | 1.86 | 1.65 | 1.49 |
|  | 0dB | -1.30 | -0.22 | 0.71 | 1.08 | 0.93 | -0.92 | 0.21 | 1.28 | 1.13 | 1.07 |
|  | 3dB | -0.92 | -0.20 | 0.44 | 0.72 | 0.64 | -0.70 | 0.08 | 0.82 | 0.79 | 0.74 |
| TDL-C 150Hz | -6dB | -3.87 | -0.24 | 2.14 | 3.63 | 2.38 | -3.18 | -0.07 | 2.70 | 3.11 | 2.77 |
|  | -3dB | -2.56 | -0.23 | 1.47 | 2.33 | 1.71 | -2.43 | -0.18 | 1.71 | 2.25 | 1.89 |
|  | 0dB | -1.69 | -0.22 | 0.92 | 1.47 | 1.14 | -1.67 | -0.17 | 1.19 | 1.50 | 1.36 |
|  | 3dB | -1.15 | -0.22 | 0.57 | 0.93 | 0.79 | -1.25 | -0.20 | 0.72 | 1.05 | 0.92 |
| TDL-C 300Hz | -6dB | -4.06 | -0.23 | 2.12 | 3.83 | 2.36 | -3.55 | -0.11 | 2.67 | 3.43 | 2.79 |
|  | -3dB | -2.46 | -0.25 | 1.35 | 2.22 | 1.60 | -2.31 | -0.14 | 1.77 | 2.17 | 1.91 |
|  | 0dB | -1.65 | -0.24 | 0.86 | 1.41 | 1.10 | -1.73 | -0.18 | 1.19 | 1.55 | 1.37 |
|  | 3dB | -1.10 | -0.22 | 0.56 | 0.88 | 0.78 | -1.23 | -0.20 | 0.75 | 1.04 | 0.94 |
| SCS=30KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | SINR | PSSCH-DMRS based |  |  |  |  | PSCCH-DMRS based |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | -6dB | -3.15 | -0.27 | 1.68 | 2.89 | 1.94 | -2.01 | 0.67 | 2.73 | 2.68 | 2.07 |
|  | -3dB | -1.93 | -0.22 | 1.07 | 1.71 | 1.29 | -1.34 | 0.40 | 1.82 | 1.74 | 1.42 |
|  | 0dB | -1.25 | -0.20 | 0.70 | 1.04 | 0.90 | -0.89 | 0.22 | 1.26 | 1.11 | 1.04 |
|  | 3dB | -0.95 | -0.22 | 0.43 | 0.73 | 0.65 | -0.68 | 0.08 | 0.85 | 0.76 | 0.77 |
| TDL-C 150Hz | -6dB | -4.17 | -0.24 | 2.08 | 3.93 | 2.32 | -3.48 | -0.05 | 2.53 | 3.42 | 2.58 |
|  | -3dB | -2.48 | -0.26 | 1.37 | 2.22 | 1.63 | -2.33 | -0.14 | 1.76 | 2.20 | 1.89 |
|  | 0dB | -1.66 | -0.22 | 0.89 | 1.45 | 1.10 | -1.60 | -0.17 | 1.14 | 1.43 | 1.31 |
|  | 3dB | -1.10 | -0.22 | 0.59 | 0.88 | 0.81 | -1.19 | -0.19 | 0.73 | 0.99 | 0.93 |
| TDL-C 300Hz | -6dB | -4.21 | -0.24 | 2.08 | 3.96 | 2.32 | -3.33 | -0.08 | 2.61 | 3.25 | 2.68 |
|  | -3dB | -2.49 | -0.22 | 1.39 | 2.26 | 1.61 | -2.48 | -0.14 | 1.72 | 2.33 | 1.86 |
|  | 0dB | -1.64 | -0.22 | 0.90 | 1.42 | 1.12 | -1.70 | -0.17 | 1.12 | 1.53 | 1.30 |
|  | 3dB | -1.09 | -0.23 | 0.55 | 0.86 | 0.78 | -1.21 | -0.10 | 0.74 | 1.01 | 0.95 |
| SCS= 60KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | SINR | PSSCH-DMRS based |  |  |  |  | PSCCH-DMRS based |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | -6dB | -3.11 | -0.29 | 1.68 | 2.82 | 1.97 | -2.15 | 0.67 | 2.67 | 2.83 | 2.00 |
|  | -3dB | -1.87 | -0.24 | 1.09 | 1.63 | 1.32 | -1.32 | 0.38 | 1.83 | 1.70 | 1.44 |
|  | 0dB | -1.27 | -0.23 | -072 | 1.05 | 0.95 | -0.90 | 0.21 | 1.25 | 1.11 | 1.04 |
|  | 3dB | -0.92 | -0.11 | 0.46 | 0.70 | 0.68 | -0.70 | 0.10 | 0.85 | 0.79 | 0.75 |
| TDL-C 150Hz | -6dB | -4.02 | -0.33 | 1.87 | 3.69 | 2.20 | -3.26 | -0.09 | 2.43 | 3.17 | 2.52 |
|  | -3dB | -2.50 | -0.34 | 1.17 | 2.17 | 1.50 | -2.32 | -0.19 | 1.51 | 2.13 | 1.71 |
|  | 0dB | -1.61 | -0.33 | 0.72 | 1.29 | 1.04 | -1.76 | -0.24 | 0.97 | 1.52 | 1.20 |
|  | 3dB | -1.27 | -0.34 | 0.41 | 0.93 | 0.75 | -1.26 | -0.25 | 0.64 | 1.01 | 0.89 |
| TDL-C 300Hz | -6dB | -4.02 | -0.35 | 1.81 | 3.66 | 2.16 | -3.39 | -0.11 | 2.44 | 3.09 | 2.54 |
|  | -3dB | -2.44 | -0.36 | 1.14 | 2.08 | 1.51 | -2.39 | -0.24 | 1.50 | 2.17 | 1.74 |
|  | 0dB | -1.70 | -0.33 | 0.72 | 1.36 | 1.06 | -1.73 | -0.28 | 0.98 | 1.45 | 1.25 |
|  | 3dB | -1.25 | -0.33 | 0.41 | 0.91 | 0.74 | -1.30 | -0.27 | 0.59 | 1.03 | 0.87 |

Summary

This contribution provides the simulation results of PSSCH-RSRP and PSCCH-RSRP measurement performance for NR V2X. It is suggested to consider the above PSSCH-RSRP and PSCCH-RSRP measurement results when defining SL L1-RSRP measurement accuracy requirements.

Reference

- R4-2002232, “Simulation assumption of PSSCH-RSRP and PSCCH-RSRP measurement”, LG Electronics