3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004325

Online, 20th – 30th April, 2020

Title: 	Further discussion on SCell BFR measurements for NR eMIMO

Source: 	Huawei, HiSilicon

Agenda item:	6.11.2.2

Document for:	Discussion

Introduction

In RAN4#94-e meeting, the RRM requirements for SCell BFR have been discussed and the progress has been captured in [1]. In this contribution, we provide discussion on the remaining issues for SCell BFR requirements.

Discussion

BFD on SCell

In RAN4, the followings agreements have been achieved for BFD on multiple serving cells

- Necessity of BFD procedure on multiple serving cell on the same FR2 band - No restriction introduced in RAN4. No RAN4 performance requirement will be defined for performing BFD measurement on more than 1 serving cell. - RS within a deactivated SCC is implicitly configured as the BFD-RS for another activated SCell: - Sharing factor for BFD Time Period for FR1 and FR2 - No BFD evaluation period will be specified for performing BFD measurement on more than 1 serving cell per band: - Sharing factor for BFD evaluation period due to BFD over multiple Sells will not be introduced for FR2 intra-band CA - No RAN4 performance requirement will be defined for more than 1 serving cell per band perform BFD measurement - FFS Sharing factor for BFD evaluation period for FR1 CA, FR2 inter-band CA and FR1-FR2 CA.

It has been agreed to define BFD evaluation period for the case that UE performs BFD measurements for one serving cell per band. The number of serving cells for BFD measurements equals to the number of NR bands configured for inter-band CA or inter-band NR-DC. The serving cell for BFD measurements could be PCell, PSCell or SCell. The existing BFD measurements requirements on PCell/PSCell shall be not impacted by BFD measurements on SCell(s). The sharing factor for BFD measurements on PCell/PSCell can be defined as 1. The BFD measurement opportunities can be shared among SCells for BFR.

For NR inter-band CA, it can be assumed that UE is able to process the BFD measurements simultaneously on one PCell/PSCell and one SCell for BFR. The number of SCells for BFR is equal to the number of configured bands - 1. It can assumed that all the SCells for BFR are equally sharing the measurement opportunities, and the sharing factor for BFD measurements on SCell can be defined as the number of configured bands - 1.

According to TS38.101, currently only two bands are configured for inter-band NR-DC. So, UE is only required to perform BFD measurements on PCell and SCell, and there will be no SCell for BFR. If NR-DC configured with three or more bands in future, it can be assumed that UE is able to process the BFD measurements simultaneously on one PCell, one PSCell and one SCell for BFR. The number of SCells for BFR is equal to the number of configured bands - 2. It can assumed that all the SCells for BFR are equally sharing the measurement opportunities, and the sharing factor for BFD measurements on SCell can be defined as the number of configured bands - 2.

Proposal 1: The scaling factor for BFD evaluation period for inter-band CA/DC can be defined as Table 1.

Table 1: The sharing factor for BFD evaluation period

| Scenario | PCell | PSCell | SCell for BFR Note |
| --- | --- | --- | --- |
| EN-DC with NR inter-band CA | N/A | 1 | Number of configured NR band(s) -1 |
| NR only inter-band CA | 1 | N/A | Number of configured NR band(s) -1 |
| NE-DC with NR inter-band CA | 1 | N/A | Number of configured NR band(s) -1 |
| NR-DC configured with ≥3 bands | 1 | 1 | Number of configured NR band(s) -2 |

CBD on SCell

In RAN4, the followings agreements have been achieved for CBD on multiple serving cells

- Sharing factor for CBD Time Period - No CBD evaluation period will be specified for performing CBD measurement on more than 1 serving cell per band: - Sharing factor for CBD evaluation period (due to CBD over multiple SCells) will not be introduced for FR2 intra-band CA. - No RAN4 performance requirement will be defined for more than 1 serving cell perform CBD measurement. - FFS Sharing factor for CBD evaluation period for FR1 CA, FR2 inter-band CA and FR1-FR2 CA.

Similarly as BFD measurements, UE is only required to perform CBD measurements for one serving cell per band. In RAN1, it has been agreed that downlink RS for CBD can be based on SSB or CSI-RS for BM and the total number of RSs for CBD and L1-RSRP measurement are part of UE capability signalling. For CBD measurement on SCell, we suggest to reuse the assumption for L1-RSRP measurement on SCell. Then, there is no need to define the sharing factor for CBD evaluation period due to CBD over multiple SCells.

Proposal 2: It is suggested not to introduce sharing factor for CBD evaluation period requirements due to CBD over multiple SCells.

UE Capability of Number of SCells for BFR

RAN1 has the following agreements on the configuration of CBD-RS for SCell BFR.

RAN1 Agreement - The new beam RS is mandatorily configured if SCell BFR is configured

In RAN1, It can be observed that CBD RS resource shall be mandatorily configured to a SCell for BFR. So, a SCell without configuring CBR-RS resources can be considered not to perform BFR procedure.

Proposal 3: It is suggested that UE is not required to perform BFD and CBD for a SCell which is not configured with CBD-RS resources.

Conclusions

This contribution provides our analysis on SCell BFR requirements for NR eMIMO. The following are provided:

Proposal 1: The scaling factor for BFD evaluation period for inter-band CA/DC can be defined as Table 1.

Table 1: The sharing factor for BFD evaluation period

| Scenario | PCell | PSCell | SCell for BFR Note |
| --- | --- | --- | --- |
| EN-DC with NR inter-band CA | N/A | 1 | Number of configured NR band(s) -1 |
| NR only inter-band CA | 1 | N/A | Number of configured NR band(s) -1 |
| NE-DC with NR inter-band CA | 1 | N/A | Number of configured NR band(s) -1 |
| NR-DC configured with ≥3 bands | 1 | 1 | Number of configured NR band(s) -1 |

Proposal 2: It is suggested not to introduce sharing factor for CBD evaluation period requirements due to CBD over multiple SCells.

Proposal 3: It is suggested that UE is not required to perform BFD and CBD for a SCell which is not configured with CBD-RS resources.

Reference

- R4-2002182, “WF on NR eMIMO RRM requirements”, Samsung