3GPP TSG-RAN WG4> Meeting # 94-e-bis	R4-2004463

# Electronic Meeting,  20 - 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: Correcting the reference angular step equations (Annex F.2.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 20-4-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The upper limit of 15 degrees is missing in the reference angular step equations for ULA in Annex F.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The upper limit of 15 degrees is added to the reference angular step equations for ULA in Annex F.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A mistake exists in the specification and the equations are not aligned with those for non-ULA and in TR 37.843. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-2. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change >

## F.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ, in degrees, are defined as:

Δθref=⁡min(180∘πλD,15∘)

Δϕref= min⁡(180∘πλDcyl,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the DUT dimensions.

Dcyl and D are calculated as:

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure F.2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of a Uniform Linear Array (ULA), when d is negligible (d ≈0) and when the EUT is mounted along the yz plane as shown in figure F.2.2-2, the reference angular step, in degrees, can be determined by

∆θref=arcsin⁡(λ/Dz)

Δϕref=arcsin⁡(λDy)

∆θref=min(180°πarcsin⁡(λDz),15°)

Δϕref=min(180°πarcsin⁡(λDy),15°)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUTalong the z-axis and  is wavelength for the measured frequency.

Figure F.2.2-2. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as:

SF=max⁡(ΔθgridΔθref,ΔϕgridΔϕref)

Where Δθ and  Δϕ are the actual angular steps used in the measurement.

Alternatively, when the EUT radiating dimensions are not known. For each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA AAS BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

<End of change>