3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004499

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	9.2.6.3

Source: 	Huawei

Title: 	TP to TR 38.820: ACS

Document for:	Approval

- Introduction

The BS requirements section for the TR is almost complete however it has been noticed that ACS is missing.

In this contribution we provide a TP on ACS based on the discussion paper [1].

References

[1] R4-1910849, Receiver ACS for 7 to 24GHz frequency range, Huawei

[2] TR 38.820

3.	TP to TR 38.820

< START OF CHANGE>

#### 7.4.2.8	RX ACS

For the uplink the following values of BS ACS and UE ACLR are currently used for FR1 and FR2 bands.

|  |  | FR1 | FR2 |  |
| --- | --- | --- | --- | --- |
|  |  | 2 GHz | 24 to 33 GHz | 37 to 52 GHz |
| ACS | dB | 45 | 24 | 23 |
| ACLR | dB | 31 | 17 | 16 |
| ACIR | dB | 30.83 | 16.21 | 15.21 |

To get accurate estimations on the ACIR for the range 7 to 24GHz the co-existence simulations would need to be carried out. It will be done when the bands are allocated. But it is no doubtful that the ACIR value would be somewhere between 31 and 17 dB. The split between UE ACLR and BS ACS should also be decided on a band specific basis based on the relative difficulty in implementing each in the BS and UE respectively.

Hence we can conclude that the ACS value for the range 7 to 24 GHz will be in the range 45 dB to 24 dB. The exact requirement need to be decided by co-existence simulations when the bands are allocated.

< END OF CHANGE>