3GPP TSG-RAN WG4 Meeting #94bis-e 	R4-2004546

Online, 20 Apr - 01 May 2020

Source: 	Huawei

Title: 	TP to TS 38.174: IAB class definitions

Agenda Item:	6.5.1.2

Document for:	Approval

# Introduction

This TP contribute the BS class definitions

The IAB-DU has been agreed to use the BS class definitions however there is an issue with the wording for example:

Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

This needs to be applied to a IAB-DU, but an IAB-DU does not have associated UE’s and hence IAB-DU to UE distance is confusing. However the requirements are of course derived from eth BS-UE scenario as they are copied from the BS requirements. The correct wording is hence:

Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

nt.

# TP to TS 38.174 v0.0.1

--- Start of changes ---

## 4.4	IAB classes

## 4.4.1	IAB-DU classes

The requirements in this specification apply to Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-DU with and without connectors.

For IAB-DU type 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

For IAB-DU type 1-H, BS classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

### 4.4.1	IAB-MT classes

The requirements in this specification apply to Wide Area IAB-MT and [Local Area IAB-MT] unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-MT with and without connectors.

For IAB-MT type 1-H, 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area IAB-MT are characterised by requirements derived from Macro Cell scenarios with a IAB-DU to IAB-MT minimum coupling loss equal to [95] dB.

-	[Local Area IAB-MT] are characterised by requirements derived from FFS

--- End of changes ---