3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004793

Electronic Meeting, 20 – 30 April 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 37.941: Coordinate system

Agenda item:	6.19.3

Document for:	Approval

# 1	Introduction

This document provides a TP to TR 37.942 for the coordinate system based on the discussions in [1].

# 2 	References

- R4-2004525, Discussions on the spherical coordinate system, Nokia, Nokia Shanghai Bell

- R4-2001807, Skeleton for TR 37.941 on OTA BS testing, Rel-15, Huawei

# 3	TP to the External TR on OTA BS testing

TR 37.941

--------------Start of text proposal-------------

# 4	General

## 4.1	Coordinate system

OTA requirements are stated in terms of electromagnetic and spatial parameters. The electromagnetic parameters are specified either in terms of power (dBm) or field strength (dBV/m). The spatial parameters are specified in a Cartesian coordinate system (x, y, z) with an addition of spherical coordinates (r,   for manufacturer declared parameters.

The orientation of these coordinates is depicted in the following figures. is the angle in the x/y plane and it is between the x-axis and the projection of the vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the vector and is defined between -90° and +90°, inclusive.

A point in the Cartesian coordinate system (x,y,z) can be transformed to spherical coordinates (r,   using the following relationships:

x=rcosθcosϕ

y=rcosθsinϕ

z=-rsinθ

The inverse transformation is given by:

θ=-arcsinzx2+y2+z2

The representation and definitions of angles are described in figure 4.1-1, figure 4.1-2 and figure 4.1-3.

Figure 4.1-1: Orthogonal representation of coordinate system

Figure 4.1-2: Definition of the  angle

Figure 4.1-3: Definition of  angle

For RF core requirements the vendor declares the location of this coordinate system origin in reference to an identifiable physical feature of the AAS BS enclosure. The vendor also declares the orientation of this coordinate system in reference to an identifiable physical feature of the AAS BS enclosure. However, for background information in this TR also other conventions for spatial angles may be used, e.g. definitions used in IEEE Standard Test Procedures for Antennas [7].

An alternative definition of the spatial angles is shown in figure 4.1-4, figure 4.1-5 and figure 4.1-6.

Figure 4.1-4: Alternative orthogonal representation of coordinate system

Figure 4.1-5: Alternative definition of the  angle

Figure 4.1-6: Alternative definition of  angle

--------------end of text proposal-------------