3GPP TSG-RAN WG4 Meeting #94bis	R4-2004807

E-meeting, 20 April – 1 May, 2020

Agenda item:	6.5.3.2

Source: 	Qualcomm Incorporated

Title: 	RRC Connection Mobility Control in IAB Networks

Document for:	Discussion

# Introduction

During the last meeting [1], RAN4 agreed to the following requirement:

Agreements IAB MT is not required to successfully identify a cell when the SMTC window is larger than 160ms and the serving cell is in low SNR. Agreements There is no requirement for RRC re-establishment for IAB-MTs if the SSB transmission periodicity is larger than 160 ms. Agreements There is no requirement for RRC release with re-direction when the periodicity of SSB is greater than 160 ms. Conclusion: No consensus of mentioning that up to four SMTC windows can be configured per frequency layer for an IAB MT.

During the last meeting, several companies wondered why higher number of SMTC windows need to be mentioned while describing core requirements of IAB-MTs. We focus on that aspect in this meeting.

# Mention of number of SMTC windows in Rel-15 Requirements

Rel-15 allowed network to configure up to two SMTC windows for each UE. This has been captured in RRC re-establishment delay requirement in the following way.

Text of 38.133 TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH …………………………………….. TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

Observation 1: Rel-15 allowed network to configure up to two SMTC windows to each UE in intra-frequency.

Observation 2: The section describing the core requirements for RRC re-establishment delay mentions the number of SMTC windows that can be configured for the UEs.

This configuration of additional number of SMTC windows needs to be captured in RAN4 RRC Re-establishment delay requirements. The remaining part of the existing requirements can be reused for IAB nodes.

# Support of higher number of SMTC windows in IAB networks

Rel-16 has allowed network to configure up to four SMTC windows for each IAB MT node [2].

Agreements: - For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4.

RAN1 is also discussing to generate an IAB-MT capability regarding this feature [3]

| Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities | Yes | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |

Observation 3: Rel-16 allows network to configure up to four SMTC windows to each IAB MT node.

Observation 4: RAN1 is discussing to generate an IAB-MT capability to support up to 4 SMTC per frequency layer.

# Inclusion of higher number SMTC

The RRC re-establishment requirements for UEs mention the number of SMTCs that can be configured for UEs. Similarly, the RRC re-establishment requirements for IAB-MTs should mention the number of SMTCs that can be configured for IAB-MTs in each frequency layer.

- Proposal 1: The description of TSMTC in RRC re-establishment delay requirement should mention that up to four SMTC windows can be configured per frequency layer for an IAB MT.

  - The following example shows how the existing text, that discusses the periodicity of SMTC for UEs in Rel-15, can be modified to allow up to four SMTC windows per frequency layer for IAB MTs.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH …………………………………….. TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 signaling of smtc2smtcK where 1<=K<=4, Tsmtc follows smtc1 or smtc2smtcK according to the physical cell ID of the target cell. TSMTC,i,: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If the UE has been provided with higher layer in TS 38.331 signaling of smtciK where 1<=K<=4 for the i-th frequency layer, Tsmtc,i follows smtciK according to the physical cell ID of the target cell. If it is not configured, the UE IAB-MT may assume that the target SSB periodicity is no larger than 20 ms.

# Conclusion

Observation 1: Rel-15 allowed network to configure up to two SMTC windows to each UE in intra-frequency.

Observation 2: The section describing the core requirements for RRC re-establishment delay mentions the number of SMTC windows that can be configured for the UEs.

Observation 3: Rel-16 allows network to configure up to four SMTC windows to each IAB MT node.

Observation 4: RAN1 is discussing to generate an IAB-MT capability to support up to 4 SMTC per frequency layer.

Proposal 1: The description of TSMTC in RRC re-establishment delay requirement should mention that up to four SMTC windows can be configured per frequency layer for an IAB MT.

  - The following example shows how the existing text, that discusses the periodicity of SMTC for UEs in Rel-15, can be modified to allow up to four SMTC windows per frequency layer for IAB MTs.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH …………………………………….. TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 signaling of smtc2smtcK where 1<=K<=4, Tsmtc follows smtc1 or smtc2smtcK according to the physical cell ID of the target cell. TSMTC,i,: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If the UE has been provided with higher layer in TS 38.331 signaling of smtciK where 1<=K<=4 for the i-th frequency layer, Tsmtc,i follows smtciK according to the physical cell ID of the target cell. If it is not configured, the UE IAB-MT may assume that the target SSB periodicity is no larger than 20 ms.

# Reference

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #94-e.

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #98bis.

- R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E