3GPP TSG-RAN WG4 Meeting #94bis-e                                                                                     R4-2004922

Electrical Meeting, 20th April–30th April, 2020

Source:	Samsung

Title:	Discussion and simulation results for PMI requirement with NR Rel-15 type II codebook

Agenda item:			6.18.1.2

Document for:	Approval

- Introduction

In the last RAN4 meeting, the test parameters of PMI requirement for NR Rel-15 type II codebook were discussed. The following options were considered as following in WF [1]:

- Codebook construction for type II codebook - Option 1: 16 Tx ports (N1,N2) =(4,2), (O1,O2)=(4,4) - Option 2: 32 Tx ports (N1,N2)=(4,4), (O1,O2)=(4,4) - Option 3: Option 1 and Option 2 - Other options not precluded - Channel model for type II codebook - Option 1: TDLA30-5 - Use option 1 as baseline, and not preclude other options - MIMO correlation for type II codebook - Option 1 : XP High - Option 2: XP medium - Other options are not precluded - Beam steering model for type II codebook - Option 1: Reusing beam steering approach with dual-cluster beams as specified in B.2.3B.4A of TS 36.101 - Relative power ratio among two beams can be fixed as 1 (p=1) - Use option 1 as baseline, and not preclude medication/other option for coming meetings - Other options not precluded - Test metric for type II codebook - Option 1: TP ratio between following PMI with type II codebook and following PMI with type I single-panel codebook - Option 2: TP ratio between following PMI and random PMI - Other options are not precluded

In this contribution, the view on remain issues for PMI requirement based on Rel-15 type II codebook are provided. Based on our preference, the simulation results are provided for alignment purpose.

# 2. Discussion on simulation assumptions

- release independence

In the last meeting, the release independence aspect for NR Rel-16 UE demodulation and CSI reporting requirements is discussed in [4].  Since type II is a UE optional feature with capability signalling, we think it is not feasible release independence for type II codebook requirement.

Proposal 1: It is not feasible release independence for PMI reporting requirements for Rel-15 type II codebook

Rel-15 Type II codebook construction consists of below parameters:

- n1-n2-codebookSubsetRestriction : (N1,N2), (O1, O2), PCSI_RS

  - Total number of antenna ports P = 2N1N2 which supporting 4, 8, 12,16,24 and 32 ports for Type II codebook

  - (N1,N2), (O1,O2) as in table below

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |
| 8 | (2,2) | (4,4) |
|  | (4,1) | (4,1) |
| 12 | (3,2) | (4,4) |
|  | (6,1) | (4,1) |
| 16 | (4,2) | (4,4) |
|  | (8,1) | (4,1) |
| 24 | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,1) |
| 32 | (4,4) | (4,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,1) |

- phaseAlphabeSize:  Npsk={4,8}

- subbandAmpltitude: true or false

- numberOfBeams:  L=2 for PCSI_RS =4, and L ={2,3,4} for 	PCSI_RS > 4

- Number of Ports and Codebook configuration

Regarding number of Tx ports, 4 ~ 32 ports can be supported for Type II codebook. NR Rel-15 Type II codebook is the extension of LTE Rel-14 advanced codebook which they share similar codebook structure. It’s better that we can align the number of Tx ports with PMI test case of LTE eFD-MIMO advanced codebook to provide comparable performance. In LTE Rel-14 eFD-MIMO WI, 16 tx ports was used for PMI test case with advanced codebook.

The test complexity especially the number of required individual MIMO channel faders also needs to be considered when we decide number of Tx ports：

In table 1 below, we summarized number of independent MIMO faders required for different PMI test cases for LTE and on-going Rel-16 eMIMO WI and performance enhancement WI. Compared to LTE, number of faders increased by 4/8 times pending on the decision of the number of Tx ports we selected.

Table 1: Summary of number of MIMO faders under PMI test cases

| Item | Rel-13 FD-MIMO | Rel-14 eFD-MIMO | Rel-15 NR | Rel-16 performance enhancement WI | Rel-16 eMIMO WI |
| --- | --- | --- | --- | --- | --- |
| Test cases | 32x2 Class A PMI test cases | 16x2 Advanced codebook PMI test case with 2 beams | 8x4 Type I PMI test case | 32x4 Type 1 PMI test case 16x4/32x4 Type II codebook with L=2 beams | 16x4/32x4 Type II codebook with L= 4beams |
| Required maximum MIMO faders | 64 faders | 64 faders | 32 faders | 128 faders for Type I 128/256 faders for Type II | 256/512 faders |

Meanwhile, from performance test perspective, both 16 Tx and 32 Tx port PMI requirements for Rel-15 type I single-panel codebook are introduced. As mentioned, type II codebook is similar with LTE eFD MIMO with advanced codebook. In case of requirement of advanced codebook, only 16 Tx port requirements are specified with considering the test complexity.

To compact the test,  in order to take a comparable performance compared to Rel-15 NR Type I test cases, we prefer to use 16 CSI-RS ports with (N1,N2) = (4,2) as a starting point to functionality test. Meanwhile, there is no necessary to replicate the test case to specify requirements with configured both 16 Tx and 32 Tx ports.

Proposal 2:  Only define the PMI requirement of Rel-15 type II codebook with 16 Tx ports (N1,N2) =(4,2),(O1,O2)=(4,4)

- PhaseAlphabeSize and subbandAmpltitude

This parameter is to indicate the phase coefficient with 2L wideband amplitude, where the phase coefficients are quantized to Npsk levels (RRC configured) or 4 levels (fixed). The difference is very minor with configured 4 or 8.

Meanwhile, TDLA30-5 is agreed to specify the type II codebook requirement, the frequency selective is no obvious. The expected gain with subband is minor.

Therefore, we prefer the following codebook construction to finalize the test parameters for PMI requirement.

Proposal 3:  Define the PMI requirement of Rel-15 type II codebook construction as

- phaseAlphabeSize:  Npsk= 4

- subbandAmpltitude: false

- numberOfBeams:  L=2

# 3. Simulation Results

In this subsection, we provide our simulation results for type II codebook:

- High XP correlation

Fig.1 Throughput performance for 16x2 rank1 in XP High correlation

Fig.2 Throughput performance for 16x2 rank2 in XP High correlation

- Medium XP correlation

Fig.3 Throughput performance for 16x2 rank1 in XP Medium correlation

Fig.4 Throughput performance for 16x2 rank2 in XP Medium correlation

The above the simulation results are based on wideband PMI feedback, according to the observations we present that:

- There is almost no gains for Type II vs. Type I under XP High channel correlation model.

- In 16x2 rank1 XP Medium channel correlation model, the gains between Type II and Type I are 1.6dB for 70% of peak throughput and 2.1dB for 90% of peak throughput respectively.

- In 16x2 rank2 XP Medium channel correlation model, gains between Type II and Type I are 1.1dB for 70% of peak throughput and 1.7dB for 90% of peak throughput respectively.

- In our opinion, the propagations from multiple dominant clusters are rich in XP medium correlation (or low correlation) channel, which lead to the dominant eigenvectors can be much better represented by a basis of multiple DFT vectors which exist in Type II codebook.

Fig.5 Subband PMI vs. wideband PMI for Type II in XP Medium correlation under 16x2 rank2

Moreover, two options for Type II test metric are provided in [1]. In our opinion, both type I single panel with large antenna ports and type II codebook for UE requirements are the optional feature with additional signaling. In case of a UE only supports one of them, this UE could not apply the “type II vs. type I metric” for testing.

Based on our result, we prefer to reuse the test metric as the TP ratio between following PMI with type II codebook and random PMI with type II.

The relative throughput metrics as ratio between Type II following PMI and Type II random PMI are collected as the following:

Table 1: TP ratio summary for 64QAM (MCS20) under wideband PMI case

|  | 16x2 rank1 XP High | 16x2 rank2 XP High | 16x2 rank1 XP Medium | 16x2 rank2 XP Medium |
| --- | --- | --- | --- | --- |
| TP ratio of SNR point at 90% throughput | 1461 | 1513.1 | 2.71 | 4.63 |

For XP High correlation, the gap between following PMI and random PMI is so remarkable that it is not suitable to define the TP ratio index. It may be caused by the channel beam energy concentrated in one cluster under the XP High channel state. And then it is difficult to catch the main beam direction for the random PMI cases.

Based on the above observations, we propose that:

Proposal 4:  Prefer to use the test metric with TP ratio between following PMI with type II codebook and random PMI with type II codebook

Proposal 5:  Define the PMI requirement of Rel-15 type II codebook with MCS 20 Rank 2 under XP medium correlation model

Proposal 6: If we choose TP ratio between following PMI and random PMI as the test metric, we’d better to definite wideband PMI feedback, because the SNR point at 90% of peak throughput is difficult to be achieved under the subband random PMI case.

# 4. Conclusion

In this contribution, we provide our views and PMI simulation results for Type II codebook, based on the results, the following proposals are made:

Proposal 1: It is not feasible release independence for PMI reporting requirements for Rel-15 type II codebook

Proposal 2:  Only define the PMI requirement of Rel-15 type II codebook with 16 Tx ports (N1,N2) =(4,2),(O1,O2)=(4,4)

Proposal 3:  Define the PMI requirement of Rel-15 type II codebook construction as

      - phaseAlphabeSize:  Npsk= 4

      - subbandAmpltitude: false

      - numberOfBeams:  L=2

Proposal 4:  Use the test metric with TP ratio between following PMI with type II codebook and random PMI with type II codebook

Proposal 5:  Define the PMI requirement of Rel-15 type II codebook with MCS 20 Rank 2 under XP medium correlation model

Proposal 6: If we choose TP ratio between following PMI and random PMI as the test metric, we’d better to definite wideband PMI feedback, because the SNR point at 90% of peak throughput is difficult to be achieved under the subband random PMI case.

# 5. Reference

[1] R4-2002394,”Simulation assumptions for NR PMI reporting requirements for more than 8Tx ports”, Ericsson, 3GPP RAN4 #94-e

[2] 3GPP TS 36.101 V15.8.0, User equipment (UE) radio transmission and reception (Release 15), 2019-09

[3] R4-2000301, “Initial simulation results for PMI reporting with Type II codebook”, Samsung, 3GPP RAN4 #94-e

[4] R4-2002390, Way forward on release independence aspects for NR Rel-16 UE demodulation and CSI reporting requirements, Huawei

# 5. Annex

Simulation assumption

| Parameter |  | Unit | Test 5.1 | Test 5.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | Medium /High XP 16 x 2 (N1,N2) = (4,2) |  |
| Beamforming Model |  |  | Option 1: As specified in Annex B.2.3B.4A of TS36.101 -Relative power ratio among two beams can be fixed as 1 (p=1) Other options not precluded |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | 5/1 |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,12) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband /Subband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | 5/1 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 |  |
|  | NPSK (phaseAlphabetSize) |  | 4 |  |
|  | subbandAmplitude |  | False |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | FFS |  |
|  | RI Restriction |  | FFS |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | MCS 20 FDD |  |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). . |  |  |  |  |