3GPP TSG-RAN WG4 Meeting #94bis-e                                                  R4-2004926

Electrical Meeting, 24th -30th April, 2020

Source:	Samsung

Title:	Initial simulation results for PMI requirement with Rel-16 enhanced type II codebook

Agenda item:			6.11.3.3

Document for:	Discussion

- Introduction

In RAN4 94-e meeting, for the test scope of NR eMIMO demodulation and CSI requirement was approved in [1], with the following agreements achieved:

Agreement

- Define the PMI reporting requirement for Enhanced of Type II Codebook

- No requirement for enhanced Rel-15 Type II codebook with Rank3/4

Furthermore, for the PMI reporting requirements for NR eMIMO was approved in [2], with the following agreements also achieved:

Agreement

- FFS on the test parameters of Rel-16 type II codebook requirement related number of ports and subband, codebook parameter, beam steering, propagation condition, MCS and Rank, as well as  the test metric

In this contribution, the views on the Rel-16 enhanced codebook are provided. Meanwhile, the initial simulation results are provided to investigate the different test parameters.

# Simulation assumption

- Table 1 Simulation assumptions for Rel-16 enhanced Type II PMI (FDD)

| Parameter |  | Unit | Test FDD 2Rx |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | 16 ports with (N1,N2) = (4,2) and(O1,O2)=(4,4) |
| Beamforming Model |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 12, (2, 4, 6, 8) for 16 ports |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) for 16 ports |
|  | CSI-RS interval and offset | slot | 5/1 |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 with L =4, pν =1/2, β=1/2 4 with L =4, pν =1/4, β=1/2 2 with L=2,pv=1/4,β=1/2 1 with L =4, pν =1/4, β=1/4 |
|  | numberOfPMISubbandsPerCQISubband |  | R = 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) for 16 ports |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) for 16 ports |
|  | n1-n2-codebookSubsetRestriction-r16 |  | FFS |
|  | typeII-RI-Restriction-r16 |  | FFS |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | MCS 20 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Taking beam steering approach as specified in B.2.3B.4A of TS 36.101 as staring point with further extension applicable for L beams |  |  |  |

Table 2: paraCombination_r16 configuration (layer v =1, 2)

| Parameter | paraCombination_r16=1 | paraCombination_r16=2 | paraCombination_r16=4 | paraCombination_r16=6 |
| --- | --- | --- | --- | --- |
| L | 2 | 2 | 4 | 4 |
| pυ | 1/4 | 1/4 | 1/4 | 1/2 |
| β | 1/4 | 1/2 | 1/8 | 1/2 |
| K0 | 2 | 4 | 8 | 16 |

In table 2, K0 represents the real number of feedback coefficients for each subband, which is calculated as formula K0=β2LM1  , and M1=p1N3R. In the enhanced Type II codebook, W2 is the linear combination subband matrix, after frequency compression, the W2 is sparse in both space and frequency dimensions, meanwhile, most energy is concentrated in a few coefficients. Therefore, the most significant K0 FD coefficients in W2 are reported and the rest are assumed to be zero.

# Simulation results

- XP High correlation model

In this section, we investigate the throughput performance of different codebooks with “MCS13 rank2” under TDL-C XP high correlation model.

- 16x2 rank2

Figure 1: PMI performance with Type I, Type II and enhanced Type II codebook under XP High correlation model (16x2 Rank2 case)

Table 3: TP ratio summary for MCS13 Rank2 case under XP High correlation model

|  | Type I | Type II | Enhanced Type II with ParaCombination_r16 =1 | Enhanced Type II with paraCombination_r16=4 | Enhanced Type II with paraCombination_r16=6 |
| --- | --- | --- | --- | --- | --- |
| TP ratio of SNR point at 90% throughput | 1.99 | 14.02 | 3.19 | 2.81 | 3.22 |

- XP Medium correlation model

- 16x2 rank1

Figure 2: PMI performance with Type I, Type II and enhanced Type II codebook under XP Medium correlation model (16x2 Rank1 case)

In table 4, we summarize absolute throughput ratio between following PMI and random PMI under 16Tx rank1 cases with different codebook configurations.

Table 4: TP ratio summary for MCS20 Rank1 case under XP Medium correlation model

|  | Type I | Type II | Enhanced Type II with ParaCombination_r16=2 | Enhanced Type II with paraCombination_r16=6 |
| --- | --- | --- | --- | --- |
| TP ratio of SNR point at 90% throughput | 1.54 | 4.87 | 3.25 | 18.97 |

- 16x2 rank2

Figure 3: PMI performance with Type I, Type II and enhanced Type II codebook under XP Medium correlation model (16x2 Rank2 case)

In table 5, we summarize relative throughput ratio between following PMI and random PMI under 16Tx rank1 cases with different codebook configurations.

Table 5: TP ratio summary for MCS20 Rank2 case

|  | Type I | Type II | Enhanced Type II with paraCombination_r16=2 | Enhanced Type II with paraCombination_r16=6 |
| --- | --- | --- | --- | --- |
| TP ratio of SNR at 90% throughput | 1.8 | 4.63 | 2.19 | 2.41 |

As observed above, we obtain that:

- A frequency compression matrix Wf is introduced to the enhanced Type II codebook, which brings more resolution than Type II codebook.

- The performance of enhanced Type II codebook will be significantly improved with the increase of feedback overhead. For the case “paraCombination_r16=6”, it achieves better performance than other cases meanwhile the UE processing complexity and reporting overhead also growth.

- In the XP Medium correlation model, Type I codebook only has one DFT vector which may not be able to accurately represent the best beams. In this case, the propagation environments have multiple dominant clusters, the style of type II and enhanced type II codebooks consist of multiple DFT vectors which are fit for the NLOS channels more preciously. Compared with Fig.1, the gain between the enhanced Type II and Type I is limited in XP High correlation model. In brief, XP medium model is recommended for type II and enhanced type II measurement condition.

# Conclusion

In this contribution, we provide simulation results for enhanced Type II codebook. The relative ratio at the 90% maximum throughput mark are also provided in table 1 and table 2.

Observation 1: The enhanced Type II codebook can bring more resolution than type II due to the frequency compression matrix introduced.

Observation 2: The performance of enhanced Type II codebook will be improved with the increase of feedback overhead and UE processing complexity.

Observation 3: The performance gain of type II and enhanced type II is more obviously in XP Medium correlation channel model than in XP high correlation case. In brief, XP medium model is recommended for type II and enhanced type II measurement condition.

# 4. Reference

[1] R4-2002419, “Way forward on demodulation and CSI requirement for NR -MIMO”, Samsung, RAN4 94-e

[2] R4-2002421, “Way forward on PMI reporting requirements for NR eMIMO“, Qualcomm, RAN4 94-e

[3] R4-2000321, “Initial simulation results for PMI reporting with enhanced type II codebook”, Samsung, RAN4 94-e