3GPP TSG-RAN WG4 Meeting #94e-bis                                                      R4-2004927

Electrical Meeting, April.20th – April.30th

Source:	Samsung

Title:	Test case design for Rel-16 enhanced type II codebook

Agenda item:			6.11.3.3

Document for:	Approval

- Introduction

Initial discussion on Rel-16 NR e-MIMO performance requirements was trigged in last RAN4 meeting, and the overall performance impact was summarized in below table following the agreements in [1]:

| Sub-items |  | BS Demod | UE Performance |  |
| --- | --- | --- | --- | --- |
|  |  |  | UE Demod | CSI |
| Item 1: CSI enhancement for MU-MIMO |  | No | Yes | Yes |
| Item 2: Multi-TRP/Panel transmission |  | No | Yes | No |
| Item3:Beam management enhancement | Item3a: L1-SINR measurement | No | No | No |
|  | Item3b: BFR for Scell | No | No | No |
|  | Item3c: DL/UL beam indication with reduced latency and overhead | No | No | No |
| Item4: Full TX power UL transmission |  | No | No | No |
| Item5: Low PAPR RS |  | FFS | FFS | No |

In this contribution, we further provided detailed test case design for PMI test cases with Rel-16 advanced Type II codebook based on the agreed WF in [2].

# Discussion

Number of Tx ports

- Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) - Option 2 : 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4) - Option 3: 4 ports with (N1,N2) = (2,1) and (O1, O2)=(4,1)

Regarding number of Tx ports, 4 ~ 32 ports can be supported for Type II codebook. NR Rel-15 Type II codebook and Rel-16 advanced Type II codebook are the extension of LTE Rel-14 advanced codebook which they share similar codebook structure. It’s better we can align the number of Tx ports with PMI test case of LTE eFD-MIMO advanced codebook to provide comparable performance. In LTE Rel-14 eFD-MIMO WI, 16 tx ports was used for PMI test case with advanced codebook.

Figure 1:  RAN4 roadmap of MIMO PMI test cases

The test complexity especially the number of required individual MIMO channel faders also needs to be considered when we decide number of Tx ports：

In table 1 below, we summarized number of independent MIMO faders required for different PMI test cases for LTE and on-going Rel-16 eMIMO WI and performance enhancement WI. Compared to LTE, number of faders increased by 4/8 times pending on the decision of the number of Tx ports we selected.

Table 1: Summary of number of MIMO faders under PMI test cases

| Item | Rel-13 FD-MIMO | Rel-14 eFD-MIMO | Rel-15 NR | Rel-16 performance enhancement WI | Rel-16 eMIMO WI |
| --- | --- | --- | --- | --- | --- |
| Test cases | 32x2 Class A PMI test cases | 16x2 Advanced codebook PMI test case with 2 beams | 8x4 Type I PMI test case | 32x4 Type 1 PMI test case 16x4/32x4 Type II codebook with L=2 beams | 16x4/32x4 Type II codebook with L= 4beams |
| Required maximum MIMO faders | 64 faders | 64 faders | 32 faders | 128 faders for Type I 128/256 faders for Type II | 256/512 faders |

With above analysis, we proposed to introduce Rel-16 Type II codebook PMI test cases with 16 Tx ports.

Rel-16 Type II codebook construction

Rel-16 Type II  Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH

- P=2N1N2= #SD dimensions

- N3= #FD dimensions

- Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank)

- Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=fk0 fk1…fkM-1, where fkmm=0M-1 are M size-N3×1 orthogonal DFT vectors - Number of FD-components M is configurable - Linear combination by W2 (for a layer) - W2=c0,0⋯c0,M-1⋮⋱⋮c2L-1,0⋯c2L-1,M-1 is composed of K=2LM linear combination coefficients

Rel-16 Type II codebook construction consists of below parameters:

- n1-n2-codebookSubsetRestriction-r16 : (N1,N2), (O1, O2), PCSI_RS

- numberOfPMISubbandsPerCQISubband: R =1 or 2

- paramCombination-r16:L, β and pυ

  - L: number of beams

  - β : compression factor in spatial domain

  - pυ : compression factor in frequency domain

Table 5.2.2.2.5-1: Codebook parameter configurations for L,  β and pυ

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |
| 7 | 6 | ¼ | - | ½ |
| 8 | 6 | ¼ | - | ¾ |

For number of PMI sub-bands perCQI band, we suggest to take R =2. Regarding paramCombination-r16, we recommend to take value 6, with this configuration, we can maximize number of candidate codebooks. From UE processing respective, this requires maximum UE calculation complexity and acts like a pressure test.

Proposal 1-codebook construction: Introduce PMI test cases with enhanced Type II codebook with below parameters:

- Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1, O2)  = (4,4)

- numberOfPMISubbandsPerCQISubband: R =2

- paramCombination-r16: 6, with L =4, pv =1/2, β=1/2

Beam-steering model

For beam-steering model, we propose to introduce a generic beam steering model into specification which the number of beams configurable.  With this generic beam steering approach, it can be applied for PMI test cases with different codebook types i.e. single beam direction (Type I codebook), dual beam directions (Rel-15 Type II  codebook)  and multi-beam directions (Rel-16 Type II codebook) in a future proof manner.

The multi-beam steering approach as specified in B.2.3B.4A of TS 36.101 can be used as starting point to further extend applied for L beams:

- beam index

- ， relative power of the l beam compared to first beam

- , total power scaling factor

For simplicity, the power of beams can be fixed as equivalent to first beams then beam steering model can as follow

Proposal 2-Beam steering: Introduce a generic beam steering model into specification in a future proof manner which the number of beams configurable.

Propagation condition, Test Metric & MCS and Rank

Regarding test metric and MIMO correlation, below options agreed in last RAN4 meeting:

- Channel Model - Option 1: TDLA30-5 - Other options not precluded - MIMO Correlation - Option 1: XP High - Option 2: XP Medium - Other options not precluded - Test Metric - Option 1: Relative Throughput ratio between following PMI and random PMI - Option 2: Relative Throughput ratio with following PMI between enhanced Type II codebook and Rel-15 Type II codebook - Option 3: Same test metric as in LTE for advanced CSI requirements - Other options not precluded

Based on the initial evaluation results in our companion paper [3], there are more performance difference among Rel-16 Type II codebook, Rel-15 Type II and Rel-15 Type I codebook under XP medium correlation compared to XP high correlation.  For test metric, we observed that it’s feasible to introduce proper test requirements under XP medium correlation channel with legacy test metric “relative throughput ratio between following PMI and random PMI”.

Proposal 3-Propagation condition: Introduce test case with MIMO correlation -XP Medium (option2).

Proposal 4-Test metric: It's feasible to introduce test case with test metric - relative throughput ratio among following PMI and random PMI (option 1).

Proposal 5-MCS&Rank: It’s feasible to use MCS20 (64QAM), Rank2 for introducing test cases.

Other test parameters

For other test parameters, we can reuse test parameters from Rel-15 NR PMI test cases as starting points.

Proposal 6-other test parameters: Reuse test parameters from Rel-15 NR PMI test cases as starting points as summarized below tables:

| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4/16/32 |
|  | CDM Type |  | FD-CDM2 for 4 ports/ CDM4 (FD2, TD2) for 16/32 ports |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) for 4 ports/ Row 12, (2, 4, 6, 8) for 16 ports/ Row 17, (2, 4, 6, 8) for 32 ports |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) for 4 ports/ (5,-) for 16 ports/ (5,12)) for 32 ports |
|  | CSI-RS interval and offset | slot | 5/1 for FDD, 10/1 for TDD |
|  | aperiodicTriggeringOffset |  | 0 |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | Option 1: 6 with L =4, pν =1/2, β=1/2 |
|  | numberOfPMISubbandsPerCQISubband |  | Option 1: R = 1 Option 2: R = 2 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 4 ports/ (4,2) for 16 ports/ (4,4) for 32 ports |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) for 4 ports/ (4,4) for 16/32 ports |
|  | n1-n2-codebookSubsetRestriction-r16 |  | FFS |
|  | typeII-RI-Restriction-r16 |  | FFS |
| CSI-RS resource/CSI reporting interval and slot offset |  | slot | FR1 FDD 15 kHz: 5/1 FR1 TDD 30 kHz: 10/1 |
| Minimum CSI delay |  | ms | 8 ms (8 slots) for FDD 6.5 ms (13 slots) for TDD |

# 3. Conclusion

In this contribution, test case design for PMI test cases with enhanced Type II codebook provided.

Proposal 1-codebook construction: Introduce PMI test cases with enhanced Type II codebook with below parameters:

- Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1, O2)  = (4,4)

- numberOfPMISubbandsPerCQISubband: R =2

- paramCombination-r16: 6, with L =4, pv =1/2, β=1/2

Proposal 2-Beam steering: Introduce a generic beam steering model into specification in a future proof manner which the number of beams configurable:

  - beam index

  - ， relative power of the l beam compared to first beam

  - , total power scaling factor

Proposal 3-Propagation condition: Introduce test case with MIMO correlation -XP Medium (option2).

Proposal 4-Test metric: It's feasible to introduce test case with test metric - relative throughput ratio among following PMI and random PMI (option 1).

Proposal 6-MCS&Rank: It’s feasible to use MCS20 (64QAM), Rank2 for introducing test cases.

A detailed simulation assumption was also provided in annex for reference.

# Reference

[1] R4-2002419, “WF on demodulation and CSI requirement of NR eMIMO”, Samsung

[2] R4-2002421, “WF on PMI reporting requirement for NR eMIMO”, Qualcomm

[3] R4-2002978, “Initial simulation results for PMI requirement with Rel-16 enhanced type II codebook”, Samsung

# 5. Annex

Simulation assumption for FR1 FDD Mode

## FDD

| Parameter |  | Unit | Test FDD 2Rx | Test FDD 4Rx |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | Option 1: TDLA30-5 as baseline |  |
| Antenna configuration |  |  | TBD (Medium or High) XP Option 1: 16 ports with (N1,N2) = (4,2) and(O1,O2)=(4,4) Option 2 : 32 ports with (N1,N2) = (4,4) and(O1,O2)=(4,4) Option 3: 4 ports with (N1,N2) = (2,1) and (O1, O2)=(4,1) |  |
| Beamforming Model |  |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | 5/1 |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | TBD 4/16/32 |  |
|  | CDM Type |  | FD-CDM2 for 4 ports/ CDM4 (FD2, TD2) for 16/32 ports |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) for 4 ports/ Row 12, (2, 4, 6, 8) for 16 ports/ Row 17, (2, 4, 6, 8) for 32 ports |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) for 4 ports/ (5,-) for 16 ports/ (5,12)) for 32 ports |  |
|  | CSI-RS interval and offset | slot | 5/1 |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | FFS |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | 5/1 |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeII-r16 |  |
|  | paramCombination-r16 |  | Option 1: 6 with L =4, pν =1/2, β=1/2 |  |
|  | numberOfPMISubbandsPerCQISubband |  | Option 1: R = 1 Option 2: R = 2 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 4 ports/ (4,2) for 16 ports/ (4,4) for 32 ports |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) for 4 ports/ (4,4) for 16/32 ports |  |
|  | n1-n2-codebookSubsetRestriction-r16 |  | FFS |  |
|  | typeII-RI-Restriction-r16 |  | FFS |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | Option 1: MCS 20 FDD |  |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Taking beam steering approach as specified in B.2.3B.4A of TS 36.101 as staring point with further extension applicable for L beams |  |  |  |  |

## TDD

| Parameter |  | Unit | Test FR1 TDD 2Rx | Test FR1 TDD 4Rx |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | Option 1: TDLA30-5 as baseline |  |
| Antenna configuration |  |  | TBD (Medium or High) XP Option 1: 16 ports with (N1,N2) = (4,2) and(O1,O2)=(4,4) Option 2 : 32 ports with (N1,N2) = (4,4) and(O1,O2)=(4,4) Option 3: 4 ports with (N1,N2) = (2,1) and (O1, O2)=(4,1) |  |
| Beamforming Model |  |  | Option 1: As specified in Annex B.2.3B.4A of TS36.101 -Relative power ratio among two beams can be fixed as 1 (p=1) Other options not precluded |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | 10/1 |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | TBD 4/16/32 |  |
|  | CDM Type |  | FD-CDM2 for 4 ports/ CDM4 (FD2, TD2) for 16/32 ports |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) for 4 ports/ Row 12, (2, 4, 6, 8) for 16 ports/ Row 17, (2, 4, 6, 8) for 32 ports |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) for 4 ports/ (5,-) for 16 ports/ (5,12)) for 32 ports |  |
|  | CSI-RS interval and offset | slot | 10/1 |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | FFS |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | 10/1 |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeII-r16 |  |
|  | paramCombination-r16 |  | Option 1: 6 with L =4, pν =1/2, β=1/2 |  |
|  | numberOfPMISubbandsPerCQISubband |  | Option 1: R = 1 Option 2: R = 2 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 4 ports/ (4,2) for 16 ports/ (4,4) for 32 ports |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) for 4 ports/ (4,4) for 16/32 ports |  |
|  | n1-n2-codebookSubsetRestriction-r16 |  | FFS |  |
|  | typeII-RI-Restriction-r16 |  | FFS |  |
|  | Codebook Type |  | typeII-r16 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | Option 1: MCS 20 TDD |  |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Taking beam steering approach as specified in B.2.3B.4A of TS 36.101 as staring point with further extension applicable for L beams |  |  |  |  |