3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004969

Electronic Meeting, 20 – 30 Apr., 2020

Source:	ZTE

Title:	gNB requirements for NR positioning

Agenda item:	6.8.2.3

Document for:	Discussion

# 1	Introduction

In RAN4 94-e, the topic of gNB requirements for NR positioning has been discussed. Companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issues listed above in the WF and provide our view.

2	Pending issues

As to the optionality of gNB accuracy requirements for NR positioning, the requirements should be mandatory once the gNB supports a certain type of positioning method. In our paper submitted to RAN4 94-e [2], we proposed to set optional requirements for gNB since positioning methods are optional. Consequently, it doesn’t make sense to set mandatory accuracy requirements if gNB doesn’t even choose to support one positioning method. However, once a gNB supports one particular method, it has to meet the accuracy requirements to guarantee the positioning accuracy. Or else the RAT-dependent positioning would be completely unreliable.

Once a gNB supports one particular method, it has to meet the accuracy requirements to guarantee the positioning accuracy.

Another pending issue of particular interest is the gNB Rx-Tx time difference measurement and report mapping. In multi-RTT based positioning methods, both gNB Rx-Tx time difference and UE Rx-Tx time difference is needed to calculate the true round trip time between gNB and UE. Since gNB Rx-Tx time difference and UE Rx-Tx time difference are two equivalent part in one equation and of equal importance to the overall precision, we propose to re-use UE Rx-Tx time difference report mapping for gNB Rx-Tx time difference report mapping.

Re-use UE Rx-Tx time difference report mapping for gNB Rx-Tx time difference report mapping.

3	Conclusion

Proposal 1: Once a gNB supports one particular method, it has to meet the accuracy requirements to guarantee the positioning accuracy.

Proposal 2: Re-use UE Rx-Tx time difference report mapping for gNB Rx-Tx time difference report mapping.

References

- R4-2002275

- R4-2000054, Discussion on gNB requirements and report mapping, ZTE Corporation