3GPP RAN WG4 Meeting #94bis-e                                                                  R4-2004988

# Electronic Meeting, 20 – 30 Apr., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | Rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of interruption for switching between NR SL and LTE SL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce interruption requirement for switching between NR SL and LTE SL by taking RF session’s conclusion into account |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce interruption requirement for switching between NR SL and LTE SL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss corresponding RRM interruption requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

## 12.7	Interruption

### 12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This sub-clause contains the requirements related to the interruptions on the PCell due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE [SidelinkUEInformation].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the serving cell(s) during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message [sl-V2X-ConfigDedicated] (setup and release). This interruption is for both uplink and downlink of the PCell.

Table 1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

### 12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

UE is allowed to drop V2X SL transmission or reception for up to [1] ms when GNSS synchronization source is changed:

- [From GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to GNSS directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to GNSS in-directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From gNB/eNB

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to gNB/eNB directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB in-directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that is synchronized to gNB/eNB in-directly

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - to syncRef UE that is synchronized to gNB/eNB directly

  - to syncRef UE that has the lowest priority

- From syncRef UE that has the lowest priority

  - to GNSS

  - to syncRef UE that is synchronized to GNSS directly

  - to syncRef UE that is synchronized to GNSS in-directly

  - to gNB/eNB

  - syncRef UE that is synchronized to gNB/eNB directly

  - syncRef UE that is synchronized to gNB/eNB in-directly]

UE is allowed to interruption any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals.

### 12.7.3	V2X Sidelink Communication Dropping due to switching between NR V2X Sidelink and E-UTRA V2X Sidelink

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to switching between NR V2X sidelink and E-UTRA V2X sidelink. It is applicable for UE capable of both NR V2X sidelink and E-UTRA V2X sidelink in TDM-ed manner.

The UE is allowed an interruption of up to one slot when switching from E-UTRA V2X sidelink to NR V2X sidelink in NR slot.

The UE is allowed an interruption of up to duration shown in table 12.7.3.-1 when switching from NR V2X sidelink to E-UTRA sidelink in NR slot.

Table 12.7.3-1: Interruption length when swiching from NR V2X sidelink to E-UTRA V2X sidelink in NR slot

|  | NR Slot length (ms) | ld (symbols) | Interruption length (number of slots) |
| --- | --- | --- | --- |
| 0 | 1 | ≤ [10] | 0 |
|  |  | > [10] | 1 |
| 1 | 0.5 | ≤ [7] | 0 |
|  |  | > [7] | 1 |
| 2 | 0.25 | 6~13 | 1 |
| NOTE 1: ld is duration of scheduled resources for transmission of PSSCH and the associated PSCCH [6] |  |  |  |

The UE is allowed an interruption of up to one subframe when switching from E-UTRA V2X sidelink to NR V2X sidelink or from NR V2X sidelink to E-UTRA V2X sidelink in E-UTRA subframe.

< END OF CHANGE #1 >