3GPP TSG-RAN WG4 Meeting #94-e-bis                                       R4-2005026

Electronic Meeting, 20th– 30th April, 2020

Source:	Samsung

Title:	EESS protection from NR BS operation in 24.25 GHz to 27.5 GHz

Agenda item:			4.6.2

Document for:	Discussion

# 1. Introduction

In RAN4#94e meeting there was discussion on how to incorporate the EESS protection unwanted emission for the 26GHz band (24.25-27.5 GHz) which is agreed in WRC-19 with two-step approach. And the WF is agreed as in [1] with summary on the agreement in WRC-19 and WF for next step RAN4 discussion as in figure 1 and figure 2 respectively.

.

Figure 1                                   Figure 2

This contribution discussed further on how to detail of core specification impact based on agreed WF.

# 2. Discussion

According to current NR BS spec, the OBUE range of FR2 is within 1.5 GHz offset from lower and higher edge of operating band. Since it’s agreed to include the additional EESS protection “as both OBUE limits and spurious emission limits” , in addition, the additional limits is agreed to be expressed in term of more generic way of frequency range with implication on operating band. OBUE limit would be applied to the operating band of which FDL,low is lower than or equal to 25.1 GHz. And the spurious emission limit should be applied to operationg band of which the FDL,low is equal to or higher than 25.5 GHz.

Consequently, for OBUE and TX spurious emission one dedicated sub-clause shall be inluced for addtional emission on EESS protection on top of category A and category B OBUE limits. And for RX spurious emission at least one additional EESS protection table should be added. And the requirements can be expressed with the same note 1/2 but dedicated note 3 for OBUE, spurious emission and RX spurious emisson as example below.

Table xxx: Additional protection limits for Earth Exploration Satellite Service

| Frequency range for emission requirement | Limit | Measurement bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm (NOTE 1) | 200 MHz |
| 23.6 – 24 GHz | -9 dBm (NOTE 2) | 200 MHz |
| NOTE 1: This limit applies to BS manufactured from 1 January 2021 to 1 September 2027. NOTE 2: This limit applies to BS manufactured after 1 September 2027. This limit will not apply to BS which has been manufactured prior to this date. NOTE 3 for OBUE: This limit applies for any part of BS transmitted carriers falls in the range of 24.25 GHz to 27.5 GHz when frequency range from ΔfOBUE below FDL,low of BS transmitter operating band to FDL,low of BS transmitter operating band covers the range of 23.6 – 24 GHz. NOTE 3 for TX spurious emission: This limit applies for any part of BS transmitted carriers falls in the range of 24.25 GHz to 27.5 GHz when FDL,low of BS transmitter operating band is higher than ΔfOBUE above 24 GHz. NOTE 3 for RX spurious emission: This limit applies for any part of BS operating carriers falls in the range of 24.25 GHz to 27.5 GHz |  |  |

Here main points are considered as:

- Corresponding requirements including OBUE and Spurious emission should be defined with the applicability in term of frequency range to comply with ITU decision

- To accommodate potential different timeline in different regional regulation, one note can be considered to mention but no need to incorporate multiple timelines in 3GPP specification

# 3. Conclusion

In this contribution, how to implement the RAN4 agreement on EESS protection in TS38.104 is discussed further in detail for OBUE, spurious emission and Rx spurious emission, taking into account WRC-19 outcomes.

# 4. Reference

[1] R4-2002466, Way forward on EESS protection for NR BS operation in Band n257 and n258

[2] RESOLUTION 750 (REV.WRC-19) Compatibility between the Earth exploration-satellite service (passive) and relevant active services