3GPP TSG-RAN WG4 Meeting #94-e-bis                                                                       R4-2005029

Electronic Meeting, 20th– 30th April, 2020

Source:        Samsung

Title:           IAB-MT mandatory feature

Agenda:        6.5.1.3

Document for:  Approval

# Introduction

In RAN#87e there was extensive discussion on whether IAB-MT will comply with Rel-15 UE mandatory feature list. Due to diverse views from companies, no agreement achieved. Hence it is suggested to

- RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation (and if found useful, for different classes of IAB-MTs as being defined by RAN4).

- RAN WGs should strive to minimize specification impact.

This contribution provides our understanding on this aspect. In discussion part, the RAN4 related RRM and RF mandatory features captured in table 4.3-1 of TR38.822 are abstracted in two tables as for system parameter and RF aspect respectively since no mandatory feature under list of baseband feature.

# Discussion

Table 1: Mandatory feature review on system parameters

| Index in 38.822 | Feature group | Mandatory type | Optional for IAB-MT or not? | Consideration |
| --- | --- | --- | --- | --- |
| 1-2 | 64QAM modulation for FR2 PDSCH | Mandatory without capability signalling | No | This is fundamental feature should be mandatory supported by IAB-MT as well. Otherwise, new capability signalling have to be discussed which is not aligned with RAN-P guideline to minimize the spec impact. |
| 1-3 | 64QAM for PUSCH | Mandatory without capability signalling | No | This is fundamental feature should be mandatory supported by IAB-MT as well. Otherwise, new capability signalling have to be discussed which is not aligned with RAN-P guideline to minimize the spec impact. |
| 1-4 | 256QAM for PDSCH | Mandatory with capability signalling for FR1 | No | This is fundamental feature should be mandatory supported by IAB-MT as well. |
| 1-6 | pi/`2-BPSK for PUSCH | Mandatory with capability signalling for FR2 | No | No reason to have exception of this feature for IAB-MT. Change the mandatory decision in Rel-15 will result in the scheduler design impact to IAB-DU. |
| 1-7 | pi/`2-BPSK for PUCCH format 3/4 | Mandatory with capability signalling for FR2 | No | No reason to have exception of this feature for IAB-MT. Change the mandatory decision in Rel-15 will result in the scheduler design impact to IAB-DU. |
| 1-8 | Active BWP switching delay | Mandatory to support either type 1 or type 2 with capability signalling | No | This feature should be reported to donor gNB or parent IAB-DU for schedule efficiency. |
| 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in `1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | No | This feature is only for FR1 sub 6GHz. According to current operating bands to be defined for IAB it may only have impact on Band n41 related ENDC comb if requested within this Release. And for IAB-MT should report its capability to donor gNB or parent IAB-DU for schedule efficiency especially for inter-vendor operation. |
| 1-11 | 7.5kHz UL raster shift | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective | No | No reason to have exception of this feature for IAB-MT if SUL to be supported. Even though the current operating bands associated with IAB does not include SUL bands so far, but considering the back compatible of adding new operating bands for IAB, it is better to keep the mandatory decision for IAB-MT |
|  |  | 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.10`1-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | No | No reason to have exception of this feature for IAB-MT if SUL to be supported. Even though the current operating bands associated with IAB does not include SUL bands so far, but considering the back compatible of adding new operating bands for IAB, it is better to keep the mandatory decision for IAB-MT |

Table 2: Mandatory feature review on RF aspect

| Index in 38.822 | Feature group | Mandatory type | Mandatory for IAB-MT or not? | Consideration |
| --- | --- | --- | --- | --- |
| 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | For FR1, all the bandwidths listed in TS38.10`1-1 v15.0.0 Table 5.3.`5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R`4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. | No | Mandatory channel bandwidth should be applied for IAB usage to reduce operational restrictions for each site in different environments. |
|  |  | For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | No | Mandatory channel bandwidth should be applied for IAB usage to reduce operational restrictions for each site in different environments. |
| 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. | No | Only mandatory case is the FR1+FR2 inter band CA mixed numerologies should be supported. This should be mandatory feature for IAB-MT as well. |
| 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.10`1-2 with capability if UE supports non-contiguous CA in FR2 | No | It is assumed that the same definition of separation will be applied for IAB-MT for FR2. And this feature should be mandatory for IAB-MT just as UE. |
| 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.10`1-3 | No | The capability should be aware by donor gNB or parent IAB-DU for scheduling if the inter-band ENDC comb is to be supported. |
| 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.10`1-1, TS 38.10`1-2 and TS 38.10`1-3 | No | The capability should be aware by donor gNB or parent IAB-DU for scheduling if the inter-band CA comb is to be supported. |
| 2-8 | UE power class | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class | No | This would be important if MPR/A-MPR reduction to be defined. It is beneficial for Network side to understand the reduction is based on which level. And even if no reduction needed under any condition, it would be helpful to deliver it network especially for inter-vendor operation. Even though IAB-MT power definition follows BS framework, this can be mapped for IAB-MT purpose. |
| 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Mandatory/Optional support depends on band combination and captured in TS 38.10`1-1 | No |  |
| 2-10 | Multiple frequency band indication | Mandatory without capability signalling | No | This is fundamental feature should be mandatory supported by IAB-MT as well. Otherwise, new capability signalling have to be discussed which is not aligned with RAN-P guideline to minimize the spec impact. |
| 2-12 | Multiple NS/P-Max | Mandatory without capability signalling | No | No harm to support this default feature. Otherwise, new capability signalling have to be discussed which is not aligned with RAN-P guideline to minimize the spec impact. |
| 2-16 | PA architectures for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. | No | This feature is for sub 6GHz only. This should be claimed if corresponding ENDC configuration to be supported by IAB-MT. |
| 2-17 | PA architectures for intra-band UL CA | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture | No | This feature is for sub 6GHz and MMW. This should be claimed if corresponding intra-band UL CA configuration to be supported by IAB-MT. |

# Summary

Based on our review on UE Rel-15 mandatory feature, the conclusion is that IAB-MT is suggested to support all Rel-15 UE mandatory feature to ensure the inter-vendor operation with minimum specification impact also. Furthermore, considering TR38.822 is not maintained after Rel-15, it is proposed to include technical background regarding IAB-MT feature in IAB TR for future reference.

# Reference

[1] TR38.822, NR User Equipment (UE) feature list