3GPP TSG-RAN WG4 Meeting # 94-e-Bis 												R4-2005448

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.9.1.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [94e Bis][218]R_L1enh_URLLC_Demod_Test

Document for:	Information

# Introduction

The URLLC performance discussion is split into two parts; a discussion on a test at ultra-low BLER and a discussion on BS and UE demodulation. This thread treats the ultra-low BLER requirement/test.

At RAN4#94-e, a Way Forward was agreed in [R4-2002422], in which many (but not all) parameters for the ultra-low BLER test was agreed. Concerning the testability, it was agreed to investigate the need and description for a parameter X added to the test SNR that would if needed reduce test time by increasing the amount of early pass devices. Whether X should exist, what value it would take and where it would be captured was not resolved. Finally, there was some brief discussion on whether to create an ultra-low BLER test for FR2 and whether to create an ultra-low BLER CQI test, which was not concluded.

The e-mail discussion is split into three main topic areas:

- Topic 1 discusses and aims to resolve the outstanding parameters for FR1

- Topic 2 discusses issues surrounding the testability including the X parameter and how many requirements and tests to create.

- Topic 3 addresses the need for FR2 and CQI requirements. In the first round, the discussion will focus on FR2/CQI or not. The second round will then if needed elaborate on parameters if needed.

# Topic #1: Remaining FR1 parameters

Topic 1 aims to resolve the remaining issues for FR1 parameters. This includes BS TDD pattern, slot aggregation, waveform, DM-RS configuration, bandwidths and number of resource blocks.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003181 | Intel | Proposal 1: Define URLLC requirements with ultra-low BLER for SNR = 10-5 + IM + X, where X = -0.5 dB. Proposal 2: Define FR2 PDSCH and PUSCH requirements for ultra-low BLER with 120 kHz SCS. Proposal 3: Use Rank 1 and MCS 13 for definition of URLLC PDSCH requirements with Ultra-low BLER. Proposal 4: Use one additional DMRS for definition of URLLC PDSCH requirements with Ultra-low BLER. Proposal 5: Do not define URLLC CQI requirements for ultra-low BLER. Proposal 6: Define PUSCH URLLC requirements with Ultra-low BLER only for CP-OFDM waveform. Proposal 7: Use one additional DMRS for definition of URLLC PUSCH requirements with Ultra-low BLER. Proposal 8: Use DSUU TDD pattern for PUSCH requirements with ultra-low BLER. |
| R4-2003678 | Huawei | Proposal 1: No need to define the requirements and test for FR2. Proposal 2: We propose DM-RS configuration of 1+1 for UE. Proposal 3: TDD patterns 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz. Proposal 4: DM-RS configuration of 1+1 for BS. Proposal 5: Bandwidth of 10MHz for 15 KHz and 40 MHz for 30 KHz with full bandwidth frequency allocation. Proposal 6: We propose the value of X is about 0.5-0.8dB. Proposal 7: No need to capture X in the core and conformance specifications. Proposal 8: We propose to define one requirement/test for ultra-low BLER target. Proposal 9: No need to define CQI tests with ultra-low BLER target. |
| R4-2003842 | Ericsson | Proposal 1: Adopt 3D1S1U (S=10:2:2) for 15kHz, 7D1S2U (S=6:4:4) for 30kHz for TDD pattern Proposal 2: Do not consider slot aggregation Proposal 3: Define the test based on CP-OFDM only. Proposal 4: Use the same assumptions for PUSCH and PDSCH DM-RS pattern as those decided for the slot aggregation requirement. Proposal 5: Use the same assumptions for PUSCH number of RB and bandwidth as agreed for the slot aggregation requirement. Proposal 6: X seems to be in the order of 1-3dB Proposal 7: Do not capture X in the core specification. Proposal 8: Include X in the test tolerance, but describe within the Annex on MU and test tolerance how the TT is calculated and the role of X. |
| R4-2003845 | Ericsson | Proposal 1: Do not develop ultra-low BLER tests for FR2. Proposal 2: Create BS and UE performance requirements for slot aggregation and PDSCH/PUSCH mapping type B for FR2. |
| R4-2003900 | Nokia | - RAN4 to consider high reliability & high confidence level requirements only for FR2. - RAN4 to consider option 1 (the “standard” TDD patterns). - RAN4 to not have high reliability & high confidence level test requirements for DFT-s-OFDM. - RAN4 to consider either DM-RS configuration 1+1, or to allow both 1+0 and 1+1 with applicability rule. - RAN4 to consider option 3 for the number of RB for PUSCH (full CBW for 5MHz/15k SCS and 10MHz/30K SCS). - RAN4 to consider option 1 for the channel bandwidth for PUSCH (5MHz for 15k SCS, 10MHz for 30k SCS). - RAN4 to choose “X” at least to be 1dB and preferably 3dB. Where “X” does not impact the minimum performance requirement, but just the test requirement. - RAN4 to consider capturing any non-zero value of “X” explicitly in the “Formula” column of the derivation of test requirements for performance tests appendix of the test specifications. - RAN4 to introduce one (1) requirement/test for high reliability & high confidence level test case, under the constraint that “X” is chosen larger than 1dB. |
| R4-2002971 | Samsung | Proposal 1: Reusing the TDD pattern 3D1S1U for 15 KHz SCS and 7D1S2U for 30 KHz SCS for URLLC test. Proposal 2: Introduce 2 DMRS configuration for PUSCH low BLER high confidence test Proposal 3: Introduce 5MHz for 15K SCS, 10MHz for 30K SCS with full bandwidth for PUSCH with lower BLER high reliability test. Proposal 4: Introduce additional margin X with part of IM implicitly for URLLC test requirement, not to specify the value into conformance and core spec. Proposal 5: Only introduce the requirement for high reliability with higher BLER with 2 slots aggregation level. Proposal 6: Deprioritize the high reliability requirement with DFT-s-OFDM waveform. Proposal 7: Non-slot scheduling with 4 symbols can be considered for the lower latency requirement. Proposal 8: Deprioritize the latency requirement with DFT-s-OFDM waveform for NR URLLC Proposal 9: Reuse the normal PUSCH test metric with 70% throughput for URLLC latency requirement Proposal 10: Deprioritize FR2 requirement for URLLC low latency requirement, if RAN4 agreed to introduce FR2 requirement for URLLC low latency, the test applicability for FR1 and FR2 requirement should be defined with only one of them chosen to test with BS declaration. |
| R4-2003631 | NTT DoCoMo | Proposal 1: For ultra-high BLER (10e-5 BLER) tests, introduce requirements for FR2. Proposal 2: For ultra-high BLER (10e-5 BLER) tests, adopt the following TDD patterns. - 15kHz SCS (FR1): 3D1S1U (S=10:2:2) - 30kHz SCS (FR1): 7D1S2U (S=6:4:4) - 60kHz SCS (FR2): 3D1S1U (S=10:2:2) - 120kHz SCS (FR2): 3D1S1U (S=10:2:2) Proposal 3: For ultra-high BLER (10e-5 BLER) tests, allow a BS to declare supported TDD patterns and choose one for the test (Same applicability rule as normal demodulation). Proposal 4: For ultra-high BLER (10e-5 BLER) requirements, adopt DM-RS 1+1 (option 2) for FR1 and DM-RS 1+0 and 1+1 with applicability rule (option 3) for FR2. Proposal 5: Define ultra-high BLER (10e-5 BLER) requirements with the following channel bandwidth. - 15kHz SCS (FR1): Full bandwidth for 5/10MHz - 30kHz SCS (FR1): Full bandwidth for 10/40MHz - 60kHz SCS (FR2): Full bandwidth for 50/100MHz - 120kHz SCS (FR2): Full bandwidth for 50/100MHz NOTE: Only one test per [per SCS per duplex mode] will be performed based on applicability rule Proposal 6: For ultra-high BLER (10e-5 BLER) test, introduce >1 requirements and test only one per supported SCS per supported duplex mode (using applicability rule). |
| R4-2004010 | NTT DoCoMo | Proposal 1: Introduce FR2 URLLC requirements for high reliability Proposal 2: Introduce 8 and 4 PDSCH aggregation level for FDD and TDD, respectively Proposal 3: Introduce UE FR2 URLLC requirements for low latency Proposal 4: For URLLC requirements for low latency, our preference on TDD pattern (30 kHz SCS) is as follows. - 1st priority: DDDSUUDDDD, S=6D:4G:4U - 2nd priority: 7D1S2U, S=6D:4G:4U Proposal 5: Introduce 4os symbol length in addition to 2os for UE performance requirements in FR2 Proposal 6: For FR1 URLLC requirements for low latency, 4 or 8 HARQ process should be applied Proposal 7: Target BLER=10^-5 should be applied for CQI reporting requirements Proposal 8: Introduce UE FR2 URLLC requirements for ultra-low BLER Proposal 9: one additional DMRS should be applied for URLLC ultra-low BLER test Proposal 10: Introduce one requirement both for FR1 FDD and FR1 TDD for FR1 URLLC ultra-low BLER test. - FFS: for FR2 Proposal 11: Define CQI testing with ultra-low BLER |
| R4-2004557 | Qualcomm | Proposal 1: Use Method 1 for testing 99.999% reliability with 99.999% confidence level. Proposal 2: Consider discussing test methodology for UE and gNB separately. |
| R4-2004781 | Qualcomm | Proposal 1: Do not define URLLC test cases for FR2. Proposal 2: Define CQI reporting tests for testing 99.999% reliability under AWGN condition. Proposal 3: Define a lower bound for median reported CQI in the CQI reporting tests for 99.999% reliability. Observation 1: Only one long test needs to be run for testing CQI reporting under AWGN condition for 1e-5 BLER with 99.999% confidence level. Proposal 4: Define CQI reporting test under AWGN condition with 99.999% confidence level. Observation 2: It is possible to have an applicability rule between CQI reporting test and FMCS test under AWGN. Proposal 5: Define an applicability rule between CQI reporting test and FMCS test under AWGN to reduce the number of tests. Proposal 6: Use 1 additional DMRS symbol for defining PDSCH 1e-5 BLER tests. Observation 3: The lowest SNR for FR1 2Rx FMCS requirements in 38.101-4 is -1.1dB. Proposal 7: Use MCS >= 10 for defining the tests with 1e-5 BLER to avoid very low SNR regime. |

## Open issues summary

### Sub-topic 1-1 BS TDD pattern

Sub-topic description: TDD pattern to apply for the BS requirement

Open issues and candidate options before e-meeting:

Issue 1-1: BS TDD pattern

- Proposals

  - Option 1: 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz (Huawei, Ericsson, Nokia, Samsung, NTT DoCoMo)

  - Option 2: DSUU (Intel)

- Recommended WF

  - Discuss in 2nd round

### Sub-topic 1-2 Slot aggregation

Sub-topic description: At the last meeting it was agreed no slot aggregation for X<=3dB, otherwise slot aggregation of 1 or 2 could be considered

Open issues and candidate options before e-meeting:

Issue 1-2: Slot aggregation

- Proposals

  - Option 1: No aggregation (Ericsson, Nokia, Apple, Intel, Huawei, Samsung)

  - Option 2: (Possible aggregation was an option at the end of the last meeting; no other company inputs so far)

- Recommended WF

  - Option 1

### Sub-topic 1-3 Waveform for BS requirement

Sub-topic description: CP-OFDM waveform is agreed, but DFT-s-OFDM is FFS

Open issues and candidate options before e-meeting:

Issue 1-3: Waveform for BS requirement

- Proposals

  - Option 1: Keep to CP-OFDM only (Intel, Ericsson, Nokia, Samsung, Huawei)

  - Option 2: CP-OFDM and DFT-s-OFDM (NTT DoCoMo)

- Recommended WF

  - Option 1

### Sub-topic 1-4 DM-RS configuration

Sub-topic description: Whether to use 1 or 2 DM-RS for PDSCH and PUSCH is not agreed yet

Open issues and candidate options before e-meeting:

Issue 1-4-1: UE PDSCH DM-RS

- Proposals

  - Option 1: 1+1 (Intel, Huawei, NTT DoCoMo, Qualcomm, Apple)

  - Option 2: Same as agreed for slot aggregation (high BLER) requirement (Ericsson)

- Recommended WF

  - Agree option 1, reconfirm or possible revise in case slot aggregation not agreed as 1+1. Agreement is for FR1 only.

Issue 1-4-2: BS DM-RS

- Proposals

  - Option 1: 1+1 (Intel, Nokia, Samsung, NTT DoCoMo, Huawei, Ericsson if also 1+1 for slot aggregation)

  - Option 2: 1+1 and 1+0 (Nokia)

- Recommended WF

  - Agree option 1, reconfirm or possible revise in case slot aggregation not agreed as 1+1. Agreement is for FR1 only.

### Sub-topic 1-5 PUSCH bandwidth and number of PRB

Sub-topic description: At least 10MHz (15k SCS) and 40MHz (30k SCS) was agreed, but additional bandwidths was left FFS. The number of PRB was not agreed.

Open issues and candidate options before e-meeting:

Issue 1-5: PUSCH bandwidth and number of PRB

- Proposals

  - Option 1: 10MHz for 15k SCS, 40-MHz for 30k SCS, full bandwidth (Huawei, Nokia, Samsung, Intel)

  - Option 2: Same as agreed for high BLER slot aggregation requirement (Ericsson)

  - Option 3: 5/10MHz for 15k SCS, 10/40MHz for 30k SCS (NTT DoCoMo, Ericsson)

- Recommended WF

  - Discuss in 2nd round

### Sub-topic 1-6 UE MCS

Sub-topic description The MCS was agreed as MCS5 from the low spectrum efficiency table in previous meetings. In this meeting, there is a proposal to use MCS13 for the UE.

Open issues and candidate options before e-meeting:

Issue 1-6: UE MCS

- Proposals

  - Option 1: Use MCS13 for the UE (Intel, Apple)

  - Option 2: Keep to previous agreed MCS5

  - Option 3: Use larger MCS to avoid low SNR requirements for UE. (Qualcomm, Intel, Huawei)

- Recommended WF

  - Further investigate and confirm MCS. MCS13 is baseline but a different MCS could be agreed if better.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 1-4-1 and 1-4-2: We support 1+1 as long as the same is also agreed for the slot aggregation requirement (so that the same FRCs can be used). Issue 1-5: We can consider option 3 as long as there is an applicability rule. But more importantly, our view is that we should align with the assumptions for the slot aggregation requirement. |
| Qualcomm | Issue 1-6: In our view, we need to consider larger MCS so that requirements could be defined at a reasonable SNR. Intel has similar view. We haven’t run simulations for MCS13 yet, so it is hard for us to agree to MCS13 at this point but we are ok to consider it as an option to give other companies a chance to run simulations and finalize the MCS in next meeting. |
| NTT DOCOMO | Issue 1-1: In our opinion, option 1 is suitable for the default TDD patterns captured in the specifications. Other TDD patterns also can be used for testing in the same manner as Rel.15. Issue 1-3: We prefer Option 2. Applicability rule that test either one according to the declaration (i.e., CP-OFDM or DFT-s-OFDM) can be introduced. Note that the number of test does not increase. Issue 1-4-1: We prefer Option 1. Issue 1-5: We prefer Option 3. Regarding Option 2, this option means that RAN4 does not support URLLC requirements for BS with channel bandwidth less than 10MHz for 15kHz SCS or 40MHz for 30kHz SCS since there are no applicable requirements for such BS. In our understanding, to allow BS to support any channel bandwidth, requirements for minimum channel bandwidth should be defined. In addition, similar applicability rule for channel bandwidth as Rel.15 can be considered for URLLC requirements. |
| Apple | Issue 1-2: Slot aggregation No slot aggregation Issue 1-4-1: UE PDSCH DM-RS Option 1: 1+1 Issue 1-6: UE MCS Option 1. |
| Intel | Issue 1-1: BS TDD pattern We assume that purpose of test is to verify the operation on ultra-low BLER, which may take log test time. Therefore, we think that it is more important to consider TDD pattern which allows to reduce test time and using of practical TDD pattern can be considered for all other URLLC requirements. Issue 1-2: Slot aggregation We support Option 1 (i.e. no aggregations for Ultra-low BLER requirements) Issue 1-5: PUSCH bandwidth and number of PRB We support Option 1 Issue 1-6: UE MCS We are fine to further investigate in this direction, but we would like note that it is better to select MCS in a way that SNR operating point is not less than -3 dB to avoid any RRM or PDCCH related issues during the test. |
| Nokia, Nokia Shanghai Bell | Issue 1-1: BS TDD pattern UL biased TDD patterns represent unrealistic loads for the hardware equipment and implementation. While we are very sympathetic to the goal of reducing test time, stressing the hardware in ways that it is not designed for, could lead to wrong conclusions. Hence, option 1 (the “standard pattern”) should be chosen. Issue 1-2: Slot aggregation In the interest of test time, we also prefer option 1 (no aggregation). Issue 1-3: Waveform for BS requirement Given the heavy burden w.r.t simulation and testing high reliability & high confidence level requirements, we think that is not justified to double the test and simulation load for DUTs that support both CP-OFDM and DFT-S-OFDM. Hence option 1 Issue 1-4-2: BS DM-RS In the spirit of providing the highest possible reliability, we propose to use as many DM-RS as acceptable. I.e., DM-RS 1+1 in most cases, and DM-RS 1+0, once the TDRA is too short to support two DM-RS. Issue 1-5: PUSCH bandwidth and number of PRB The number of PRB should be chosen as large as possible (within reason w.r.t. TBS). Such a configuration is advantageous for high reliability, since frequency diversity needs to be exploited in real systems. Hence, we would prefer to not go below 10MHz/15kHz and 40MHz/30kHz. Albeit that for the given AWGN only channel model, also option 3 should not show an error floor. |
| Huawei | Issue 1-2: We prefer option 1. Issue 1-3: We prefer option 1 Issue 1-4-2: We prefer option 1 Issue 1-6: We are running the simulation, I will update the results when the simulation is finished. |
| Samsung | Issue 1-1 We are fine with option 1 to align with the TDD patterns, using in Rel-15 NR PUSCH demodulation requirement Issue 1-2 We are fine with option 1, meanwhile, based on TDD pattern for 15KHz, the effective PUSCH transmission is only 1 slots Issue 1-3 We are fine option 1 Issue 1-4-2 We are fine with option1, based on our initial results, about 1dB gain can be achieved. Issue 1-5 We are fine with option 1 |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 BS TDD pattern | Tentative agreements: None Candidate options: - Option 1: 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz (Huawei, Ericsson, Nokia, Samsung, NTT DoCoMo) - Option 2: DSUU (Intel) Recommendations for 2nd round: Continue discussion in 2nd round |
| Sub-topic#1-2 Slot aggregation | Tentative agreements: No slot aggregation Candidate options: Recommendations for 2nd round: Topic concluded; no 2nd round discussion |
| Sub-topic#1-3 Waveform for BS | Tentative agreements: CP-OFDM only Candidate options: Recommendations for 2nd round: Topic concluded; no 2nd round discussion |
| Sub-topic#1-4 DM-RS configuration | Tentative agreements: UE DM-RS configuration is 1+1. Reconfirm or revise this decision in case the decision for DM-RS configuration for the slot aggregation requirement differs. BS DM-RS configuration is 1+1. Reconfirm or revise this decision in case the decision for DM-RS configuration for the slot aggregation requirement differs. These agreements apply for FR1 only. Candidate options: Recommendations for 2nd round: Topic concluded; no 2nd round discussion |
| Sub-topic#1-5: PUSCH BW and PRB number | Tentative agreements: None Candidate options: - Option 1: 10MHz for 15k SCS, 40-MHz for 30k SCS, full bandwidth (Huawei, Nokia, Samsung, Intel) - Option 2: Same as agreed for high BLER slot aggregation requirement (Ericsson) - Option 3: 5/10MHz for 15k SCS, 10/40MHz for 30k SCS (NTT DoCoMo, Ericsson) Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic#1-6 UE MCS | Tentative agreements: Further investigate and confirm UE MCS. MCS13 is baseline but a different MCS could be agreed if better. Candidate options: Recommendations for 2nd round: Topic concluded; no 2nd round discussion |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Recommended to use a single WF for the whole thread to capture conclusions at the end of the 2nd round. | Moderator (Ericsson) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Testing issues

## Companies’ contributions summary

See section 1.1

## Open issues summary

This section considers issues relating to testing. In the previous meeting, it was agreed that the test SNR would be calculated as (SNR for 10^-5 BLER +IM + X). The value for the parameter X as well as how and where to document X was not resolved. Also not resolved was the number of requirements and tests.

### Sub-topic 2-1 X parameter definition and value

Sub-topic description: The value for the X parameter, as well as how to incorporate it into the specifications was not resolved in the previous meeting.

Open issues and candidate options before e-meeting:

Issue 2-1-1: Value for X

- Proposals

  - Option 1: 0dB (Qualcomm)

  - Option 2: 0.5dB (Intel, Apple)

  - Option 3: 0.5-0.8dB (Huawei)

  - Option 4: 2-3dB (Ericsson)

  - Option 5: 1-3dB (Nokia)

  - Compromise 1-2dB (Ericsson, Samsung), 1dB (Nokia)

- Recommended WF

  - Further discussion in second round

Issue 2-1-2: How to document X in the specifications

- Proposals

  - Option 1: Do not capture in specifications; include directly into core spec requirement by assuming part of IM (Samsung)

  - Option 2: Capture as part of TT in the conformance specification (Ericsson, Nokia, NTT DoCoMo, Intel)

  - Option 3: Do not capture in specifications, X is not part of IM. (Huawei)

- Recommended WF

  - Further discussion

Issue 2-1-3: Same or separate X for BS and UE

- Proposals

  - Option 1: Separate (Qualcomm, Samsung slight preference)

  - Option 2: Same (Nokia, NTT DoCoMo, Intel)

- Recommended WF

  - Further discussion

### Sub-topic 2-2 Number of requirements and tests

Sub-topic description There is a need to decide how many requirements and how many tests should be specified for BS and UE

Open issues and candidate options before e-meeting:

Issue 2-2-1: Number of UE requirements

- Proposals

  - Option 1: 1 (Huawei)

  - Option 2: 2 (One TDD, one FDD) (NTT DoCoMo)

  - Option 3: 4 (Two duplex * Two antenna configuration) (Ericsson, Apple)

  - Option 4: Also CQI requirements; comes to 8 (Qualcomm)

- Recommended WF

  - It is not of importance to agree a number; rather agree the number of different scenarios and the number of requirements will fall out. Note that currently it appears to be 4 for PDSCH plus potentially CQI, but this depends on final set of scenarios.

Issue 2-2-2: Number of UE tests

- Proposals

  - Option 1: 1 (Huawei)

  - Option 2: 2 (Intel, Apple)

- Recommended WF

  - TBA

Issue 2-2-3: Number of BS requirements

- Proposals

  - Option 1: 1 (Huawei, Samsung)

  - Option 2: At least 4 (NTT DoCoMo; 2 SCS*2 Bandwidth, Ericsson but based on 2*SCS=2*mapping type, Intel, Nokia)

- Recommended WF

It is not of importance to agree a number; rather agree the number of different scenarios and the number of requirements will fall out. Note that currently 2 SCS and 2 mapping type are agreed; this number could change dependin on agreements (There can be applicability rules to reduce testing of course).

Issue 2-2-4: Number of BS tests

- Proposals

  - Option 1: 1 (Huawei, Nokia if X>1dB,Samsung)

  - Option 2: Up to 4 (NTT DoCoMo; one test per SCS and Duplex mode)

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| Ericsson | Issue 2-1-1: The X value should ensure early pass. We noticed that the BLER curve may become less steep below 10^-5, but are still investigating. Possibly our proposal may be a bit higher than needed to ensure early pass; we’d be interested to understand the level of support for a compromise somewhere in the range 1-2dB. Issue 2-1-2: We think it is important not to include X in the core specification because it is not about the core requirement; just facilitating reasonable test time. Issue 2-1-3: We can consider this (i.e. different BS/UE), although it depends on whether the PDSCH and PUSCH could be expected to behave differently with regard to early pass/fail. Issue 2-2-1: Currently the number of UE requirements applicable to FR1 are one FDD, one TDD with 2*2, and 2*4 antenna configuration configured. That means 4 requirements, although not necessarily 4 tests conducted. Issue 2-2-3: Currently at least 4 requirements seem to be needed (2*mapping type, 2*SCS/bandwidth). Applicability rules should be used to limit the number of tests. |
| Qualcomm | Issue 2-1-1: If we want to ensure the SNR-BLER relationship performance is good for the UE, X should be 0dB and any margin needed on top of baseband results can be included in IM, just like other UE demod test cases. I am not sure how X = 0.5-0.8dB matters. That can be absorbed in IM. For other demod tests, we add IM of 0.5dB on top of impairment results anyway. So, we prefer Option 1. Issue 2-1-3: In our opinion, it should be discussed separately for UE and BS. Based on previous meeting’s discussion, BS vendors wanted higher value of X and UE vendors wanted lower value of X. So, it will be beneficial to separate these discussions to make progress. Issue 2-2-1/2-2-2: This issue is rather ambiguous. In our opinion, for each scenario, there will be at least 4 sets of requirements for combination of FDD/TDD, 2Rx/4Rx with applicability rule for 2Rx and 4Rx. Our preference is to define requirements for 2 scenarios: FMCS and CQI reporting. So, if this issue is asking for number of scenarios, our preference is 2. If this issue is asking for total number of tests defined in the spec, our preference is 8 with applicability rules for FMCS & CQI reporting, 2Rx & 4Rx. |
| NTT DOCOMO | Issue 2-1-2: We prefer Option 2. Issue 2-1-3: We support Option2. The motivation for adopting Option 1 is not clear for us. The value of X does not depend on test equipment or environment, so the common value for UE and BS should be considered. Issue 2-2-3: If it is agreed to create FR2 requirements, the number of requirements is 4 SCS*2 CBW. Issue 2-2-4: We need to clarify the definition of “one test”. Our proposal is “one test per supported SCS per supported duplex mode”, which means that the test shall apply only for each subcarrier spacing declared to be supported and apply only for each duplex mode declared to be supported. Some examples are shown below: Example 1: BS that declares support 30kHz and TDD => The total number of tests is 1 (TDD 30kHz SCS) Example 2: BS that declares support 15/30kHz and TDD => The total number of tests is 2 (TDD 15kHz SCS, TDD 30kHz SCS) Example 3: BS that declares support 15/30kHz and TDD/FDD => The total number of tests is 4 (TDD 15kHz SCS, FDD 15kHz SCS, TDD 30kHz SCS, FDD 30kHz SCS) |
| Intel | Issue 2-1-1: Value for X The intention to consider non-zero X is to trigger early pass. Based on our analysis, using of target BLER 10^(-6) instead of 10^(-5) allows to reduce test time. Also, our results show that SNR difference on 10^(-6) and 10^(-5) is about 0.5-dB. Therefore, we suggest to use this value for X. Same time, probably we can keep this question open and decide later based on results collection among the companies. Issue 2-1-2: How to document X in the specifications In case X will be agreed as non-zero, we suggest to capture it as part of TT in the conformance specification (Option 2), because such option allows to clear reflect and document assumptions on URLCC test definition. Also, in case Option 2 will be adopted, we need to inform RAN5 to capture this part in UE conformance specification Issue 2-1-3: Same or separate X for BS and UE Support Option 2. In case we consider CP-OFDM waveform, PDSCH and PUSCH BLER curves behaviour will be the same. Issue 2-2-1: Number of UE requirements We assume that 4 requirements should be defined (FDD/TDD and 2Rx/4RX) Issue 2-2-2: Number of UE tests We assume 2 tests will be executed: 2 Rx with FDD/TDD or 4 Rx with FDD/TDD, based on UE capability. Issue 2-2-3: Number of BS requirements We support Ericsson’s view (i.e. 4 requirements two SCS/CBW combination and two PUSCH mapping types ) |
| Apple | Issue 2-1-1: Value for X Option 2. The purpose of having an additional margin over IM is to ensure that the test can be completed with early pass criteria. Issue 2-2-1: Number of UE requirements 4 requirements for FDD, TDD for 2RX and 4RX Issue 2-2-2: Number of UE tests Option 2: 2 |
| Nokia, Nokia Shanghai Bell | 2.2.1 Sub-topic 2-1 X parameter definition and value Remark on the sub-topic description: The standard difference between minimum requirement and test requirement is usually denoted “TT” (test tolerance) in LTE and Rel-15 NR. We should avoid using “IM” (implementation margin), which seems to only be common in RF requirements. Issue 2-1-1: Value for X Marginal DUTs require too many samples to be practically testable in UL high reliability & high confidence level test cases. However, we cannot assume non-marginal DUTs, since this would effectively require vendors to design the DUTs for even lower BLER, than required for the test case. From our presented exploratory simulations, we see that between 1 and 3 dB of additional margin are required to make a good DUT (designed for 1e-6 BLER) make reliably terminate reasonably early. If required, we can compromise to the lower bound of our identified margin to make progress. Issue 2-1-2: How to document X in the specifications The test specification (38.141-1/2) already have a precedent of relaxing test requirements w.r.t. to the performance requirements. The calculation of the former from the later is given in the appendix and even contains methods to change the KPI. Explicit and transparent inclusion of “X” makes it possible for operators to plan with faithful minimum performance figures and it is possible to centrally account for future advances in performance testing equipment. Hence we should capture any non-zero value of “X” explicitly in the “Formula” column of the derivation of test requirements for performance tests appendix of the test specifications, for example as part of the TT, or as a fixed factor. Issue 2-1-3: Same or separate X for BS and UE Even though, the BS has longer testing times than the UE (due to TDD patterns for example), we should aim to keep the value of X the same for both. The testing time is still an issue for the UE side and the proposed values of X are not larger than 3dB. Issue 2-2-3: Number of BS requirements The main bulk of the test cases is provided in the parallel discussion on relaxed testing with lower confidence levels and KPI targets, where testability is not an issue. Thus a single test case (or max one per SCS) is sufficient and saves a lot of simulation time. We can compromise to a new option “1 for each FR1 SCS.” Issue 2-2-4: Number of BS tests It is clear that a DUT cannot test a SCS that it does not support, hence the DUT should be able to declare (1) test to be carried out. |
| Samsung | Issue 2-1-1 We don’t have details proposal for the value. Considering the value is adding to early pass, possible higher is need. With refer the TT, about 0.3 to 0.6 dB are considering. Meanwhile, In eMBB scenario, we have similar discussed whether considering additional margin for PUSCH FR2 without considering the impact of phase noise. As agreed for FR2 for BS side, the IM gap can be allowed for 4dB. Therefore, a compromise way with 1 to 2dB mentioned by Ericsson is fine for us. Issue 2-1-2 We prefer option 1. In eMBB scenario, we have similar discussed whether considering additional margin for PUSCH FR2 without considering the impact of phase noise. Eventually, RAN4 agreed this additional margin can be considered as in the part of IM, depended on implementation, we would like to apply the same approach not to capture both core spec and conform test spec Issue 2-1-3 We are no strong view, As Rel-15, we have also separated the discussion on the additional margin value for UE and BS side, we slightly prefer to separate Issue 2-2-3 We are fine with option 1 Issue 2-2-4 We are fine with option 1 |
| Huawei | Issue 2-1-2: We cannot agree with the WF. X should not be captured in specifications; IM value is decided by companies. X is an additional value and it should not be part of IM. RAN4 has not discussed this question in history. We just need to agree the margin in the WF and no need to mention the X in the specifications. In specification, there is only an agreed SNR value based on the IM and X. Issue 2-1-3 We prefer option 1. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1-1 Value for X | Tentative agreements: None Candidate options: - Option 1: 0dB (Qualcomm) - Option 2: 0.5dB (Intel, Apple) - Option 3: 0.5-0.8dB (Huawei) - Option 4: 2-3dB (Ericsson) - Option 5: 1-3dB (Nokia) - Compromise 1-2dB (Ericsson, Samsung), 1dB (Nokia) Recommendations for 2nd round: Further discussion in second round. Candidate for discussing in call. |
| Sub-topic#2-1-2 How to document X | Tentative agreements: None Candidate options: - Option 1: Do not capture in specifications; include directly into core spec requirement by assuming part of IM (Samsung) - Option 2: Capture as part of TT in the conformance specification (Ericsson, Nokia, NTT DoCoMo, Intel) - Option 3: Do not capture in specifications, X is not part of IM. (Huawei) Recommendations for 2nd round: Further discussion in 2nd round needed. Candidate for discussing in call. |
| Sub-topic#2-1-3 Separate or same X | Tentative agreements: None Candidate options: - Option 1: Do not capture in specifications; include directly into core spec requirement by assuming part of IM (Samsung) - Option 2: Capture as part of TT in the conformance specification (Ericsson, Nokia, NTT DoCoMo, Intel) - Option 3: Do not capture in specifications, X is not part of IM. (Huawei) Recommendations for 2nd round: Further discussion in 2nd round. Candidate for discussing in call. |
| Sub-topic#2-2-1 Number of UE requirements | Tentative agreements: Regarding number of UE requirements: It is not of importance to agree a number; rather agree the number of different scenarios and the number of requirements will fall out. Note that currently it appears to be 4 for PDSCH plus potentially CQI, but this depends on final set of scenarios. Candidate options: Recommendations for 2nd round: Topic concluded for this meeting; no 2nd round discussion |
| Sub-topic#2-2-2 Number of UE tests | Tentative agreements: None Candidate options: Regarding number of UE tests: - Option 1: 1 (Huawei) - Option 2: 2 (Intel, Apple) Recommendations for 2nd round: Check if further progress can be made in second round by considering applicability rules. Not recommended for discussion during calls. |
| Sub-topic#2-2-3 Number of BS requirements | Tentative agreements: Regarding number of BS requirements: It is not of importance to agree a number; rather agree the number of different scenarios and the number of requirements will fall out. Note that currently 2 SCS and 2 mapping type are agreed; this number could change depending on agreements (There can be applicability rules to reduce testing of course). Candidate options: Recommendations for 2nd round: Topic concluded for this meeting; no 2nd round discussion |
| Sub-topic#2-2-4 Number of BS tests | Tentative agreements: None Candidate options: Regarding number of BS tests: - Option 1: 1 (Huawei, Nokia if X>1dB,Samsung) - Option 2: Up to 4 (NTT DoCoMo; one test per SCS and Duplex mode) Recommendations for 2nd round: Check if further progress can be made in second round by considering applicability rules. Not recommended for discussion during calls. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Recommended to use a single WF for the whole thread to capture conclusions at the end of the 2nd round. | Moderator (Ericsson) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Scope

## Companies’ contributions summary

See section 1.1

## Open issues summary

This section considers additional issues regarding the scope of the requirements; in particular whether to create ultra-low BLER requirements for CQI and FR2.

In the first round, the discussion is kept at the level of whether to introduce or not. The second round can discuss parameters if needed.

### Sub-topic 3-1 CQI

Sub-topic description Whether to create CQI requirements

Open issues and candidate options before e-meeting:

Issue 3-1: Create ultra-low BLER CQI requirements

- Proposals

  - Option 1: Create ultra-low BLER CQI requirements (NTT DoCoMo, Qualcomm)

  - Option 2: Do not create ultra-low BLER CQI requirements (Intel, Huawei, Ericsson, Apple)

- Recommended WF

  - TBA

Issue 3-1b: Define applicability rule between Ultra-low BLER CQI requirements and FMCS requirements

- Proposals

  - Option 1: Yes (Qualcomm)

  - Option 2: No

- Recommended WF

  - TBA

### Sub-topic 3-2 FR2

Sub-topic description Whether to create FR2 requirements

Open issues and candidate options before e-meeting:

Issue 3-2: Create FR2 requirements and tests

- Proposals

  - Option 1: Create FR2 requirements and tests for ultra-low BLER (Intel, NTT DoCoMo)

  - Option 2: Do not create FR2 requirements and tests for ultra-low BLER (Huawei, Ericsson, Nokia, Samsung, Qualcomm)

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| Ericsson | Issue 3-1: We do not believe there is a need to create CQI requirements at ultra-low BLER. The high BLER CQI requirements demonstrate that CQI works reliably. The BS anyhow cannot follow CQI exactly at ultra-low BLER (since e.g. the CQI does not predict exactly future channel conditions, or future interference and also includes some UE measurement error… so the BS will need to allow a significant margin over CQI in order to ensure ultra-low BLER). Issue 3-2: Although in our view some of the URLLC features are applicable to FR2 (e.g. reduced symbols due to limited transport block sizes), we believe that these will not be related to the 99.999% reliability aspect of URLLC but rather other less demanding URLLC applications. |
| Qualcomm | Issue 3-1: In our opinion, we should test CQI reporting with 1e-5 BLER to ensure proper UE processing based on BLER target of 1e-5. If we test it for higher MCS, we may end up in a situation where effective BLER is higher than 1e-5 in the field unless network takes it to very low MCS. This will degrade the overall system performance. To reduce the number of test cases, we have proposed an applicability rule between FMCS and CQI reporting test so that we only have to run one of those tests and verify both the requirements properly. To Ericsson: Even in regular CQI reporting tests, there is no mandate on network to follow reported CQI from UE and it will add some margin while applying it in practice. But, we do verify UE processing to make sure UE is properly reporting CQI. So, in our opinion, we should do the same even for URLLC case to ensure proper UE processing. Otherwise, UE may just report some MCS which is based on higher BLER and network adds some margin and still end up with effective BLER higher than 1e-5. That will defeat the purpose of having CQI reporting test at all. Issue 3-2: In our opinion, 1e-5 BLER practical use cases will mostly be deployed under FR1. So, we prefer not to create FR2 requirements. |
| NTT DOCOMO | Issue 3-1: We prefer Option 1. We prefer to intrude the CQI table 3 requirements with 10^-5 BLER since this table was introduced for the URLLC scenario. Issue 3-2: We support Option 2. In our views, some URLLC features (e.g., New MCS table) are FR agnostic features and available in both FR1 and FR2. To verify the performance and functionality of URLLC features in FR2, the requirements for FR2 should be defined. |
| Intel | Issue 3-1: Create ultra-low BLER CQI requirements Support Option 2. Based on our review of 38.521-4, early termination methodology is not used for CQI requirements and fixed test duration is considered: - Median CQI search: “Continue transmission of the PDSCH until 2000 wideband CQI reports have been gathered.” - BLER for Median CQI: “Continue to gather data until the number of filtered ACK+NACK responses reaches 1000.” - BLER for Median CQI+1 ot -1: “…record and filter the ACK, NACK and statDTX responses … until 1000 filtered ACK+NACK responses are gathered.” This methodology is used to CQI table with BLER 0.1. If we reduce the BLER from 0.1 to 10^-5 then we assume that at least test time for BLER calculation should be increased by 10^4 (i.e. keep 2000 reports for median CQI search). We get ~5 hrs for 15 kHz SCS for testing of one SNR point in one test. However, we have two test for different SNR regions and two SNR points in each test. As result, testing of CQI for ultra-low BLER will take from 10 to 20 hrs for SCS 15 kHz, which is much longer than testing with fixed MCS Issue 3-2: Create FR2 requirements and tests We’ve proposed to introduce FR2 requirements, because we think that it is rather important to show that FR2 devices support operation in ultra-low BLER. |
| Apple | Issue 3-1: Create ultra-low BLER CQI requirements Option 2: Do not create ultra-low BLER CQI requirements. Issue 3-2: Create FR2 requirements and tests Option 2: Do not create FR2 requirements and tests for ultra-low BLER |
| Nokia, Nokia Shanghai Bell | Issue 3-2: Create FR2 requirements and tests It is our understanding that for high reliability & high confidence level requirements, the use cases are mostly in FR1, due to generally improved coverage and reliability. Theoretical testing time in FR2 is reduced, but practical testing time might be increased due to higher technical requirements on the TE and test environment in FR2. Hence we prefer option 2 (no FR2 requirements). |
| Samsung | Issue 3-2 The current URLLC requirement is prioritized with Rel-15 URLLC RAN1 feature. Based on TS 38.824, all the evaluation for Rel-15 enable use case in RAN1 are targeting carrier frequency with 4GHz and FDD, or 700MHz and FDD. In that sense, the typical Rel-15 URLLC scenario is FR1. We should define the requirements based on the typical scenario for URLLC, while not to duplicate the requirements specified for eMBB scenario. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 CQI requirements | Tentative agreements: None Candidate options: Whether to create ultra-low BLER CQI requirements: - Option 1: Create ultra-low BLER CQI requirements (NTT DoCoMo, Qualcomm) - Option 2: Do not create ultra-low BLER CQI requirements (Intel, Huawei, Ericsson, Apple) Recommendations for 2nd round: No clear consensus; continue discussion in second round. Recommended as a candidate to discuss during call. |
| Sub-topic#3-2 FR2 requirements | Tentative agreements: None Candidate options: Whether to create FR2 test: - Option 1: Create FR2 requirements and tests for ultra-low BLER (Intel, NTT DoCoMo) - Option 2: Do not create FR2 requirements and tests for ultra-low BLER (Huawei, Ericsson, Nokia, Samsung, Qualcomm) Recommendations for 2nd round: No clear consensus; continue discussion in second round. Recommended as a candidate to discuss during call. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Recommended to use a single WF for the whole thread to capture conclusions at the end of the 2nd round. | Moderator (Ericsson) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |