3GPP TSG-RAN WG4 Meeting #94-e-Bis		                    R4-2005604

Online, 20th April – 30th April 2020

Source: 	Keysight Technologies, Rohde & Schwarz

Title: 	WF on EVM equalizer calculation for NR

Agenda Item:	4.7.1

Document for:	Approval

# Introduction

In RAN4#94eBis meeting, issue raised by R4-2003458 on NR BS EVM equalizer moving average modification discussed.

TE venders, Keysight and Rohde & Schwarz raised this modification because of impact of leaving current text in [1-3] is not small on measured EVM result when RB allocation has gap inside channel band.

# Discussion in RAN4#94eBis

Comments and feedback received on proposed modification text.

In summary, comments are on following three points;

- Agreement  EVM equalizer inaccuracies caused by gap of DM-RS allocation due to low RB allocation

  - Issue presents most significantly in TM2

- Modification wording on moving average to be fine tuned

- Clarification on detail of moving average calculation at the edge

State at the end of meeting on proposed modification (not conclusion);

- (in description text of step 3 on equalizer description, blue text shows modification)

The moving average window size is 19 and averaging is over the DM-RS subcarriers in the allocated RBs. For DM-RS subcarriers at or near the edge of a set of contiguously allocated RBs or when the allocation size is smaller than the moving average window, the window size is reduced accordingly as per figure L.6-1.

- Add clarification text in the figure for the case of smaller RBs allocation than moving average size

Figure L.6-1: Reference subcarrier smoothing in the frequency domain

# Way forward

- Proposal 1; Further discuss and finalize wording modification in next RAN#4 meeting (RAN4#95-e)
                   Interested companies are encouraged to provide further comment and feedback.

- Proposal 2; TE venders are to provide clarification on calculation detail.

# References

[1] TS38.104 v15.9.0 (2020-03)

[2] TS38.141-1 v15.5.0 (2020-03)

[3] TS38.141-2 v15.5.0 (2020-03)