3GPP TSG-RAN WG4 Meeting #95-e                                                             R4-2006289

# Electronic Meeting, 25 May – 5 June, 2020

Source: 	CATT

Title: 	Open issue on IMT parameters for other frequency ranges below 6GHz

Agenda item:	11.2

Document for:	Discussion

- Introduction

TSG RAN received the LS in [1] from ITU-R, requesting parameters for IMT for sharing and compatibility studies. There were some discussions on the parameters for the following frequency ranges and a WF was approved [1]

470-694 MHz, 694-960 MHz, 1710–1885 MHz, 1885-1980 MHz, 2010-2025 MHz, 2110-2170 MHz, 2500-2690 MHz, 3300-3400 MHz, 3600-3800 MHz, 4800-4990 MHz

This contribution will give analysis on the open issue in the WF.

- Discussion

As agreed in the WF [1], the open issues on antenna characteristics for the frequency ranges below 6 GHz are shown as follows:

- IMT parameters not yet agreed: - BS Average output power: TBD - UE Average output power: TBD - AAS BS antenna characteristics not yet agreed: - Scenarios included: [Small cell Indoor/Indoor Urban] - Element gain: [8 dBi],[6 dBi],[related element spacing and beamwidth] [all scenarios] - H/V beamwidth of single element (degrees): [80/65], [90/65], [90/90] - Antenna array configuration: [8x8], [8x4], [16x8],[12x8],[12x8x2] - H/V radiating element spacing: [0.6λ/0.9λ], [0.5λ/0.9λ], [0.5λ/0.5λ] , [0.5λ/0.7λ] - Conducted power (before Ohmic loss) per antenna element: [25/28/31 dBm/100 MHz], [43 to 46 dBm],[only one value?] - BS vertical coverage range: [90 - 100°, including mechanical/electrical/beamforming tilt]

BS Average output power

Propose not to reply this parameter since this is not a defined requirement in 3GPP.

UE Average output power

Propose not to reply this parameter since this is not a defined requirement in 3GPP. It is a simulation output for UE.

Proposal 1: Do not reply BS/UE average output power.

Antenna configuration

Because NR CSI-RS supports up to 64 ports, we think it is reasonable and enough to choose 8*8 antenna configuration as baseline for WA sub-urban and WA urban.

Proposal 2: Choose 8*8 antenna configuration.

Element parameters

According to [1], the following candidates were discussed for element parameters,

  - Element gain: [8 dBi],[6 dBi],[related element spacing and beamwidth] [all scenarios]

  - H/V beamwidth of single element (degrees): [80/65], [90/65], [90/90]

  - H/V radiating element spacing: [0.6λ/0.9λ], [0.5λ/0.9λ], [0.5λ/0.5λ], [0.5λ/0.7λ]

Generally, due to mutual coupling in the antenna array, smaller element spacing can achieve wider 3dB beamwidth and smaller gain of radiating element than larger element spacing. And wider 3dB beamwidth for beamforming can achieve lower (better) sidelobe level, which can be used to reduce interference to other users or other systems. Narrower 3dB beamwidth for beamforming can achieve bigger (better) directivity gain, which can be used to improve the coverage distance. Since the motivation is for ITU-R co-existence study between IMT and other services, we think the antenna parameters should well consider the motivation. For 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz frequency ranges, the co-existence with space service is the main concern, we proposed to use [0.5/0.7] as the element spacing in horizontal and vertical. 0.7lamda in vertical plane is a good compromise between coverage and sidelobe performance. For frequency ranges below 5GHz, we are fine to use any one from [0.5, 0.7], [0.5, 0.9] and [0.6, 0.9] as the element spacing.

Proposal 3: The following element parameters are recommended for frequency ranges below 6GHz

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Horizontal radiating element spacing | 0.5λ | 0.5λ |
| Vertical radiating element spacing | 0.9λ | 0.9λ |
| Hroizontal 3dB beamwidth(degree) | 90 | 90 |
| Vertical 3dB beamwidth(degree) | 65 | 65 |
| Radiating element gain (GE,max) (dBi) | 6.5 | 6.5 |

Additional parameters

The down tilt angle for sub-urban is usually about 9 degrees; while down tilt angle for urban is usually about 12 degree. The vertical 3dB beamwidth of array beam after beamforming is about 10 degree (75/8). So the vertical coverage range for sub-urban is 94 to 104 degree, and vertical coverage range for urban is 97 to 107 degree. Generally, TRP is the base station output power declared by the base station manufacturer, it can be decided according to the link budget by considering the intended coverage and beamforming gain etc. So, we propose

Proposal 4：Adopt the following additional parameters for Sub-urban and Urban.

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Vertical coverage range (degrees) | 94 to 104 | 97 to 107 |
| BS output power, TRP (dBm) | Decide according to 38.104 and link budget. | Decide according to 38.104 and link budget |

- Conclusion

This contribution further discussed the BS antenna Characteristics and gave the following proposals.

Proposal 1: Do not reply BS/UE average output power.

Proposal 2: Choose 8*8 antenna configuration.

Proposal 3: The following element parameters are recommended for frequency ranges below 5GHz.

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Horizontal radiating element spacing | 0.5λ | 0.5λ |
| Vertical radiating element spacing | 0.9λ | 0.9λ |
| Hroizontal 3dB beamwidth(degree) | 90 | 90 |
| Vertical 3dB beamwidth(degree) | 65 | 65 |
| Radiating element gain (GE,max) (dBi) | 6.5 | 6.5 |

Proposal 4：Adopt the following additional parameters for Sub-urban and Urban.

| Parameter | Wide area Sub-urban | Wide area Urban |
| --- | --- | --- |
| Vertical coverage range (degrees) | 94 to 104 | 97 to 107 |
| BS output power, TRP (dBm) | Decided according to 38.104 and link budget. | Decided according to 38.104 and link budget |

- References

- R4-2005170,WF on LS Reply to ITU-R WP5D for the frequency ranges below 6 GHz,Ericsson,RAN4#94-ebis