3GPP TSG-RAN WG4 Meeting # 95-e                                                                         R4-2006293

# Electronic Meeting, 25 May -5 June, 2020

Agenda Item:	4.6.3

Source: 	CATT

Title: 	WRC-19 protection requirements for EESS operation in 36-37GHz

Document for:	Approval

Introduction

There was a discussion on additional requirement for EESS operating in the band 36-37GHz based on the input in [1]. A WF on how to handle the requirement was approved in [2] where companies are encouraged to input for the following items

- Applicability for NR bands

- Applicability of limits

- Alternatives for how to define a requirement (OBUE/spurious, frequency ranges, wording, etc.)

- Expression as generic or band specific limit

- Possible draft CRs:

- Core requirements in TS 38.104

- Test requirement in TS 38.141-2

This paper will give our view on how to define the EESS protection for Band n260.

Discussion

36--37GHz EESS protection limits in resolution com4/9 of WRC-19

As pointed out in [1], considering the EESS service running currently in 36--37GHz, for compatibility and co-existence perspective, the resolution com4/9 of WRC-19 has given the requirement for IMT system operating in frequency band 37--43.5GHz as shown below in table 1.

Table 1

| Frequency band for the EESS (passive) | Frequency band for IMT stations | Unwanted emission mean power for IMT stations 1 | Recommended limits for IMT stations1 |
| --- | --- | --- | --- |
| 36-37 GHz | 37-40.5 GHz | −43 dB(W/MHz) and −23 dB(W/GHz) within the 36-37 GHz frequency band | −30 dB(W/GHz) |

1 	The unwanted emission power level is considered in terms of total radiated power (TRP). The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere.

Applicability for NR bands

Currently in 3GPP specifications, 4 FR2 bands are specified as shown in Table 2. It is seen that Band n260 completely falls in the concerning IMT frequency range in com4/9. So the EESS protection limits in Table 1 are applicable for band n260.

If other frequency bands are introduced in the frequency range 37-40.5GHz or overlapped with this frequency range, such limits are also applicable.

Table 2 operating band in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

Proposal 1: 	It is proposed to define a band specific requirement for Band n260 for protection of EESS operating in the range 36-37GHz.

Applicability of limits

Regarding the form of the limits, conversion from dBW to dBm may be needed since all power related requirements are specified in terms of dBm in the current specification. The unwanted emission mean power for Band n260 should be less than 7 dBm/GHz in the frequency rang 36--37GHz and -13dBm/MHz in all other frequency range.

Proposal 2: 	The unwanted emission mean power for Band n260 should be less than 7 dBm/GHz in the frequency rang 36--37GHz and -13dBm/MHz in all other frequency range.

How to define the requirements in spec

According to the definition of ΔfOBUE outside the downlink operating band in FR2, the EESS operating frequency range 36-37GHz falls completely in the OBUE frequency range immediately outside the operating band. We propose to create a new clause in 9.7.4.3.4 Additional requirements and define the additional limits for EESS protection in the new clause.

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 3250 MHz | 1500 |

Proposal 3: 	It is proposed to create a new clause in 9.7.4.3.4 Additional requirements for EESS protection requirement.

Draft CRs are provide in [3-6] based on the analysis in this paper.

Conclusion

In this contribution, we provided analysis for protection of EESS operation in 36-37GHz and gave the following proposals.

Proposal 1: 	It is proposed to define a band specific requirement for Band n260 for protection of EESS operating in the range 36-37GHz.

Proposal 2: 	The unwanted emission mean power for Band n260 should be less than 7 dBm/GHz in the frequency rang 36--37GHz and -13dBm/MHz in all other frequency range.

Proposal 3: 	It is proposed to create a new clause in 9.7.4.3.4 Additional requirements for EESS protection requirement.

- References

R4-2004089, on the EESS protection for 36-37Ghz outcome of WRC-19, ZTE

R4-2005461, Way Forward on EESS protection at 36 GHz, Ericsson, ZTE