3GPP TSG-RAN WG4 Meeting #95-e                                                         R4-2006404

Online, 25th May-5th June, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-21: DC_1-41_n77-n257

Agenda item:			8.7.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-21 to include DC_1-41_n77-n257 according to the request in [1].

# 2. Reference

- RP-200080, Revised WID on LTE x bands (xDL/1UL, x=1,2,3,4) and NR 2 bands (2DL/1UL) EN-DC in Rel-16.

# 3. Text Proposal

<Start of Text Proposal>

## 7.x	DC_1-41_n77-n257

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Based on the co-existence studies of DC_1_n77A, DC_41_n77A, DC_1_n257A and DC_41_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.x.4	∆TIB and ∆RIB values

For DC_1-41_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-41_n77 are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n77-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n77-n257 | 1 | 0 |
|  | 41 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

### 7.x.5	MSD

As mentioned in 7.x.3, there is no need to specify additional MSD requirement for this UL DC configuration.

<End of Text Proposal>