3GPP TSG-RAN WG4 Meeting #95-e                                                         R4-2006419

Online, 25th May-5th June, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-31-11: DC_1-11-18_n78

Agenda item:			8.5.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-31-11 to include DC_1-11-18_n78 according to the request in [1].

# 2. Reference

- RP-200102, Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_1-11-18_n78

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11-18_n78 | DC_1-11-18 | n78 | No |

### 5.1.x.2	Configurations for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A | CA_1A-11A-18A | n78 |

### 5.1.x.3		∆TIB and ∆RIB values

Table 5.1.x.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n78 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.x.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n78 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

<End of Text Proposal>