3GPP TSG-RAN4 Meeting #95-e	R4-2006462

# Electronic Meeting, 25-29 May, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0211 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to Foffset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-06-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no definition of values for Foffset, RAT as we see for MSR BS in 37.141 Throughout the document in many instances is used FOffset-RAT instead of Foffset, RAT. Furthermore the term FOffset, which is not defined, is used in at least two instances. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to Foffset, RAT and table of values. Correct the note on raster alignment Editorial corrections: FOffset-RAT is replaced with Foffset, RAT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion around Foffset. Editorial mistakes in specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.5, 4.11.1, 4.11.2.1.2, 4.11.2.1.3, 4.11.2.2.2, 4.11.2.3.2, 4.11.2.4.2, 4.11.2.5.2, 4.11.2.5.3, 4.11.2.6.2, 4.11.2.9.2, 4.11.2.10.2, 4.11.2.11.2, 4.11.2.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------- start of changes -------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Roll-off factor

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth (for E-UTRA)

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink

BWtot	Total RF bandwidth

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation

f	Frequency

f	Separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax	The largest value of f used for defining the requirement

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset	Separation between the Base Station RF bandwidth edge frequency and the centre of the measuring filter

f_offsetmax	The maximum value of f_offset used for defining the requirement

FBW RF,high	Upper RF bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low	Lower RF bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC,high	Centre frequency of the highest transmitted/received carrier

FC,low	Centre frequency of the lowest transmitted/received carrier

Fedge_low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset, RAT

Fedge_high	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset, RAT

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fuw	Frequency offset of unwanted signal

Ncells	A declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all TAB connectors supporting the operating band

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

NRXU,active	The number of active receiver units. The same as the [number of receiver diversity branches] to which compliance is declared for chapter 8 performance requirements.

NRXU,counted	The number of active receiver units that are taken into account for unwanted emission scaling.

NRXU,countedpercell	The number of active receiver units that are taken into account for unwanted emissions scaling per cell

NTXU, active	The number of active transmitter units

NTXU,counted	The number of active transmitter units , as calculated in subclause 6.1, that are taken into account for conducted TX power limit

NTXU,countedpercell	The number of active transmitter units  that are taken into account for emissions scaling per cell

PEM,B32,ind	Declared emission level in Band 32, ind=a, b, c, d, e (see table 4.10, D6.8)

Pmax,c,cell	The maximum carrier output power per TAB connector TX min cell group

Pmax,c,TABC	The maximum carrier output power per TAB connector

PRated,c,TABC	The rated carrier output power per TAB connector

PRated,c,sys	The sum of PRated,c,TABC for all TAB connectors for a single carrier

PRated,t,group	The sum of PRated,t,TABC for all TAB connectors belonging to a specified group

PRated,t,TABC	The rated total output power per TAB connector

PREFSENS	Reference Sensitivity power level

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

Figure 3.2-1: Illustration of RF bandwidth related symbols and definitions for Multi-standard Radio

Figure 3.2-2: Illustration of RF bandwidth related symbols and definitions
for non-contiguous Multi-standard Radio

Figure 3.2-3: Illustration of maximum Radio Bandwidth and
Total RF bandwidth for Multi-band Multi-standard Radio

--------- end of changes -------

--------- start of changes -------

## 4.5	Operating bands and band categories

The operating bands and band categories for AAS BS are the same as for non-AAS BS, as described in 3GPP TS 37.104 [5]. In addition, band category aspects described in TS 37.141, subclauses 4.4.1, 4.4.2 and 4.4.3, shall apply.

NOTE 1:	AAS BS does not support GSM, but BC2 is still applicable for protection of/against GSM operation in BC2 operating bands.

NOTE 2:	AAS BS does not support Band 46 (and all its sub-bands defined in 3GPP TS 36.104 [12], subclause 5.5) operation, but Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation.

NOTE 3:	Bands in BC1 and BC2 categories are also used for NB-IoT operation. NB-IoT is not applicable for AAS BS.

--------- end of changes -------

--------- start of changes -------

## 4.11	Test signal configurations for AAS BS

### 4.11.1	General

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.10.

For test contiguous spectrum operation configurations used in receiver tests only the carriers in the outermost frequency positions in the TAB connector Base Station RF Bandwidth need to be generated by the test equipment. For non-contiguous spectrum operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.

The applicable test models for generation of the carrier transmit test signal are defined in subclause 4.12.2.

NOTE:	If required, carriers are shifted to align with the channel raster.

### 4.11.1a	NR Test signal used to build Test Configurations

The signal's Channel Bandwidth and Subcarrier spacing used to build NR Test Configurations shall be selected according to table 4.11.1a-1.

--------- end of changes -------

--------- start of changes -------

### 4.11.2	Test signal configurations

#### 4.11.2.1	ATC1: UTRA multicarrier operation

##### 4.11.2.1.1	General

The purpose of ATC1 is to test UTRA multi-carrier aspects.

##### 4.11.2.1.2	ATC1a generation

ATC1 should be constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18).

-	Place one UTRA FDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA FDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	For transmitter tests, alternately place a UTRA FDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the TAB connector does not support more carriers. The nominal carrier spacing defined in subclause 4.6 shall apply.

-	The carrier(s) may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.1.3	ATC1b generation

ATC1b is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18).

-	Place one UTRA TDD carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA TDD carrier adjacent to the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	For transmitter tests, alternately place a UTRA TDD carrier adjacent to the already placed carriers at the low and high Base Station RF Bandwidth edges until there is no more space to fit a carrier or the TAB connector does not support more carriers. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.1.4	ATC1 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for UTRA according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.2	ANTC1: UTRA FDD multicarrier non-contiguous operation

##### 4.11.2.2.1	General

The purpose of ANTC1 is to test UTRA FDD multicarrier non-contiguous aspects.

##### 4.11.2.2.2	ANTC1 generation

The purpose of NTC1a is to test UTRA multicarrier non-contiguous aspects. NTC1a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (see table 4.10-1, D6.19) of the TAB connector. The station RF bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation.

-	For transmitter tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	For receiver tests, place one UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and one UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. For single-band operation, if the maximum Base Station RF Bandwidth for non-contiguous operation is at least 35 MHz and the TAB connector supports at least 4 UTRA FDD carriers, place a UTRA FDD carrier adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in subclause 4.6 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset, RAT for the carrier adjacent to the sub-block gap.

-	The UTRA FDD carrier in the lower sub-block may be shifted maximum100 kHz towards lower frequencies and the UTRA FDD carrier in the upper sub-block may be shifted maximum100 kHz towards higher frequencies to align with the channel raster.

##### 4.11.2.2.3	ANTC1 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals PRated,RAT,TABC according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.3	ATC2: E-UTRA multicarrier operation

##### 4.11.2.3.1	General

The purpose of ATC2a is to test E-UTRA multi-carrier aspects excluding CA occupied bandwidth.

The purpose of ATC2b is to test E-UTRA Contiguous CA occupied bandwidth.

##### 4.11.2.3.2	ATC2a generation

ATC2a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18) of the TAB connector.

-	Select the narrowest supported E-UTRA carrier and place it adjacent to the low Base Station RF Bandwidth edge. Place a 5 MHz E-UTRA carrier adjacent to the high Base Station RF Bandwidth edge. The specified Foffset, RAT - shall apply.

-	For transmitter tests, select as many 5 MHz E-UTRA carriers that the TAB connector supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The specified Foffset, RAT  shall apply.

-	If 5 MHz E-UTRA carriers are not supported by the TAB connector the narrowest supported channel bandwidth shall be selected instead.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the TAB connector (see table 4.10-1, D6.29). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.11.2.3.3	ATC2b generation

ATC2b is constructed on a per band basis using the following method:

-	All component carrier combinations supported by the TAB connector, which have different sum of channel bandwidth of component carrier, shall be tested. For all component carrier combinations which have the same sum of channel bandwidth of component carriers, only one of the component carrier combinations shall be tested.

-	Of all component carrier combinations which have same sum of channel bandwidth of component carrier, select those with the narrowest carrier at the lower Base Station RF Bandwidth edge.

-	Of the combinations selected in the previous step, select one with the narrowest carrier at the upper Base Station RF Bandwidth edge.

-	If there are multiple combinations fulfilling previous steps, select the one with the smallest number of component carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the lowest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the highest carrier.

-	If there are multiple combinations fulfilling previous steps, select the one with the widest carrier being adjacent to the carrier which has been selected in the previous step.

-	If there are multiple combinations fulfilling previous steps, repeat the previous step until there is only one combination left.

-	The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.3.4	ATC2 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for E-UTRA according to the manufacturer's declaration in subclause 4.10.

For a TAB connector declared to support only CA operation (see table 4.10-1, D6.22), set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (PRated,t,TABC) according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.4	ANTC2: E-UTRA multicarrier non-contiguous operation

##### 4.11.2.4.1	General

The purpose of ANTC2 is to test E-UTRA multicarrier non-contiguous aspects.

##### 4.11.2.4.2	ANTC2 generation

ANTC2 is constructed as NTC2 in 3GPP TS 37.141[16] subclause 4.8.2a.1.

The purpose of NTC2 is to test E-UTRA multicarrier non-contiguous aspects. NTC2 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth (D.17).

-	For transmitter tests, place a 5 MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. The specified Foffset, RAT  shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead.

-	For receiver tests, place a 5 MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier adjacent to the lower Base Station RF Bandwidth edge. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead.

-	For single-band operation receiver tests, if the remaining gap is at least 15 MHz plus two times the channel bandwidth used in the previous step and the TAB connector supports at least 4 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to each already placed carrier for each sub-block. The nominal carrier spacing defined in subclause 4.6 shall apply.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset, RAT  for the carrier adjacent to the sub-block gap.

##### 4.11.2.4.3	ANTC2 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.5	ATC3: UTRA and E-UTRA multi RAT operation

##### 4.11.2.5.1	General

The purpose of ATC3 is to test UTRA and E-UTRA multi-RAT aspects.

If the rated total output power and total number of supported carriers are not simultaneously supported in Multi-RAT operations, two instances of ATC3 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The rated total output power and the reduced number of supported carriers at the rated total output power in Multi-RAT operations.

2)	The reduced total output power at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

Tests that use ATC3 shall be performed using both instances 1) and 2) of ATC3.

##### 4.11.2.5.2	ATC3a generation

ATC3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (D6.17) of the TAB connector.

-	Select an FDD UTRA carrier to be placed at the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the TAB connector. The specified Foffset, RAT shall apply.

-	For transmitter tests, alternately add FDD UTRA carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (D6.25) is reached. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.5.3	ATC3b generation

ATC3b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (D6.17) of the TAB connector.

-	Select a UTRA TDD carrier to be placed at the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the TAB connector. The specified Foffset, RAT shall apply.

-	For transmitter tests, alternately add UTRA TDD carriers at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers is reached. The nominal carrier spacing defined in clause 4.6 shall apply.

##### 4.11.2.5.4	ATC3 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals PRated,RAT,TABC according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.6	ANTC3: UTRA and E-UTRA multi RAT non-contiguous operation

##### 4.11.2.6.1	General

The purpose of ANTC3 is to test UTRA and E-UTRA multi RAT non-contiguous aspects.

If PRated,t,TABC and total number of supported carriers are not simultaneously supported in Multi-RAT operations, two instances of ANTC3 shall be generated using the following values for PRated,t,TABC  and the total number of supported carriers:

1)	PRated,t,TAB and the reduced number of supported carriers ( DUID26) at the rated total output power in Multi-RAT operations.

2)	The reduced total output power at the total number of supported carriers in Multi-RAT operations (DUID27) at the total number of supported carriers (DUID25).

If the reduced number of supported carriers is 4 or more, only instance 1) of ANTC3 shall be used in the tests, otherwise both instances 1) and 2) of ANTC3 shall be used in the tests.

##### 4.11.2.6.2	ANTC3a generation

ANTC3a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19).

-	For transmitter tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For receiver tests, place an UTRA carrier at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the TAB connector, the narrowest supported channel bandwidth shall be selected instead. The UTRA FDD may be shifted maximum 100 kHz towards lower frequencies to align with the channel raster.

-	For single-band operation receiver tests, if the remaining gap is at least 20 MHz plus the channel bandwidth of the E-UTRA carrier used in the previous step and the TAB connector supports at least 2 UTRA and 2 E-UTRA carriers, place a E-UTRA carrier of this channel bandwidth adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The UTRA FDD may be shifted maximum 100 kHz towards higher frequencies to align with the channel raster.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset, RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.6.3	ANTC3 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals the PRated,t,TABC according to the cases in subclause 4.11.2.6.1.

#### 4.11.2.7	ATC4: Single carrier for receiver tests

##### 4.11.2.7.1	ATC4a generation

ATC4a is constructed using the following method:

-	Place a single UTRA carrier in the middle of the maximum supported Base Station RF Bandwidth. The carrier may be shifted maximum 100 kHz towards lower frequencies for BRFBW and MRFBW and towards higher frequencies for TRFBW to align with the channel raster.

##### 4.11.2.7.2	ATC4b generation

ATC4b is constructed using the following method:

-	Place the narrowest supported E-UTRA carrier in the middle of the maximum supported Base Station RF Bandwidth.

##### 4.11.2.7.3	ATC4c generation

ATC4c is constructed using the following method:

-	Place a single UTRA TDD carrier in the middle of the maximum supported Base Station RF Bandwidth.

##### 4.11.2.7.4	ATC4d generation

ATC4d is constructed using the following method:

-	Place a single NR carrier as specified in subclause 4.11.1a in the middle of the maximum radiated Base Station RF Bandwidth.

#### 4.11.2.8	Generation of MB-MSR test configurations

##### 4.11.2.8.1	ATC5a: MB-MSR test configuration for full carrier allocation

4.11.2.8.1.1	General

The purpose of ATC5a is to test multi-band TAB connectors, considering maximum supported number of carriers.

4.11.2.8.1.2	ATC5a generation

ATC5a is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.17) of the multi-band TAB connector.

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers by the multi-band TAB connector in each band (see table 4.10-1, D6.25). Carriers shall first be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16) Additional carriers shall next be placed at the edges of the Base Station RF Bandwidths, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.1.2-1, where the declared parameters for multi-band operation shall apply. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	If a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth (DUID9) in the other band.

-	If the sum of the maximum Base Station RF bandwidths of each of the supported operating bands is greater than the declared Total RF Bandwidth BWtot (D6.76) of transmitter/receiver for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Total RF Bandwidth of the TAB connector is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands for the multi-band TAB connector is larger than the declared total number of supported carriers for the declared band combinations (see table 4.10-1, D6.42) of the AAS BS, repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

Table 4.11.2.8.1.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 3A | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- | --- |
| BC1 | ATC1a | ATC2a | ATC3a | ATC6 | ATC1a | ATC2a |
| BC2 | ATC1a | ATC2a | ATC3a | ATC6 | ATC1a | ATC2a |
| BC3 | ATC1b | ATC2a | ATC3b | ATC6 | ATC1b | ATC2a |

4.11.2.8.1.3	ATC5a power allocation

Unless otherwise stated, set the power of each carrier (PRated,c,TABC) in all supported operating bands to the same power so that the sum of the carrier powers equals the rated total output power (PRated,MB,TABC) according to the manufacturer's declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) (PRated,MB,TABC ) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated carrier output power declared for that carrier (PRated,c,TABC), the exceeded power shall, if possible, be reallocated into the other carriers.

##### 4.11.2.8.2	ATC5b: MB-MSR test configuration with high PSD per carrier

4.11.2.8.2.1	General

The purpose of ATC5b is to test multi-band operation aspects considering higher PSD cases with reduced number of carriers and non-contiguous operation (if supported) in multi-band mode.

4.11.2.8.2.2	ATC5b generation

ATC5b is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.16) of the multi-band TAB connector.

-	The allocated Radio Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	The maximum number of carriers is limited to two per band. Carriers shall be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.2.2-1, where the declared parameters for multi-band operation shall apply. Narrowest supported E-UTRA channel bandwidth shall be used in the test configuration. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	For AAS BS supporting CSA4 in the band, if a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to AUTC2 while the remaining carrier shall be placed at the edge of the Maximum Base Station RF Bandwidth in the other band.

-	If the sum of the maximum Base Station RF bandwidths of each of the supported operating bands is greater than the declared Total RF Bandwidth BWtot (D6.76) of transmitter/receiver for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Total RF Bandwidth of the TAB connector is not exceeded and vice versa.

Table 4.11.2.8.2.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 3A | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- | --- |
| BC1 | ANTC1a | ANTC2 | ANTC3a | ANTC6 | ANTC1 | ANTC2 |
| BC2 | ANTC1a | ANTC2 | ANTC3a | ANTC6 | ANTC1 | ANTC2 |
| BC3 | ATC1b | ANTC2 | ANTC3a | ANTC6 | N/A | ANTC2 |

4.11.2.8.2.3	ATC5b power allocation

Unless otherwise stated, set the power of each carrier (PRated,c,TABC) in all supported operating bands to the same power so that the sum of the carrier powers equals the rated total output power (PRated,MB,TABC) according to the manufacturer's declaration.

If the allocated power of a supported operating band(s) exceeds the declared rated total output power of the operating band(s) (PRated,t,TABC) in multi-band operation, the exceeded part shall, if possible, be reallocated into the other band(s). If the power allocated for a carrier exceeds the rated carrier output power declared for that carrier (PRated,c,TABC), the exceeded power shall, if possible, be reallocated into the other carriers.

#### 4.11.2.9	ATC6: E-UTRA and NR multi RAT operation

##### 4.11.2.9.1	General

The purpose of ATC6 is to test E-UTRA and NR multi-RAT aspects.

If the rated total output power and total number of supported carriers are not simultaneously supported in Multi-RAT operations, two instances of ATC6 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The rated total output power and the reduced number of supported carriers at the rated total output power in Multi-RAT operations.

2)	The reduced total output power at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

Tests that use ATC6 shall be performed using both instances 1) and 2) of ATC6.

##### 4.11.2.9.2	ATC6 generation

ATC6 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (D6.17) of the TAB connector.

-	Select a NR carrier as specified in subclause 4.11.1a to be placed at the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	Place a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. If that is not possible use the narrowest E-UTRA carrier supported by the beam. The specified Foffset, RAT  shall apply.

-	For transmitter tests, alternately add NR carriers as specified in subclause 4.11.1a at the low end and 5 MHz E-UTRA carriers at the high end adjacent to the already placed carriers until the Base Station RF Bandwidth is filled or the total number of supported carriers (see table 4.10-1, D9.14) is reached. The nominal carrier spacing defined in subclause 4.6 shall apply.

##### 4.11.2.9.3	ATC6 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals PRated,RAT,TABC according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.10	ANTC6: E-UTRA and NR multi RAT non-contiguous operation

##### 4.11.2.10.1	General

The purpose of ANTC6 is to test E-UTRA and NR multi RAT non-contiguous aspects.

If PRated,t,TABC and total number of supported carriers are not simultaneously supported in Multi-RAT operations, two instances of ANTC6 shall be generated using the following values for PRated,t,TABC  and the total number of supported carriers:

1)	PRated,t,TAB and the reduced number of supported carriers ( DUID26) at the rated total output power in Multi-RAT operations.

2)	The reduced total output power at the total number of supported carriers in Multi-RAT operations (DUID27) at the total number of supported carriers (DUID25).

If the reduced number of supported carriers is 4 or more, only instance 1) of ANTC6 shall be used in the tests, otherwise both instances 1) and 2) of ANTC6 shall be used in the tests.

##### 4.11.2.10.2	ANTC6 generation

ANTC6 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19).

-	For transmitter tests, place an NR carrier as specified in subclause 4.11.1a at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	For receiver tests, place a NR carrier as specified in subclause 4.11.1a at the lower Base Station RF Bandwidth edge and a 5 MHz E-UTRA carrier at the upper Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply. If 5 MHz E-UTRA carriers are not supported by the beam, the narrowest supported channel bandwidth shall be selected instead.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset, RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.10.3	ANTC6 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals the PRated,t,TABC according to the cases in subclause 4.11.2.6.1.

#### 4.11.2.11	ATC7: NR multicarrier operation

##### 4.11.2.11.1	General

The purpose of ATC7 is to test NR multi-carrier aspects excluding CA occupied bandwidth.

##### 4.11.2.11.2	ATC7 generation

ATC7 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for contiguous operation (see table 4.10-1, D6.18) of the TAB connector.

-	Select the NR carrier as specified in subclause 4.11.1a and place it adjacent to the low Base Station RF Bandwidth edge. Place a similar NR carrier adjacent to the high Base Station RF Bandwidth edge. The specified Foffset, RAT -shall apply.

-	For transmitter tests, select as many similar NR carriers that the TAB connector supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.6 shall apply. The specified Foffset, RAT shall apply.

The test configuration should be constructed on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the TAB connector (see table 4.10-1, D6.29). All configured component carriers are transmitted simultaneously in the tests where the transmitter should be on.

##### 4.11.2.11.3	ATC7 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC for NR according to the manufacturer's declaration in subclause 4.10.

For a TAB connector declared to support only CA operation (see table 4.10-1, D6.22), set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (PRated,t,TABC) according to the manufacturer's declaration in subclause 4.10.

#### 4.11.2.12	ANTC7: NR multicarrier non-contiguous operation

##### 4.11.2.12.1	General

The purpose of ANTC7 is to test NR multicarrier non-contiguous aspects.

##### 4.11.2.12.2	ANTC7 generation

ANTC7 is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth for non-contiguous operation (D6.19) of the TAB connector. The Base Station RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum Base Station RF Bandwidth (D.17).

-	For transmitter tests, place a NR carrier as specified in subclause 4.11.1a adjacent to the upper Base Station RF Bandwidth edge and a similar NR carrier adjacent to the lower Base Station RF Bandwidth edge. The specified Foffset, RAT shall apply.

-	For receiver tests, place a similar NR carrier adjacent to the upper Base Station RF Bandwidth edge and a similar NR carrier adjacent to the lower Base Station RF Bandwidth edge.

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset, RAT for the carrier adjacent to the sub-block gap.

##### 4.11.2.12.3	ANTC7 power allocation

Set the power of each carrier to the same power so that the sum of the carrier powers equals Prated,RAT,TABC according to the manufacturer's declaration in subclause 4.10.

--------- end of changes -------