3GPP TSG-RAN4#95	R4-2006499

25th May – 05th June 2020

Source: 	Nokia, AT&T

Title: 					TP for TR 37.716-21-11: DC_29-66_n2 and DC_29-66-66_n2

Agenda Item: 			8.4.2

Document for:	Approval

# 1	Introduction

This is a text proposal to introduce DC_29A-66A_n2A and DC_29A-66A-66A_n2A into TR 37.716-21-11.

# 2 	Text proposal

************************************Start of TP********************************************

## 5.1.X	DC_29-66_n2 and DC_29-66-66_n2

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_29-66_n2 | CA_29-66 | n2 |  |
| DC_29-66-66_n2 | CA_29-66-66 | n2 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_29A-66A_n2A | DC_66A_n2A | CA_29A-66A | n2 |
| DC_29A-66A-66A_n2A | DC_66A_n2A | CA_29A-66A-66A | n2 |

### 5.1.X.3		Co-existence studies

There is no impact from UL configuration DC_66_n2 into DL band of 29.

### 5.1.X.5		∆TIB and ∆RIB values

For DC_29-66_n2 and DC_29-66-66_n2, the TIB,c and RIB,c values are reused from EN-DC combination DC_66_n2, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.x.5	REFSENS requirements

There is no additional REFSENS requirement for this band combination.

************************************End of TP********************************************