3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006515

Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-3: Time mask requirements for switching between 1Tx and 2Tx transmissions for inter-band EN-DC without SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No requirements are defined for UE Tx switching between case 1 and case 2 transmissions for inter-band EN-DC without SUL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE time mask requirements for switching and transient periods are defined based on the following agreed RAN4 documents: R4-1916083 LS on UE capability and RRC signalling for Tx switching R4-1916084 WF on Tx switching between case 1 and 2 R4-2002815 Way forward on RF requirements for Tx switching between two uplink carriers R4-2005664 WF on RF requirements for Tx switching between two uplink R4-2005665 LS on UE capability for Tx switching between two uplink carriers R4-2002816 LS on UE Tx switching period delay and DL interruption Note: RAN1 has not agreed the EN-DC functionality and therefore, the RAN1 approved CR to TS38.214 in R1-2003148 still indicates [TBD – no agreements] for EN-DC. Terminology for single-layer transmission and 2-layer UL-MIMO transmission follows TS38.211. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Tx switching between case 1 and case 2 transmissions is not allowed for inter-band EN-DC without SUL by TS38.101-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.0, 6.3B.4, 6.3B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... TS38.306, TS38.331 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Output power dynamics

Output power dynamics for EN-DC operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively. E-UTRA as specified in TS 36.101 [4]. For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in clause 6.3.1 of TS 38.101-1 [2] and clause 6.3.2 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are set to minimum value. Similarly, OFF power requirements specified in clause 6.3.2 of TS 38.101-1 [2] and clause 6.3.3 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are OFF. The OFF power condition in transmit ON/OFF time mask requirements specified in clause 6.3.3 of TS 38.101-1 [2] and clause 6.3.4 of TS 36.101 [4] is applicable only when all NR and E-UTRA carriers are OFF. If both E-UTRA and NR transition between ON and OFF states simultaneously, the longer transient time shall apply to both. If either E-UTRA or NR is OFF and the other carrier transitions from OFF to ON, then the transiet time associated with that carrier applies.

## 6.3A	Output power dynamics for CA

For inter-band NR CA between FR1 and FR2, output power dynamics as specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

6.3B	Output power dynamics for DC

### 6.3B.0	General

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For inter-band EN-DC, output power dynamics requirement for E-UTRA single carrier and CA operation specified in clauses 6.3 of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.3 of TS 38.101-1 [2] and clause 6.3, 6.3A and 6.3D of TS 38.101-2 [3] apply. For inter-band EN-DC without SUL and UE supporting switching between single-layer transmission with one antenna port on E-UTRA uplink carrier and two-layer transmission with two antenna ports NR uplink carrier the time mask requirements of clause 6.3B.4.1 apply.

### 6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

#### 6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time < 0.5 us for TDM based UL sharing from UE perspective within FR1 time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time < 20 us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

### 6.3B.1a	Output power dynamics for NE-DC with UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR. Unless otherwise specified, the 6.3B.1.1 clauses for NE-DC are applicable.

### 6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability

For intra-band contiguous EN-DC configurations DC_(n)41 and DC_(n)71, and all intra-band non-contiguous EN-DC configurations without dual PA capability, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply. Unless otherwise stated, for other intra-band contiguous EN-DC configurations, the switching time in 6.3B.1.1 applies.

Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

Figure 6.3B.2-2: NR to E-UTRA switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

### 6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability

For both intra-band contiguous and non-contiguous EN-DC with dual PA capability, time masks in Figure 6.3B.3-1 and Figure 6.3B.3-2 shall apply.

Figure 6.3B.3-1: E-UTRA to NR switching time mask for intra-band EN-DC with dual PA capability

Figure 6.3B.3-2: NR to E-UTRA switching time mask for intra-band EN-DC with dual PA capability

### 6.3B.4	Output power dynamics for inter-band EN-DC without SUL

#### 6.3B.4.1	Time mask for switching between single-layer and two-layer transmissions for inter-band EN-DC without SUL

The time mask requirements in this clause apply for the UE indicating support of the switching between single-layer transmission with one antenna port on E-UTRA uplink carrier and two-layer transmission with two antenna ports on NR uplink carriers for inter-band EN-DC without SUL.

The time mask for switching between single-layer transmission with one antenna port on E-UTRA and NR uplink carriers configured in different bands and two-layer transmission with two antenna ports on NR uplink carrier defines the switching period and transient period allowed between single-layer transmission and two-layer transmissions on E-UTRA and NR bands. The switching period, which is located on NR uplink carrier, shall be less than indicated in the UL switching period capability TBD as shown in Figure 6.3B.4.1-1. The transient period on NR uplink carrier shall be less than 10 us and the transient period on E-UTRA uplink carrier less than 20 us as shown in Figure 6.3B.4.1-1.

The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on E-UTRA and NR uplink carriers configured in different bands shall be able to transmit both two-layer and single-layer transmissions on the NR uplink carrier. The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on E-UTRA and NR uplink carriers configured in different bands shall be able to transmit simultaneously single-layer transmission on E-UTRA and NR uplink carriers configured in NR bands without any switching period or transient period as shown in Figure 6.3B.4.1-2.

The UE shall follow the BS scheduling and rank adaptation for switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on the E-UTRA and NR uplink carriers configured in different bands and transmitting simultaneously with single-layer transmission with one antenna port on E-UTRA and NR uplink carriers configured in different bands.

The UE shall meet all the UE maximum output power requirements of the indicated UE power class for all the transmissions.

Figure 6.3B.4.1-1: Time mask for switching between single-layer transmissions with one antenna port on E-UTRA and NR uplink carriers and two-layer transmission with two ports on NR uplink carrier in inter-band EN-DC without SUL

Figure 6.3B.4.1-2: Simultaneous single-layer transmission with one port on E-UTRA uplink carrier and NR uplink carrier without switching and transient periods in inter-band EN-DC

## 6.4	Void