3GPP TSG-RAN WG4 Meeting #95-e                 	R4-2006582

Electronic Meeting, 25 May – 5 June, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 relaxed high reliability and low latency BS demodulation requirement simulation results

Agenda item:	6.9.1.2.2

Document for:	Discussion

Introduction

During the last RAN4#94-bis-e meeting, good progress was made on the topic of NR Rel-16 relaxed high reliability and low latency BS demodulation requirements [1]. With the agreements captured in the WF [2], it is now possible to start simulation campaigns.

In this contribution we provide the results of our NR Rel-16 relaxed high reliability and low latency BS demodulation requirements simulation campaign.

Simulation results relaxed BS high reliability

Parameters

Unless stated explicitly otherwise in any of the result tables, the simulation setup follows the baseline as captured in WF [2] and the previous ones. 
We summarize the settings as follows:

Table 1: Simulation parameters BS high reliability.

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | n2 for 30 kHz SCS FFS 15 kHz SCS |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Propagation condition |  | TDLB100-400 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz for 10 MHz 30 kHz for 40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Maximum number of HARQ transmissions |  | 4 |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |

Results

The simulations are still ongoing at the time of writing this contribution and might be updated during the meeting.

Simulation results BS low latency

Parameters

Unless stated explicitly otherwise in any of the result tables, the simulation setup follows the baseline as captured in WF [2] and the previous ones. 
We summarize the settings as follows:

Table 2: Simulation parameters BS low latency.

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 4 or 7 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | FFS: 0 for 4os, 1 for 7os |
| SCS and BW |  | 15 kHz for 10 MHz 30 kHz for 40 MHz |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| MCS Table |  | Table 3, MCS 5 |
| Propagation condition |  | TDLC300-100 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth For MCS5 |
| Test metric |  | 70% TP or 30% BLER |

  - Results

The simulations are still ongoing at the time of writing this contribution, hence the figure and numbers below are a preliminary snapshot and might be updated during the meeting.

Figure 1: PUSCH low latency simulation results.

Table 3: PUSCH low latency simulation results.

| Results | SCS/CBW | Mapping Type | SNR @ target 70% TP (dB) |  |
| --- | --- | --- | --- | --- |
| Set 3 | 15 kHz /10 MHz | TypeA | -7.2 |  |
|  |  | TypeB | -7.2 |  |
|  | 30 kHz/40 MHz | TypeA | -7.4 |  |
|  |  | TypeB | -7.3 |  |

Conclusion

In this contribution we provide the results of our NR Rel-16 relaxed high reliability and low latency BS demodulation requirements simulation campaigns.

- References

- R4-2005590 Email discussion summary for [94e Bis][219] NR_L1enh_URLLC_Demod_Requirements, Moderator (Huawei, HiSilicon), RAN4#94-e-bis.

- R4-2005528, Way forward on NR URLLC BS performance requirements, Huawei, HiSilicon, RAN4#94-e-bis.