3GPP TSG RAN WG4 Meeting #95-e-Bis		R4-2006587

Online, May 25 - June 5, 2020

Title:	draft Reply LS on power control for NR-DC

Response to:

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact person:	Hisashi Onozawa (Email: hisashi.onozawa_atmark_nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 thanks RAN2 for the LS on power control for NR-DC (R4-2003363/R2-2000294).

RAN4 agreed previously to further study FR2 p-Max in Rel-16 as was communicated to RAN2 in LS (R4-1910262/R2-1912048). RAN4 is aware in the reply LS (R4-1913113/ R2-1914222) of that p-Max is currently ignored and Pcompensation is set 0 for FR2.

In RAN4#95-e meeting, RAN4 agreed to introduce p-Max for FR2 based on TRP (Total Radiated Power) metric for the purpose of interference suppression in operator coordination of unsynchronized networks. Further RAN4 agreed to recommend to RAN2 that Pcompensation is also based on TRP metric.

# 2	Actions

To RAN4

ACTION: RAN4 kindly asks RAN2 to change the range of p-Max for FR2 to (-20..43) and also asks to define Pcompensation factor based on TRP (Total Radiated Power) metric in the following

For FR2, if the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4:

max(PEMAX1 – TRPMAX, 0) – (min(PEMAX2, TRPMAX) – min(PEMAX1, TRPMAX)) (dB);

else:

max(PEMAX1 – TRPMAX, 0) (dB)

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #96 (Cancelled)	24 – 28 Aug 2020				Toulouse, FR

TSG RAN WG4 Meeting #96bis	12 – 16 Oct 2020				Kochi, IN