3GPP TSG-RAN WG4 Meeting #95-e 	DRAFT R4-200xxxx

Electronic Meeting, 25th May. 2020 – 5th June. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Guardband design for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per RAN4 agreement, R4-1916160, specific intra-cell guardbands are defined for wideband operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draftCR is based on the running CR for NR-U R4-2008126. In clause 5.3, clause 5.3F is included with the NR-U requirements related to intra-cell guardbands for 15kHz and 30kHz SCS. Requirements for intra-cell guardbands for 60kHz SCS left TBD and will be added to Table 5.3F.3.1-1 and Table 5.3F.3.1-2 after descussion related to 60kHz SCS have been finalized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The will be no intra-cell guardbands specified for NR-U operation with 15kHz or 30kHz SCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************** Start of Changes ****************************************

## 5.3F	UE channel bandwidth for for operation with shared spectrum channel access

### 5.3F.1	General

Unless otherwise specified the general requirements of clause 5.3 applies for NR-U operation.

### 5.3F.2	Void

### 5.3F.3	Minimum guardband and transmission bandwidth configuration

#### 5.3F.3.1	Nominal intra-cell guard band

If a UE supporting wideband operation is configured with channel bandwidths of 20 MHz or greater as specified in 38.214, the nominal intra-cell guard bands and the corresponding sizes (transmission bandwidth configuration) of the RB sets separated by the said guard bands are as specified in Table 5.3F.3.1-1 for each UE channel bandwidth and sub-carrier spacing for the downlink and uplink. The intra-cell guard band configuration in Table 5.3F.3.1-1 is applicable when the IE intraCellGuardBand in TS 38.331 is not provided.

Table 5.3F.3.1-1: Nominal intra-cell guard bands for wideband operation

| SCS (kHz) | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- | --- |
| 15 | 106 (106) | 105-6-105 (216) | N/A | N/A |
| 30 | 51 (51) | 50-6-50 (106) | 50-6-50-6-50 (162) | 50-6-50-5-50-6-50 (217) |
| 60 | TBD | TBD | TBD | TBD |
| NOTE 1: The intra-cell guard band is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the carrier as indicated by offsetToCarrier. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between parentheses. |  |  |  |  |

For each UE channel bandwidth and sub-carrier spacing given by Table 5.3F.3.1-1, the maximum transmission bandwidth configuration including intra-cell guard band(s), if configured by IE intraCellGuardBands in TS 38.331 in the uplink and/or downlink, and corresponding RB-set(s) shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1 for the uplink and downlink. Minimum requirements specified for wideband operation in Clause 6 and Clause 7 also apply for intra-cell guard bands larger than the nominal sizes in Table 5.3F.3.1-1 as listed in Table 5.3F.3.1-2 for each sub-carrier spacing; each guard band in order of CRB index must be larger than or equal to the corresponding nominal guard band specified in Table 5.3F.3.1-1 for each channel bandwidth.

Table 5.3F.3.1-2: Applicable intra-cell guard bands for wideband operation

| Parameter | Unit | SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz | 60 kHz |
| Intra-cell guard band (size) | PRB | 6,7 | 5,6,7 | TBD |
| Transmission bandwidth (size) of RB-set | PRB | 104,105 | 49,50,51 | TBD |

If the UE is configured with zero intra-cell guard bands by IE intraCellGuardBands in 38.331 in the uplink and/or downlink on a carrier greater than 20 MHz, the maximum transmission bandwidth configuration for the uplink and downlink shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band for the UE channel bandwidth as specified in Table 5.3.3-1.

************************************** End of Changes ***************************************