3GPP TSG-RAN WG4 Meeting # 95-e	R4-2007339

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0275 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#94bis-e and RAN4#95-e into TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_2BLTE_1BNR_3DL2UL |  |  |  |  |  | Date: |  |  | 2020-06-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The core requirements for DC combinations are complete based on the following contributions approved at RAN4#94bis-e: R4-2003123 R4-2003129 R4-2003130 R4-2003131 R4-2003167 R4-2003454 R4-2003457 R4-2003488 R4-2003581 R4-2003582 R4-2003583 R4-2003927 R4-2003929 R4-2003930 R4-2003931 R4-2003932 R4-2003934 R4-2003935 R4-2003936 R4-2004496 R4-2004497 R4-2004498 R4-2004610 R4-2004611 R4-2004612 R4-2005004 R4-2005005 R4-2005006 R4-2005007 R4-2005131 R4-2005133 R4-2005134 R4-2005143 R4-2005146 R4-2005147 R4-2005148 R4-2003440 R4-2003937 R4-2004593 R4-2005154 R4-2005165 The core requirements for DC combinations are complete based on the following contributions approved at RAN4#95-e: R4-2006395 R4-2006498 R4-2006499 R4-2006672 R4-2006679 R4-2006681 R4-2008340 R4-2006689 R4-2006934 R4-2007614 R4-2007615 R4-2007616 R4-2007617 R4-2007618 R4-2008345 R4-2008346 R4-2006394 R4-2006396 R4-2006397 R4-2007619 R4-2007620 R4-2008378 |  |  |  |  |  |  |  |  |
| Summary of change: |  | DC combanations above are added in corresponding clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These DC combanations are not included in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.2, 5.5B.5.2, 6.2B.4.2.3.2, 7.3B.2.3.5.2, 7.3B.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B DC_1A-1A-3A-3A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5 DC_1A-3A_n77C5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n77(2A) | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5 DC_1A-3A_n78C5 DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n78(2A)5 DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A_n3A-n78A | DC_1A_n3A DC_1A_n78A |
| DC_1A-3A_n79A5 DC_1A-3A_n79C5 | DC_1A_n79A DC_3A_n79A |
| DC_1A-5A_n78A5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A-8A_n77A | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A) | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A_n8A-n78A | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(2A) | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A | DC_1A_n78A DC_11A_n78A |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5 DC_1A-19A_n77C5 | DC_1A_n77A DC 19A_n77A |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41 DC_20A_n41 |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 DC_1A-21A_n77C5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A |
| DC_1A-41A_n3A DC_1A-41C_n3A | DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A DC_1A-41C_n28A | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A DC_1A-41C_n79A | DC_1A_n79A |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A DC_1A-42A_n77C DC_1A-42C_n77A DC_1A-42C_n77C DC_1A-42D_n77A DC_1A-42D_n77C DC_1A-42E_n77A DC_1A-42E_n77C | DC_1A_n77A |
| DC_1A-42A_n77(2A) DC_1A-42C_n77(2A) | DC_1A_n77A |
| DC_1A-42A_n78A DC_1A-42A_n78C DC_1A-42C_n78A DC_1A-42C_n78C DC_1A-42D_n78A DC_1A-42D_n78C DC_1A-42E_n78A DC_1A-42E_n78C | DC_1A_n78A |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42D_n79C DC_1A-42E_n79A DC_1A-42E_n79C | DC_1A_n79A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A, |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-5A_n2A | DC_5A_n2A |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-7A_n38A | 2A8 |
| DC_2A-2A-7A_n38A | 2A8 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A_(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A DC_2A-66A-66A_n5A DC_2A-2A-66A-66A_n5A DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A | DC_66A_n25A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A DC_2A-66A-66A_n71A DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A |
| DC_3A_n1A-n77A | DC_3A_n1A DC_3A_n77A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | DC_3A_n1A DC_3A_n78A |
| DC_3A-3A_n1A-n78A | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A_n3A-n77A | DC_3A_n77A DC_3A_n3A2 |
| DC_3A_n3A-n78A | DC_3A_n78A DC_3A_n3A2 |
| DC_3A-5A_n78A5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A |
| DC_3A-5A_n79A | DC_3A_n79A DC_5A_n79A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A DC_3A-7A-7A_n1A DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A DC_3A-7A-7A_n77A DC_3A-3A-7A-7A_n77A | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 DC_3A-7C_n78A5 DC_3C-7C_n78A5 | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-3A-7A_n78A DC_3A-7A-7A_n78A5 DC_3A-3A-7A-7A_n78A | DC_3A_n78A DC_7A_n78A |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n78A |
| DC_3A-8A_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n77A | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77(2A) | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A DC_3C-8A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-3A-8A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n79A | DC_3A_n79A DC_8A_n79A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n77A5 DC_3A-19A_n77C5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6 DC_3C-20A_n28A | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 | DC_3A_n78A DC_20A_n78A |
| DC_3A-21A_n77A5 DC_3A-21A_n77C5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_3C_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n41A | DC_3A_n41 DC_28A_n41 |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n77(2A) | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n78A5 DC_3C-28A_n78A DC_3A-28A_n78C5 | DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5 DC_3C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_3C_n28A |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | DC_3A_n78A |
| DC_3A-38A_n78A | DC_3A_n78A |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A | DC_3A_n40A DC_3A_n41A |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A |
| DC_3A-41A_n28A | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n28A | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A DC_3A-41C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A DC_3A-42A_n77C DC_3A-42C_n77A DC_3A-42C_n77C DC_3A-42D_n77A DC_3A-42D_n77C DC_3A-42E_n77A DC_3A-42E_n77C | DC_3A_n77A |
| DC_3A-42A_n77(2A) DC_3A-42C_n77(2A) | DC_3A_n77A |
| DC_3A-42A_n78A DC_3A-42A_n78C DC_3A-42C_n78A DC_3A-42C_n78C DC_3A-42D_n78A DC_3A-42D_n78C DC_3A-42E_n78A DC_3A-42E_n78C | DC_3A_n78A |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42D_n79C DC_3A-42E_n79A DC_3A-42E_n79C | DC_3A_n79A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A, DC_3A_n80A_ULSUP-TDM_n79A, |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A_(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-66A_n2A DC_5B-66A_n2A | DC_5A_n2A |
| DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | DC_5A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A-66A_n66A DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5A C_7A_n78A DC_7C_n78A |
| DC_7A_n7A-n78A | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A | DC_7A_n1A, DC_8A_n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-8A_n77A | DC_7A_n77A, DC_8A_n77A |
| DC_7A-8A_n78A | DC_7A_n78A, DC_8A_n78A |
| DC_7A-7A-8A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A-13A_n66A DC_7A-7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n78A5 DC_7C-28A_n78A5 | DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5 DC_7C_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_7A-40A_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n38A | 66A9 |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78A DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78A DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_8A_n1A-n78A | DC_8A_n1A DC_8A_n78A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n77A | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A) | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A | DC_8A_n78A DC_11A_n78A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A-42A_n28A | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A DC_8A-42C_n77A | DC_8A_n77A |
| DC_8A-42A_n77(2A) DC_8A-42C_n77(2A) | DC_8A_n77A |
| DC_8A_SUL_n41A-n81A | DC_8A_41A, DC_8A_n81A_ULSUP-TDM_n41A, DC_8A_n81A_ULSUP-FDM |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_12A_(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A | DC_13A_n66A |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A-42A_n77A DC_18A-42C_n77A | DC_18A_n77A |
| DC_18A-42A_n78A DC_18A-42C_n78A | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |
| DC_19A-21A_n77A5 DC_19A-21A_n77C5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-42A_n77A DC_19A-42A_n77C DC_19A-42C_n77A DC_19A-42C_n77C DC_19A-42D_n77A DC_19A-42D_n77C | DC_19A_n77A |
| DC_19A-42A_n78A DC_19A-42A_n78C DC_19A-42C_n78A DC_19A-42C_n78C DC_19A-42D_n78A DC_19A-42D_n78C | DC_19A_n78A |
| DC_19A-42A_n79A DC_19A-42A_n79C DC_19A-42C_n79A DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n28A-n75A6 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n78A DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_21A-28A_n77A DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A_n78A DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A_n79A DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A-42A_n77A DC_21A-42A_n77C DC_21A-42C_n77A DC_21A-42C_n77C DC_21A-42D_n77A DC_21A-42D_n77C DC_21A-42E_n77A DC_21A-42E_n77C | DC_21A_n77A |
| DC_21A-42A_n78A DC_21A-42A_n78C DC_21A-42C_n78A DC_21A-42C_n78C DC_21A-42D_n78A DC_21A-42D_n78C DC_21A-42E_n78A DC_21A-42E_n78C | DC_21A_n78A |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A DC_28A-41C_n79A | DC_28A_n79A DC_41A_n79A |
| DC_28A_n3A-n78A | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A |
| DC_28A-42A_n77A DC_28A-42A_n77C DC_28A-42C_n77A | DC_28A_n77A |
| DC_28A-42A_n78A DC_28A-42A_n78C DC_28A-42C_n78A | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_41A-42A_n77A DC_41A-42C_n77A DC_41C-42A_n77A DC_41C-42C_n77A | DC_41A_n77A |
| DC_41A-42A_n78A DC_41A-42C_n78A DC_41C-42A_n78A DC_41C-42C_n78A | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_48A_(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A_(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A_(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 DC_66A_n78(2A)_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. NOTE 7: Void. NOTE 8: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 9: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |

------------------------------ Modified section ------------------------------

5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n257A2 DC_1A-3A_n257D2 DC_1A-3A_n257E2 DC_1A-3A_n257F2 DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I DC_1A-3A_n257J DC_1A-3A_n257K DC_1A-3A_n257L DC_1A-3A_n257M DC_1A-3C_n257A DC_1A-3C_n257D DC_1A-3C_n257E DC_1A-3C_n257F DC_1A-3C_n257G DC_1A-3C_n257H DC_1A-3C_n257I DC_1A-3C_n257J DC_1A-3C_n257K DC_1A-3C_n257L DC_1A-3C_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257IDC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M |
| DC_1A-5A_n257A2 DC_1A-5A_n257D DC_1A-5A_n257E DC_1A-5A_n257F DC_1A-5A_n257G DC_1A-5A_n257H DC_1A-5A_n257I DC_1A-5A_n257J DC_1A-5A_n257K DC_1A-5A_n257L DC_1A-5A_n257M | DC_1A_n257A DC_5A_n257A |
| DC_1A-7A_n257A2 DC_1A-7A_n257D DC_1A-7A_n257E DC_1A-7A_n257F DC_1A-7A_n257G DC_1A-7A_n257H DC_1A-7A_n257I DC_1A-7A_n257J DC_1A-7A_n257K DC_1A-7A_n257L DC_1A-7A_n257M | DC_1A_n257A DC_7A_n257A |
| DC_1A-7A-7A_n257A2 DC_1A-7A-7A_n257D DC_1A-7A-7A_n257E DC_1A-7A-7A_n257F DC_1A-7A-7A_n257G DC_1A-7A-7A_n257H DC_1A-7A-7A_n257I DC_1A-7A-7A_n257J DC_1A-7A-7A_n257K DC_1A-7A-7A_n257L DC_1A-7A-7A_n257M | DC_1A_n257A DC_7A_n257A DC_7A-7A_n257A |
| DC_1A-8A_n257A2 DC_1A-8A_n257D DC_1A-8A_n257E DC_1A-8A_n257F DC_1A-8A_n257G DC_1A-8A_n257H DC_1A-8A_n257I DC_1A-8A_n257J DC_1A-8A_n257K DC_1A-8A_n257L DC_1A-8A_n257M | DC_1A_n257A DC_8A_n257A |
| DC_1A-11A_n257A DC_1A-11A_n257D DC_1A-11A_n257G DC_1A-11A_n257H DC_1A-11A_n257I | DC_1A_n257A DC_1A-257G DC_1A-257H DC_1A-257I DC_11A_n257A DC_11A-257G DC_11A-257H DC_11A-257I |
| DC_1A-18A_n257A2 DC_1A-18A_n257D DC_1A-18A_n257E DC_1A-18A_n257F DC_1A-18A_n257G DC_1A-18A_n257H DC_1A-18A_n257I DC_1A-18A_n257J DC_1A-18A_n257K DC_1A-18A_n257L DC_1A-18A_n257M | DC_1A-257A DC_1A-257G DC_1A-257H DC_1A-257I DC_18A_n257A DC_18A-257G DC_18A-257H DC_18A-257I |
| DC_1A-19A_n257A2 DC_1A-19A_n257D2 DC_1A-19A_n257E2 DC_1A-19A_n257F2 DC_1A-19A_n257G DC_1A-19A_n257H DC_1A-19A_n257I DC_1A-19A_n257J DC_1A-19A_n257K DC_1A-19A_n257L DC_1A-19A_n257M | DC_1A_n257A DC_1A-257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-21A_n257A2 DC_1A-21A_n257D2 DC_1A-21A_n257E2 DC_1A-21A_n257F2 DC_1A-21A_n257G DC_1A-21A_n257H DC_1A-21A_n257I DC_1A-21A_n257J DC_1A-21A_n257K DC_1A-21A_n257L DC_1A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_1A-28A_n257A2 DC_1A-28A_n257D2 DC_1A-28A_n257E2 DC_1A-28A_n257F2 DC_1A-28A_n257G2 DC_1A-28A_n257H2 DC_1A-28A_n257I2 | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-41A_n257A DC_1A-41A_n257D DC_1A-41A_n257E DC_1A-41A_n257F DC_1A-41A_n257G DC_1A-41A_n257H DC_1A-41A_n257I DC_1A-41A_n257J DC_1A-41A_n257K DC_1A-41A_n257L DC_1A-41A_n257M DC_1A-41C_n257A DC_1A-41C_n257D DC_1A-41C_n257E DC_1A-41C_n257F DC_1A-41C_n257G DC_1A-41C_n257H DC_1A-41C_n257I DC_1A-41C_n257J DC_1A-41C_n257K DC_1A-41C_n257L DC_1A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F DC_1A-42A_n257G DC_1A-42A_n257H DC_1A-42A_n257I DC_1A-42A_n257J DC_1A-42A_n257K DC_1A-42A_n257L DC_1A-42A_n257M DC_1A-42C_n257A DC_1A-42C_n257D DC_1A-42C_n257E DC_1A-42C_n257F DC_1A-42C_n257G DC_1A-42C_n257H DC_1A-42C_n257I DC_1A-42C_n257J DC_1A-42C_n257K DC_1A-42C_n257L DC_1A-42C_n257M DC_1A-42D_n257A DC_1A-42D_n257D DC_1A-42D_n257E DC_1A-42D_n257F DC_1A-42D_n257G DC_1A-42D_n257H DC_1A-42D_n257I DC_1A-42D_n257J DC_1A-42D_n257K DC_1A-42D_n257L DC_1A-42D_n257M DC_1A-42E_n257A DC_1A-42E_n257D DC_1A-42E_n257E DC_1A-42E_n257F DC_1A-42E_n257G DC_1A-42E_n257H DC_1A-42E_n257I DC_1A-42E_n257J DC_1A-42E_n257K DC_1A-42E_n257L DC_1A-42E_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_2A-5A_n257A2 | DC_2A_n257A DC_5A_n257A |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M DC_2A-2A-5A_n260A DC_2A-2A-5A_n260G DC_2A-2A-5A_n260H DC_2A-2A-5A_n260I DC_2A-2A-5A_n260J DC_2A-2A-5A_n260K DC_2A-2A-5A_n260L DC_2A-2A-5A_n260M | DC_2A_n260A DC_5A_n260A |
| DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260I DC_5A_n260I |
| DC_2A-5A_n261A DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(A-H) DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(2A-H) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(G-H) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(2G) DC_2A-5A_n261(2H) DC_2A-5A_n261(H-I) | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(2G) | DC_2A_n261G DC_5A_n261G |
| DC_2A-5A_n261(2A-H) DC_2A-5A_n261(G-H) DC_2A-5A_n261(2H) | DC_2A_n261H DC_5A_n261H |
| DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(H-I) | DC_2A_n261I DC_5A_n261I |
| DC_2A-12A_n260A DC_2A-12A_n260G DC_2A-12A_n260H DC_2A-12A_n260I DC_2A-12A_n260J DC_2A-12A_n260K DC_2A-12A_n260L DC_2A-12A_n260M DC_2A-2A-12A_n260A DC_2A-2A-12A_n260G DC_2A-2A-12A_n260H DC_2A-2A-12A_n260I DC_2A-2A-12A_n260J DC_2A-2A-12A_n260K DC_2A-2A-12A_n260L DC_2A-2A-12A_n260M | DC_2A_n260A DC_12A_n260A |
| DC_2A-13A_n257A2 | DC_2A_n257A DC_13A_n257A |
| DC_2A-29A_n260A DC_2A-29A_n260G DC_2A-29A_n260H DC_2A-29A_n260I DC_2A-29A_n260J DC_2A-29A_n260K DC_2A-29A_n260L DC_2A-29A_n260M | DC_2A_n260A |
| DC_2A-13A_n260A2 DC_2A-13A_n260G DC_2A-13A_n260H DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260(2A) DC_2A-13A_n260(3A) DC_2A-13A_n260(4A) DC_2A-13A_n260(5A) DC_2A-13A_n260(6A) DC_2A-13A_n260(2G) DC_2A-13A_n260(2H) DC_2A-13A_n260(A-G) DC_2A-13A_n260(A-H) DC_2A-13A_n260(A-2G) DC_2A-13A_n260(2A-G) DC_2A-13A_n260(2A-2G) DC_2A-13A_n260(3A-G) DC_2A-13A_n260(G-H) | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260I DC_13A_n260I |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(3A) DC_2A-13A_n261(4A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) DC_2A-13A_n261(H-I) | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(A-G) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(2G) | DC_2A_n261G DC_13A_n261G |
| DC_2A-13A_n261(A-H) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(G-H) DC_2A-13A_n261(2H) | DC_2A_n261H DC_13A_n261H |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I DC_2A-14A_n260J DC_2A-14A_n260K DC_2A-14A_n260L DC_2A-14A_n260M DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I DC_2A-2A-14A_n260J DC_2A-2A-14A_n260K DC_2A-2A-14A_n260L DC_2A-2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-2A-30A_n260A DC_2A-2A-30A_n260G DC_2A-2A-30A_n260H DC_2A-2A-30A_n260I DC_2A-2A-30A_n260J DC_2A-2A-30A_n260K DC_2A-2A-30A_n260L DC_2A-2A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-46A_n258A DC_2A-46C_n258A DC_2A-46D_n258A | DC_2A_n258A |
| DC_2A-46A_n258(2A) DC_2A-46A_n258(3A) DC_2A-46A_n258(4A) DC_2A-46A_n258(5A) DC_2A-46C_n258(2A) DC_2A-46C_n258(3A) DC_2A-46C_n258(4A) DC_2A-46C_n258(5A) DC_2A-46D_n258(2A) DC_2A-46D_n258(3A) DC_2A-46D_n258(4A) DC_2A-46D_n258(5A) | DC_2A_n258A |
| DC_2A-46A_n260A DC_2A-46C_n260A DC_2A-46D_n260A DC_2A-46E_n260A DC_2A-46A_n260G DC_2A-46C_n260G DC_2A-46D_n260G DC_2A-46E_n260G DC_2A-46A_n260H DC_2A-46C_n260H DC_2A-46D_n260H DC_2A-46E_n260H DC_2A-46A_n260I DC_2A-46C_n260I DC_2A-46D_n260I DC_2A-46E_n260I DC_2A-46A_n260J DC_2A-46C_n260J DC_2A-46D_n260J DC_2A-46E_n260J DC_2A-46A_n260K DC_2A-46C_n260K DC_2A-46D_n260K DC_2A-46E_n260K DC_2A-46A_n260L DC_2A-46C_n260L DC_2A-46D_n260L DC_2A-46E_n260L DC_2A-46A_n260M DC_2A-46C_n260M DC_2A-46D_n260M DC_2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-2A-46A_n260A DC_2A-2A-46C_n260A DC_2A-2A-46D_n260A DC_2A-2A-46E_n260A DC_2A-2A-46A_n260G DC_2A-2A-46C_n260G DC_2A-2A-46D_n260G DC_2A-2A-46E_n260G DC_2A-2A-46A_n260H DC_2A-2A-46C_n260H DC_2A-2A-46D_n260H DC_2A-2A-46E_n260H DC_2A-2A-46A_n260I DC_2A-2A-46C_n260I DC_2A-2A-46D_n260I DC_2A-2A-46E_n260I DC_2A-2A-46A_n260J DC_2A-2A-46C_n260J DC_2A-2A-46D_n260J DC_2A-2A-46E_n260J DC_2A-2A-46A_n260K DC_2A-2A-46C_n260K DC_2A-2A-46D_n260K DC_2A-2A-46E_n260K DC_2A-2A-46A_n260L DC_2A-2A-46C_n260L DC_2A-2A-46D_n260L DC_2A-2A-46E_n260L DC_2A-2A-46A_n260M DC_2A-2A-46C_n260M DC_2A-2A-46D_n260M DC_2A-2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-46A_n261A DC_2A-46C_n261A DC_2A-46D_n261A DC_2A-46A_n261(2A) DC_2A-46C_n261(2A) DC_2A-46D_n261(2A | DC_2A_n261A |
| DC_2A-66A_n257A2 DC_2A-66A_n257(2A) | DC_2A_n257A DC_66A_n257A |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n260(2A) DC_2A-66A_n260(3A) DC_2A-66A_n260(4A) DC_2A-66A_n260(5A) DC_2A-66A_n260(6A) DC_2A-66A_n260(2G) DC_2A-66A_n260(2H) DC_2A-66A_n260(A-G) DC_2A-66A_n260(A-H) DC_2A-66A_n260(A-2G) DC_2A-66A_n260(2A-G) DC_2A-66A_n260(2A-2G) DC_2A-66A_n260(3A-G) DC_2A-66A_n260(G-H) | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260I DC_66A_n260I |
| DC_2A-2A-66A_n260A DC_2A-2A-66A_n260G DC_2A-2A-66A_n260H DC_2A-2A-66A_n260I DC_2A-2A-66A_n260J DC_2A-2A-66A_n260K DC_2A-2A-66A_n260L DC_2A-2A-66A_n260M DC_2A-66A-66A_n260A DC_2A-66A-66A_n260G DC_2A-66A-66A_n260H DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n261A | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261G DC_2A-66A_n261H DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(2A) DC_2A-66A_n261(3A) DC_2A-66A_n261(4A) DC_2A-66A_n261(2G) DC_2A-66A_n261(2H) DC_2A-66A_n261(A-G) DC_2A-66A_n261(A-H) DC_2A-66A_n261(A-I) DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(A-2G) DC_2A-66A_n261(A-G-H) DC_2A-66A_n261(A-G-I) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(G-H) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261A DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(2H) DC_2A-66A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(A-G) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(2G) DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) | DC_2A_n261G DC_66A_n261G |
| DC_2A-66A_n261(A-H) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(G-H) DC_2A-66A_n261(2H) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(2H) | DC_2A_n261H DC_66A_n261H |
| DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(H-I) | DC_2A_n261I DC_66A_n261I |
| DC_3A-3A-7A_n257A DC_3A-3A-7A_n257D DC_3A-3A-7A_n257E DC_3A-3A-7A_n257F DC_3A-3A-7A_n257G DC_3A-3A-7A_n257H DC_3A-3A-7A_n257I DC_3A-3A-7A_n257J DC_3A-3A-7A_n257K DC_3A-3A-7A_n257L DC_3A-3A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-3A-7A-7A_n257A DC_3A-3A-7A-7A_n257D DC_3A-3A-7A-7A_n257E DC_3A-3A-7A-7A_n257F DC_3A-3A-7A-7A_n257G DC_3A-3A-7A-7A_n257H DC_3A-3A-7A-7A_n257I DC_3A-3A-7A-7A_n257J DC_3A-3A-7A-7A_n257K DC_3A-3A-7A-7A_n257L DC_3A-3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-5A_n257A2 DC_3A-5A_n257D DC_3A-5A_n257E DC_3A-5A_n257F DC_3A-5A_n257G DC_3A-5A_n257H DC_3A-5A_n257I DC_3A-5A_n257J DC_3A-5A_n257K DC_3A-5A_n257L DC_3A-5A_n257M | DC_3A_n257A DC_5A_n257A |
| DC_3A-7A_n257A2 DC_3A-7A_n257D DC_3A-7A_n257E DC_3A-7A_n257F DC_3A-7A_n257G DC_3A-7A_n257H DC_3A-7A_n257I DC_3A-7A_n257J DC_3A-7A_n257K DC_3A-7A_n257L DC_3A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-7A-7A_n257A2 DC_3A-7A-7A_n257D DC_3A-7A-7A_n257E DC_3A-7A-7A_n257F DC_3A-7A-7A_n257G DC_3A-7A-7A_n257H DC_3A-7A-7A_n257I DC_3A-7A-7A_n257J DC_3A-7A-7A_n257K DC_3A-7A-7A_n257L DC_3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-8A_n257A DC_3A-8A_n257D DC_3A-8A_n257E DC_3A-8A_n257F DC_3A-8A_n257G DC_3A-8A_n257H DC_3A-8A_n257I DC_3A-8A_n257J DC_3A-8A_n257K DC_3A-8A_n257L DC_3A-8A_n257M | DC_3A_n257A DC_8A_n257A |
| DC_3A-18A_n257A DC_3A-18A_n257D DC_3A-18A_n257E DC_3A-18A_n257F DC_3A-18A_n257G DC_3A-18A_n257H DC_3A-18A_n257I DC_3A-18A_n257J DC_3A-18A_n257K DC_3A-18A_n257L DC_3A-18A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n257A2 DC_3A-19A_n257D2 DC_3A-19A_n257E2 DC_3A-19A_n257F2 DC_3A-19A_n257G DC_3A-19A_n257H DC_3A-19A_n257I DC_3A-19A_n257J DC_3A-19A_n257K DC_3A-19A_n257L DC_3A-19A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-21A_n257A2 DC_3A-21A_n257D2 DC_3A-21A_n257E2 DC_3A-21A_n257F2 DC_3A-21A_n257G DC_3A-21A_n257H DC_3A-21A_n257I DC_3A-21A_n257J DC_3A-21A_n257K DC_3A-21A_n257L DC_3A-21A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-28A_n257A2 DC_3A-28A_n257D2 DC_3A-28A_n257E2 DC_3A-28A_n257F2 DC_3A-28A_n257G DC_3A-28A_n257H DC_3A-28A_n257I DC_3A-28A_n257J DC_3A-28A_n257K DC_3A-28A_n257L DC_3A-28A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-41A_n257A DC_3A-41A_n257D DC_3A-41A_n257E DC_3A-41A_n257F DC_3A-41A_n257G DC_3A-41A_n257H DC_3A-41A_n257I DC_3A-41A_n257J DC_3A-41A_n257K DC_3A-41A_n257L DC_3A-41A_n257M DC_3A-41C_n257A DC_3A-41C_n257D DC_3A-41C_n257E DC_3A-41C_n257F DC_3A-41C_n257G DC_3A-41C_n257H DC_3A-41C_n257I DC_3A-41C_n257J DC_3A-41C_n257K DC_3A-41C_n257L DC_3A-41C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n257A2 DC_3A-42A_n257D2 DC_3A-42A_n257E2 DC_3A-42A_n257F2 DC_3A-42A_n257G DC_3A-42A_n257H DC_3A-42A_n257I DC_3A-42A_n257J DC_3A-42A_n257K DC_3A-42A_n257L DC_3A-42A_n257M DC_3A-42C_n257A2 DC_3A-42C_n257D2 DC_3A-42C_n257E2 DC_3A-42C_n257F2 DC_3A-42C_n257G DC_3A-42C_n257H DC_3A-42C_n257I DC_3A-42C_n257J DC_3A-42C_n257K DC_3A-42C_n257L DC_3A-42C_n257M DC_3A-42D_n257A2 DC_3A-42D_n257D DC_3A-42D_n257E DC_3A-42D_n257F DC_3A-42D_n257G DC_3A-42D_n257H DC_3A-42D_n257I DC_3A-42D_n257J DC_3A-42D_n257K DC_3A-42D_n257L DC_3A-42D_n257M DC_3A-42E_n257A2 DC_3A-42E_n257D DC_3A-42E_n257E DC_3A-42E_n257F DC_3A-42E_n257G DC_3A-42E_n257H DC_3A-42E_n257I DC_3A-42E_n257J DC_3A-42E_n257K DC_3A-42E_n257L DC_3A-42E_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257ADC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_5A-7A_n257A2 DC_5A-7A_n257D DC_5A-7A_n257E DC_5A-7A_n257F DC_5A-7A_n257G DC_5A-7A_n257H DC_5A-7A_n257I DC_5A-7A_n257J DC_5A-7A_n257K DC_5A-7A_n257L DC_5A-7A_n257M | DC_5A_n257A DC_7A_n257A |
| DC_5A-7A-7A_n257A | DC_5A_n257A DC_7A_n257A |
| DC_5A-7A-7A_n257D DC_5A-7A-7A_n257E DC_5A-7A-7A_n257F DC_5A-7A-7A_n257G DC_5A-7A-7A_n257H DC_5A-7A-7A_n257I DC_5A-7A-7A_n257J DC_5A-7A-7A_n257K DC_5A-7A-7A_n257L DC_5A-7A-7A_n257M | DC_5A_n257A DC_7A_n257A |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_30A_n260A |
| DC_8A-11A_n257A DC_8A-11A_n257D DC_8A-11A_n257G DC_8A-11A_n257H DC_8A-11A_n257I | DC_8A_n257A DC_11A_n257A |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M | DC_5A_n260A DC_66A_n260A |
| DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260I DC_66A_n260I |
| DC_5A-66A-66A_n260A DC_5A-66A-66A_n260G DC_5A-66A-66A_n260H DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260A DC_66A_n260A |
| DC_5A-66A_n261A DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(2G) DC_5A-66A_n261(2H) DC_5A-66A_n261(A-G) DC_5A-66A_n261(A-H) DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(G-H) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261A DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(2H) DC_5A-66A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(A-G) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(2G) DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) | DC_5A_n261G DC_66A_n261G |
| DC_5A-66A_n261(A-H) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(G-H) DC_5A-66A_n261(2H) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(2H) | DC_5A_n261H DC_66A_n261H |
| DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(H-I) | DC_5A_n261I DC_66A_n261I |
| DC_11A-18A_n257A DC_11A-18A_n257G DC_11A-18A_n257H DC_11A-18A_n257I | DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_13A-66A-66A_n260A DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260A DC_13A-66A_n260G DC_13A-66A_n260H DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260I DC_66A_n260I |
| DC_13A-66A-66A_n261A DC_13A-66A-66A_n261G DC_13A-66A-66A_n261H DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261A DC_13A-66A_n261G DC_13A-66A_n261H DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(2A) DC_13A-66A_n261(3A) DC_13A-66A_n261(4A) DC_13A-66A_n261(2G) DC_13A-66A_n261(2H) DC_13A-66A_n261(A-G) DC_13A-66A_n261(A-H) DC_13A-66A_n261(A-I) DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(A-2G) DC_13A-66A_n261(A-G-H) DC_13A-66A_n261(A-G-I) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(G-H) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J) DC_13A-66A_n261(H-I) DC_13A-66A-66A_n261(2A) DC_13A-66A-66A_n261(3A) DC_13A-66A-66A_n261(4A) DC_13A-66A-66A_n261(2G) DC_13A-66A-66A_n261(2H) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(A-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(A-2G) DC_13A-66A-66A_n261(A-G-H) DC_13A-66A-66A_n261(A-G-I) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(A-G) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(2G) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(2G) | DC_13A_n261G DC_66A_n261G |
| DC_13A-66A_n261(A-H) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(G-H) DC_13A-66A_n261(2H) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(2H) | DC_13A_n261H DC_66A_n261H |
| DC_8A-11A_n257A DC_8A-11A_n257D | DC_8A_n257A DC_11A_n257A |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M | DC_12A_n260A DC_66A_n260A |
| DC_12A-66A-66A_n260A DC_12A-66A-66A_n260G DC_12A-66A-66A_n260H DC_12A-66A-66A_n260I DC_12A-66A-66A_n260J DC_12A-66A-66A_n260K DC_12A-66A-66A_n260L DC_12A-66A-66A_n260M | DC_12A_n260A DC_66A_n260A |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I DC_14A-30A_n260J DC_14A-30A_n260K DC_14A-30A_n260L DC_14A-30A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_13A-66A_n257A2 | DC_13A_n257A DC_66A_n257A |
| DC_13A-66A_n260A2 | DC_13A_n260A DC_66A_n260A |
| DC_18A-28A_n257A2 | DC_18A_n257A DC_28A_n257A |
| DC_18A-42A_n257A DC_18A-42A_n257D DC_18A-42A_n257E DC_18A-42A_n257F DC_18A-42A_n257G DC_18A-42A_n257H DC_18A-42A_n257I DC_18A-42A_n257J DC_18A-42A_n257K DC_18A-42A_n257L DC_18A-42A_n257M DC_18A-42C_n257A DC_18A-42C_n257D DC_18A-42C_n257E DC_18A-42C_n257F DC_18A-42C_n257G DC_18A-42C_n257H DC_18A-42C_n257I DC_18A-42C_n257J DC_18A-42C_n257K DC_18A-42C_n257L DC_18A-42C_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_18A-41A_n257A DC_18A-41A_n257G DC_18A-41A_n257H DC_18A-41A_n257I DC_18A-41C_n257A DC_18A-41C_n257G DC_18A-41C_n257H DC_18A-41C_n257I | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_19A-21A_n257A2 DC_19A-21A_n257D2 DC_19A-21A_n257E2 DC_19A-21A_n257F2 DC_19A-21A_n257G DC_19A-21A_n257H DC_19A-21A_n257I DC_19A-21A_n257J DC_19A-21A_n257K DC_19A-21A_n257L DC_19A-21A_n257M | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_19A-42A_n257A2 DC_19A-42A_n257D2 DC_19A-42A_n257E2 DC_19A-42A_n257F2 DC_19A-42A_n257G2 DC_19A-42A_n257H2 DC_19A-42A_n257I2 DC_19A-42C_n257A2 DC_19A-42C_n257G2 DC_19A-42C_n257H2 DC_19A-42C_n257I2 DC_19A-42D_n257D2 DC_19A-42D_n257E2 DC_19A-42D_n257F2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-28A_n257A2 DC_21A-28A_n257D2 DC_21A-28A_n257E2 DC_21A-28A_n257F2 | DC_21A_n257A DC_21A_n257D DC_28A_n257A DC_28A_n257D |
| DC_21A-42A_n257A2 DC_21A-42A_n257D2 DC_21A-42A_n257E2 DC_21A-42A_n257F2 DC_21A-42A_n257G DC_21A-42A_n257H DC_21A-42A_n257I DC_21A-42A_n257J DC_21A-42A_n257K DC_21A-42A_n257L DC_21A-42A_n257M DC_21A-42C_n257A2 DC_21A-42C_n257G DC_21A-42C_n257H DC_21A-42C_n257I DC_21A-42C_n257J DC_21A-42C_n257K DC_21A-42C_n257L DC_21A-42C_n257M DC_21A-42D_n257A DC_21A-42D_n257D DC_21A-42D_n257E DC_21A-42D_n257F DC_21A-42D_n257G DC_21A-42D_n257H DC_21A-42D_n257I DC_21A-42D_n257J DC_21A-42D_n257K DC_21A-42D_n257L DC_21A-42D_n257M DC_21A-42E_n257A DC_21A-42E_n257D DC_21A-42E_n257E DC_21A-42E_n257F DC_21A-42E_n257G DC_21A-42E_n257H DC_21A-42E_n257I DC_21A-42E_n257J DC_21A-42E_n257K DC_21A-42E_n257L DC_21A-42E_n257M | DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_28A-41A_n257A DC_28A-41A_n257G DC_28A-41A_n257H DC_28A-41A_n257IDC_28A-41C_n257A DC_28A-41C_n257G DC_28A-41C_n257H DC_28A-41C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n257A2 DC_28A-42A_n257D2 DC_28A-42A_n257G2 DC_28A-42A_n257H2 DC_28A-42A_n257I2 DC_28A-42C_n257A2 DC_28A-42C_n257D2 DC_28A-42C_n257G2 DC_28A-42C_n257H2 DC_28A-42C_n257I2 | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_29A-30A_n260A DC_29A-30A_n260G DC_29A-30A_n260H DC_29A-30A_n260I DC_29A-30A_n260J DC_29A-30A_n260K DC_29A-30A_n260L DC_29A-30A_n260M | DC_30A_n260A |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_66A_n260A |
| DC_30A-66A-66A_n260A DC_30A-66A-66A_n260G DC_30A-66A-66A_n260H DC_30A-66A-66A_n260I DC_30A-66A-66A_n260J DC_30A-66A-66A_n260K DC_30A-66A-66A_n260L DC_30A-66A-66A_n260M | DC_30A_n260A DC_66A_n260A |
| DC_41A-42A_n257A DC_41A-42A_n257D DC_41A-42A_n257E DC_41A-42A_n257F DC_41A-42A_n257G DC_41A-42A_n257H DC_41A-42A_n257I DC_41A-42A_n257J DC_41A-42A_n257K DC_41A-42A_n257L DC_41A-42A_n257M DC_41A-42C_n257A DC_41A-42C_n257D DC_41A-42C_n257E DC_41A-42C_n257F DC_41A-42C_n257G DC_41A-42C_n257H DC_41A-42C_n257I DC_41A-42C_n257J DC_41A-42C_n257K DC_41A-42C_n257L DC_41A-42C_n257M DC_41C-42A_n257A DC_41C-42A_n257D DC_41C-42A_n257E DC_41C-42A_n257F DC_41C-42A_n257G DC_41C-42A_n257H DC_41C-42A_n257I DC_41C-42A_n257J DC_41C-42A_n257K DC_41C-42A_n257L DC_41C-42A_n257M DC_41C-42C_n257A DC_41C-42C_n257D DC_41C-42C_n257E DC_41C-42C_n257F DC_41C-42C_n257G DC_41C-42C_n257H DC_41C-42C_n257I DC_41C-42C_n257J DC_41C-42C_n257K DC_41C-42C_n257L DC_41C-42C_n257M | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_46A-48A_n260A DC_46C-48A_n260A DC_46D-48A_n260A DC_46A-48C_n260A DC_46A-48D_n260A DC_46C-48C_n260A DC_46C-48D_n260A DC_46D-48C_n260A DC_46D-48D_n260A DC_46A-48A_n260(2A) DC_46C-48A_n260(2A) DC_46D-48A_n260(2A) DC_46A-48C_n260(2A) DC_46A-48D_n260(2A) DC_46C-48C_n260(2A) DC_46C-48D_n260(2A) DC_46D-48C_n260(2A) DC_46D-48D_n260(2A) DC_46A-48A_n260(3A) DC_46C-48A_n260(3A) DC_46D-48A_n260(3A) DC_46A-48C_n260(3A) DC_46A-48D_n260(3A) DC_46C-48C_n260(3A) DC_46C-48D_n260(3A) DC_46D-48C_n260(3A) DC_46D-48D_n260(3A) DC_46A-48A_n260(4A) DC_46C-48A_n260(4A) DC_46D-48A_n260(4A) DC_46A-48C_n260(4A) DC_46A-48D_n260(4A) DC_46C-48C_n260(4A) DC_46C-48D_n260(4A) DC_46D-48C_n260(4A) DC_46D-48D_n260(4A) | DC_48A_n260A DC_48C_n260A |
| DC_46A-48A_n261A DC_46C-48A_n261A DC_46D-48A_n261A DC_46A-48C_n261A DC_46A-48D_n261A DC_46C-48C_n261A DC_46C-48D_n261A DC_46D-48C_n261A DC_46D-48D_n261A DC_46A-48A_n261(2A) DC_46C-48A_n261(2A) DC_46D-48A_n261(2A) DC_46A-48C_n261(2A) DC_46A-48D_n261(2A) DC_46C-48C_n261(2A) DC_46C-48D_n261(2A) DC_46D-48C_n261(2A) DC_46D-48D_n261(2A) | DC_48A_n261A DC_48C_n261A |
| DC_46A-66A_n258A DC_46C-66A_n258A DC_46D-66A_n258A | DC_66A_n258A |
| DC_46A-66A_n258(2A) DC_46A-66A_n258(3A) DC_46A-66A_n258(4A) DC_46A-66A_n258(5A) DC_46C-66A_n258(2A) DC_46C-66A_n258(3A) DC_46C-66A_n258(4A) DC_46C-66A_n258(5A) DC_46D-66A_n258(2A) DC_46D-66A_n258(3A) DC_46D-66A_n258(4A) DC_46D-66A_n258(5A) | DC_66A_n258A |
| DC_46A-66A_n260A DC_46C-66A_n260A DC_46D-66A_n260A DC_46E-66A_n260A DC_46A-66A_n260G DC_46C-66A_n260G DC_46D-66A_n260G DC_46E-66A_n260G DC_46A-66A_n260H DC_46C-66A_n260H DC_46D-66A_n260H DC_46E-66A_n260H DC_46A-66A_n260I DC_46C-66A_n260I DC_46D-66A_n260I DC_46E-66A_n260I DC_46A-66A_n260J DC_46C-66A_n260J DC_46D-66A_n260J DC_46E-66A_n260J DC_46A-66A_n260K DC_46C-66A_n260K DC_46D-66A_n260K DC_46E-66A_n260K DC_46A-66A_n260L DC_46C-66A_n260L DC_46D-66A_n260L DC_46E-66A_n260L DC_46A-66A_n260M DC_46C-66A_n260M DC_46D-66A_n260M DC_46E-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_46A-66A_n260(2A) DC_46C-66A_n260(2A) DC_46D-66A_n260(2A) | DC_66A_n260A |
| DC_46A-66A_n261A DC_46C-66A_n261A DC_46D-66A_n261A DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) | DC_66A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

------------------------------ Modified section ------------------------------

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n5 | 0.3 |
| DC_1-3_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n7 | 0.6 |
| DC_1-3_n8 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n8 | 0.3 |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n3-n28 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n38 | 0.5 |
| DC_1-3_n40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_1-3_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n41 | 0.31 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n71 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n71 | 0.3 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1_n3-n78 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5_n79 | 1 | 0.3 |
|  | 5 | 0.3 |
| DC_1-7_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
| DC_1-7_n5 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n5 | 0.3 |
| DC_1-7_n7 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
| DC_1-7_n8 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
| DC_1-7_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7_n40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
| DC_1-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n7-n78 | 1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
| DC_1-8_n77 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_1-8_n78 DC_1_n8-n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
| DC_1-11_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_1-11_n78 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_1-18-n3 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |
| DC_1-18_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-18_n78 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_1-20_n8 | 1 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_1-20_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-20_n38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |
| DC_1-20_n41 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.51 |
|  |  | 1.22 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-28-n3 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |
| DC_1-28_n5 | 1 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_1-28_n7 | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
| DC_1-28_n40 | 1 | 0.6 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
| DC_1-32_n78 | 1 | 0.5 |
|  | n78 | 0.8 |
| DC_1-(n)38 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n38 | 0.5 |
| DC_1_n40-n78 | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n3 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| DC_1-41_n28 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
| DC_1-(n)41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-28_n77 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n78 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
| DC_1-42_n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_SUL_n77-n80 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n80 | 0.6 |
| DC_1_SUL_n77-n84 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n84 | 0.6 |
| DC_1_SUL_n78-n84 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n84 | 0.3 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_1_SUL_n78-n80 | 1 | 0.6 |
|  | n80 | 0.6 |
|  | n78 | 0.8 |
| DC_2-(n)71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_2-4_n38 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_2-4_n41 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n41 | 0.5 |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
| DC_2-5_n71 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | n71 | 0.5 |
| DC_2-7_n38 DC_2-2-7_n38 | 2 | 0.5 |
| DC_2-7_n71 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n71 | 0.6 |
| DC_2-7_n66 DC_2-7-7_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7_n78 | 2 | 0.5 |
|  | 7 | 0.5 |
| DC_2_n7-n78 | 2 | 0.6 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |
| DC_2-12_n2 | 2 | 0.3 |
|  | 12 | 0.3 |
| DC_2_(n)12 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |
| DC_2-12_n66, DC_2-2-12_n66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | n66 | 0.5 |
| DC_2-13_n2 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_2-13_n5 DC_2-2-13_n5 | 2 | 0.3 |
|  | 13 | 0.5 |
|  | n5 | 0.5 |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
| DC_2-14_n2 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n66 | 0.5 |
| DC_2-29_n66 DC_2-2-29_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_2-30_n66, DC_2-2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2_n41-n66 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |
| DC_2-46_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_2-46_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2-48_n12 | 2 | 0.6 |
|  | 48 | 0.3 |
|  | n12 | 0.8 |
| DC_2-48_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_2-48_n71 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n71 | 0.3 |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-66_n12 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n12 | 0.8 |
| DC_2-66_n25 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.9 |
| DC_2-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |
| DC_2-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-66_n71 DC_2_n66-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |
| DC_2-71_n38 DC_2-2-71_n38 | 2 | 0.5 |
|  | 71 | 0.3 |
|  | n38 | 0.5 |
| DC_2-71_n66 DC_2-2-71_n66 | 2 | 0,5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |
| DC_2-71_n78 DC_2-2-71_n78 | 2 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1-n7 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n7 | 0.6 |
| DC_3_n1-n28 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |
| DC_3_n1-n77 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n1-n78 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1-n79 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0.0 |
| DC_3_n3-n77 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n3-n78 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5_n79 | 3 | 0.3 |
|  | 5 | 0.3 |
| DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1 | 3 | 0.3 |
|  | 7 | 0.6 |
|  | n1 | 0.5 |
| DC_3-7_n5 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n5 | 0.3 |
| DC_3-7_n7 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |
| DC_3-7_n8 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n8 | 0.6 |
| DC_3-7_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |
| DC_3-7_n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |
| DC_3-7_n78, DC_3-7-7_n78, DC_3-3-7_n78, DC_3-3-7-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n7-n78 | 3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n1 DC_3-3-8_n1 | 3 | 0.3 |
|  | 8 | 0.3 |
|  | n1 | 0.3 |
| DC_3-8_n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
| DC_3-8_n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_3-8_n78 DC_3-3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n79 | 3 | 0.3 |
|  | 8 | 0.3 |
| DC_3-18-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_3-18-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_3-18-n79 | 3 | 0.3 |
|  | 18 | 0.3 |
| DC_3-19_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n79 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-20_n1 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_3-20_n7 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.5 |
| DC_3-20_n8 | 3 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_3-20_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n38 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |
| DC_3-20_n41 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.51 |
|  |  | 1.22 |
| DC_3-20_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n20-n78 | 3 | 0.5 |
|  | n20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-21_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n5 | 3 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_3-28_n7 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |
| DC_3-28_n40 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
| DC_3-28_n41 | 3 | 0.5 |
|  | 28 | 0.5 |
|  | n41 | 0.31/0.82 |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_3-32_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-38_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-40_n1 | 3 | 0.5 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |
| DC_3_n40-n41 | 3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.53 |
|  |  | 0.84 |
| DC_3_n40-n78 | 3 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| DC_3-41_n28 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
| DC_3-(n)41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3-41_n41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3-41-n77 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| DC_3-41_n78 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| DC_3-41-n79 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
| DC_3_SUL_n41-n80 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
|  | n80 | 0.5 |
| DC_3-42_n28 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
| DC_3-42_n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-42_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-42_n79 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n79 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_SUL_n77-n80 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n80 | 0.6 |
| DC_3_SUL_n77-n84 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n84 | 0.6 |
| DC_3_n78-n79 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_3_SUL_n78-n80 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| DC_3_SUL_n78-n82 | 3 | 0.5 |
|  | n78 | 0.8 |
|  | n82 | 0.3 |
| DC_3_SUL_n78-n84 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |
| DC_5-7_n71 | 5 | 0.5 |
|  | 7 | 0.3 |
|  | n71 | 0.6 |
| DC_5-7_n78, DC_5-7-7_n78, DC_5_n7-n78 | 5 | 0.6 |
|  | 7 or n7 | 0.6 |
|  | n78 | 0.8 |
| DC_5_(n)12 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | n12 | 0.4 |
| DC_5-13_n2 | 5 | 0.5 |
|  | 13 | 0.5 |
|  | n2 | 0.3 |
| DC_5-30_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_5-41_n79 | 5 | 0.3 |
|  | 41 | 0.3 |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_5-66_n5 DC_5-66-66_n5 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_5-66_n66 DC_5-5-66_n66 DC_5-66-66_n66 DC_5-5-66-66_n66 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_5-66_n71 | 5 | 0.5 |
|  | 66 | 0.3 |
|  | n71 | 0.5 |
| DC_5-66_n78 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |
| DC_5-66_n5 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_5-66_n66 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n3-n78 | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n7-n78 | 7 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |
| DC_7-8_n1 DC_7-7-8_n1 | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.5 |
| DC_7-8_n3 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n3 | 0.5 |
| DC_7-8_n77 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_7-8_n78 DC_7-7-8_n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_7-13_n66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
| DC_7-20_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |
| DC_7-20_n3 | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |
| DC_7-20_n8 | 7 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_7-20_n28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_7-20_n78 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_7-28_n3 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |
| DC_7-28_n5 | 7 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_7-28_n7 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n7 | 0.3 |
| DC_7-28_n40 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.6 |
| DC_7-28_n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_7-40_n1 | 7 | 0.8 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |
| DC_7-46_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_7-66_n38 | 66 | 0.5 |
| DC_7-66_n66 DC_7-7-66_n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.5 |
| DC_7-66_n78 DC_7-7-66_n78 DC_7-66-66_n78 DC_7-7-66-66_n78 | 7 | 0.5 |
|  | 66 | 0.5 |
| DC_7_n66-n78 DC_7-7_n66-n78 | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| DC_7_SUL_n78-n80 | 7 | 0.6 |
|  | n80 | 0.6 |
|  | n78 | 0.8 |
| DC_8_n1-n78 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
| DC_8_n3-n28 | 8 | 0.6 |
|  | n3 | 0.3 |
|  | n28 | 0.5 |
| DC_8-11_n3 | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
| DC_8-11_n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_8-11_n78 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_8-20_n78 | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_8-42_n28 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
| DC_8-42_n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_8_SUL_n41-n81 | 8 | 0.3 |
|  | n41 | 0.3 |
|  | n81 | 0.3 |
| DC_8_SUL_n78-n80 | 8 | 0.6 |
|  | n80 | 0.6 |
|  | n78 | 0.8 |
| DC_8_SUL_n78- n81 | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n81 | 0.6 |
| DC_11-18_n77 | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_11-18_n78 | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_12_(n)5 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n7-n78 | 12 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |
| DC_12-30_n2 | 12 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |
| DC_12-30_n66 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_13-46_n5 | 13 | 0.5 |
|  | n5 | 0.5 |
| DC_12-66_n2 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_12-66_n25 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
| DC_12-66_n66 | 12 | 0.8 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_13-48_n2 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |
| DC_13-48_n66 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_13-66_n2 DC_13-66-66_n2 | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_13-66_n48 DC_13-66-66_n48 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |
| DC_13-66_n66 DC_13-66-66_n66 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_14-66_n66 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_18_n3-n78 | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_18-28_n77 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_18-28_n78 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_18-28_n79 | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_18-41_n3 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| DC_18-41_n77 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_18-42_n77 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_18-42_n78 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_18-42_n79 | 18 | 0.3 |
|  | 42 | 0.8 |
| DC_19-21_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-21_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
| DC_19-42_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-42_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n79 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78-n79 | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_20_n1-n28 | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n28 | 0.6 |
| DC_20_n1-n78 | 20 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n3-n78 | 20 | 0.3 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |
| DC_20_n8-n75 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28-n75 | 20 | 0.5 |
|  | n28 | 0.7 |
| DC_20_n28-n78 | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_20-32_n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20-(n)38 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | n38 | 0.3 |
| DC_20-38_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_20_n75-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20_n76-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20_SUL_n78-n80 | 20 | 0.3 |
|  | n80 | 0.5 |
|  | n78 | 0.8 |
| DC_20_SUL_n78-n82 | 20 | 0.6 |
|  | n78 | 0.8 |
|  | n82 | 0.6 |
| DC_20_SUL_n78-n83 | 20 | 0.8 |
|  | n78 | 0.8 |
|  | n83 | 0.8 |
| DC_21-42_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-42_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n79 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78-n79 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | 25 | 0.5 |
|  | 41 | 0.41 |
|  |  | 0.92 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_28_n3-n78 | 28 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_28_n7-n78 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |
| DC_28-41_n77 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_28-41_n78 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_28-41_n79 | 28 | 0.3 |
|  | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_28_n8-n78 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.3 |
| DC_28-42_n77 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_28-42_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_28-42_n79 | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_28_SUL_n78-n83 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n83 | 0.5 |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_30-66_n2 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_41-42_n77 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_41-42_n78 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_46-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_46-66_n25 | 66 | 0.5 |
|  | n25 | 0.5 |
| DC_48_(n)5 | 5 | 0.3 |
|  | 48 | 0.3 |
|  | n5 | 0.3 |
| DC_48_(n)12 | 12 | 0.3 |
|  | n12 | 0.3 |
|  | 48 | 0.3 |
| DC_48-66_n12 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n12 | 0.3 |
| DC_48-66_n71 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n71 | 0.3 |
| DC_48-66_n5 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n5 | 0.3 |
| DC_41-42_n79 | 41 | 0. |
|  | 42 | 0.8 |
| DC_66_n7-n78 | 66 | 0.6 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |
| DC_66_(n)12 | 12 | 0.8 |
|  | n12 | 0.8 |
|  | 66 | 0.5 |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_66_n25-n71 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n71 | 0.3 |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.6 |
| DC_66_n66-n78 | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| DC_66_(n)71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 | 0.3 |
| DC_66-71_n38 | 66 | 0.5 |
|  | 71 | 0.5 |
|  | n38 | 0.8 |
| DC_66-71_n66 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n66 | 0.3 |
| DC_66-71_n78 | 66 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |
| DC_66_SUL_n78-n86 | 66 | 0.6 |
|  | n78 | 0.8 |
|  | n86 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

------------------------------ Modified section -----------------------------

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 ¦fn78-fB1¦ |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 ¦2*fn78-3*fB3¦ |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 ¦fB1+fn3¦ |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | IMD2 ¦fn78-fB1¦ |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 ¦fn78-2*fB5¦ |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 ¦fn78-3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 ¦fn78 -3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 ¦fB7 -3*fB1¦ |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | IMD4 |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 ¦2*fn8+fB1¦ |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | IMD3 |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | IMD3 |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-11A_n78A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n78A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-18A_n77A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A_n40A-n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 ¦3*fB1-fn40¦ |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 ¦3*fB1 -fn78¦ |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 ¦3*fB1-2*fn3¦ |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 ¦fB1+fBn28¦ |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 |  | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 |  |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | IMD3 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 |  |  |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 |  |  |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 |  |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A-SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A-SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_2A-4A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 ¦2*fBn41-2*fB4¦ |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | N/A |
| DC_2A_n7A-n78A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 ¦2*fB2 -3*fn7¦ |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 ¦2*fn66-2*fB13¦ |
|  | 13 | 780 | 10 | 50 | 749 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 | 2 | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 ¦fB2+Bn66¦ |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 ¦fBn48-fB66¦ |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n25A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 ¦2*fBn41-2*fB66¦ |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 ¦3*fB2-fBn48¦ |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD5 ¦2*fBn48-3*fB66¦ |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 66/n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 ¦3*fB2-fBn78¦ |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 ¦fBn78-fB66¦ |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 ¦3*fB66-fBn78¦ |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 ¦2*fBn78-3*fB66¦ |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
| DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 ¦fB2+fn66¦ |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 ¦fB2 -3*fn66¦ |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 ¦fB3+fn1¦ |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 ¦fn77-fB3¦ |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 ¦fB3+fn1¦ |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | IMD5 ¦2*fn78-3*fB3¦ |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD33 |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 ¦fB77-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 ¦2*fB77-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 ¦fB77-4*fB3¦ |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 ¦2*fB77-3*fB3¦ |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 ¦fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 ¦2*fn78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 ¦2*fn7-fB3¦ |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD2 ¦fn8-fB3¦ |
| DC_3A-20A_n8A | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 ¦3*fn8-fB20¦ |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | IMD4 |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1744 | 5 | 25 | 1839 | 26.0 | IMD 2 |
|  | n41 | 2680 | 10 | 52 | 2680 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | n41 | 2590 | 10 | 52 | 2590 | N/A | N/A |
|  | 20 | 852 | 10 | 50 | 811 | 26.0 | IMD 2 |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2660 | 10 | 52 | 2660 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.5 | IMD 3 |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 ¦fB3+2*fn20¦ |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 ¦fn78-2*fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 28 | 735 | 5 | 25 | 790 | 26.0 | IMD21 |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 ¦3*fB3 -2*fn28¦ |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n77 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 4.8 | IMD5 ¦2*fB3-3*fn40 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 ¦3*fB3 -2*fn78¦ |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 4.9 | IMD4 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1475 | 0 | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 ¦fB41-fn28¦ |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 ¦3*fB3 -2*fn77¦ |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 ¦2*fB41 –fn77¦ |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 ¦2*fB41 – fn78¦ |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |
|  | 41 | 2670 | 5 | 25 | 2670 | 30.2 | IMD2 ¦fB3 -fn79¦ |
|  | 41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 29.4 | IMD2 ¦fB41 -fn79¦ |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 6.5 | IMD5 ¦4*fB5-fBn71¦ |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 ¦fn78-fb5¦ |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 ¦fn78-fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 ¦2*fn78-3fB7¦ |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A_n7A-n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 ¦fn78-fb5¦ |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 ¦fB7 +fn5¦ |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 ¦2*fB41-fn79¦ |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 ¦fn79-3*fB5¦ |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_5A-66A_n2A DC_5BA-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | 66 | 1712 | 5 | 25 | 2132 | 7.2 | IMD4 ¦2*fB5-2*fBn2¦ |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | N/A |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | IMD3 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD33 |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 ¦fB77-fB7¦ |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 ¦2*fB77-3fB7¦ |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 ¦fB77-fB8¦ |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 ¦fB78-fB7¦ |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 ¦2*fB78-3fB7¦ |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 ¦fB78-fB8¦ |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7-13_n66 | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 31 | IMD2 ¦fB7-fB66¦ |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_7-13_n66 | 7 | 2540 | 5 | 25 | 2660 | 18 | IMD3 ¦2*fBn66-fB13¦ |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 ¦2*fB7-3*fBn1¦ |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 896 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 ¦2*fn8-fB7¦ |
| DC_7A-20A_n8A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 ¦2*fn8+fB20¦ |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 ¦1*fn8+2*fB20¦ |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 ¦fn78-fB7¦ |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 ¦2*fn78-3*fB7¦ |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 ¦fn78-fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n40A | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 ¦fB7 +fn28¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 ¦fn78 -fB7¦ |
| DC_7A-40A_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 32.1 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 ¦2*fn78-2*fB7¦ |
|  | n78 | 3625 | 10 | 50 | 3475 | N/A | N/A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 ¦2*fB7-fn66¦ |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  | N/A | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
| DC_8A_n1A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n78 | 3745 | 10 | 50 | 3745 | 14.9 | IMD3 |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 30.4 | IMD2 |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.1 | IMD4 |
|  | 8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 42 | 3443 | 5 | 25 | 3443 | 8.7 | IMD4 |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD4 |
|  | n80 | 1750 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 8 | IMD33 |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |  |
|  | n77 | 3706 | 10 | 50 | 3706 | N/A | N/A |  |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 ¦fBn77-2*fB11¦ |  |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n78 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 ¦fBn77-2*fB11¦ |
| DC_12A_n7A-n78A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3624 | 10 | 50 | 3624 | 9 | IMD4 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | n7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 12.0 | IMD4 |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1736 | 5 | 25 | 2156 | 7..2 | IMD3 ¦2*fB13-2*fBn2¦ |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 ¦2*fB13-fBn48¦ |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 15.2 | IMD33 |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 ¦2*fB18-fBn3¦ |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD21 ¦fB41-fBn3¦ |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 ¦2*fn77-3*fB41¦ |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 ¦2*fn78-3*fB41¦ |  |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 ¦fn7 + fB28¦ |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 ¦fn78 - fB28¦ |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | 8.6 | IMD4 |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 | 2650 | N/A | N/A |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 15.9 | IMD3 |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3458 | 10 | 50 | 3458 | 9.1 | IMD4 ¦3*fn8 +fB28¦ |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | IMD5 ¦fn78 -4*fB28¦ |
|  | n78 | 3787 | 10 | 50 | 3787 | N/A | N/A |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 2.5 dB | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_66A_n7A-n78A | 66 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | 5 | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | 10 | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 2.8 | IMD5 ¦ 2*fn78-3*fB66¦ |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. |

------------------------------ Modified section ------------------------------

7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1_n3-n28 | n28 | 0.2 |
| DC_1-3_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n8 | n8 | 0.2 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n40 | 7 | 0.3 |
|  | n40 | 0.8 |
| DC_1-7_n78 DC_1-7-7_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n7-n78 | 1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n28 | 8 | 0.2 |
|  | n28 | 0.2 |
| DC_1-8_n77 |  |  |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n8-n78 | 1 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |
| DC_1-11_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1-11_n78 | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-18_n78 | n78 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28-n3 | 28 | 0.2 |
| DC_1-28_n7 | 28 | 0.2 |
| DC_1-28_n40 | 28 | 0.2 |
| DC_1-28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-32_n78 | n78 | 0.5 |
| DC_1_n40-n78 | n78 | 0.5 |
| DC_1-41_n3 | 41 | 01/0.52 |
| DC_1-41_n28 | n28 | 0.2 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n28 | 42 | 0.5 |
|  | n28 | 0.5 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n77-n79 | DC_1_n77-n79 | DC_1_n77-n79 |
|  | DC_1_n77-n79 | DC_1_n77-n79 |
| DC_1_SUL_n77-n80 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_SUL_n77-n84 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78-n79 | n78 | 0.5 |
| DC_1_SUL_n78-n80 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-SUL_n78-n84 | n78 | 0.5 |
| DC_2-4_n38 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_2-4_n41 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | n41 | 0.5 |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-7_n38 DC_2-2-7_n38 | n38 | 0.2 |
| DC_2-7_n66 DC_2-7-7_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7_n71 | n71 | 0.2 |
| DC_2_n7-n78 | 2 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |
| DC_2-12_n66, DC_2-2-12_n66 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | n66 | 0.3 |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-29_n66 DC_2-2-29_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
| DC_2-30_n66, DC_2-2-30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2_n41-n66 | 2 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n41-n71 | 2 | 0 |
|  | n41 | 0 |
|  | n71 | 0 |
| DC_2-48_n12 | 2 | 0.2 |
|  | 48 | 0.5 |
| DC_2-48_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n66 | 0.3 |
| DC_2-48_n71 | 2 | 0.2 |
|  | 48 | 0.5 |
| DC_2-66_n5 DC_2-2-66_n5 DC_2-66-66_n5 DC_2-2-66-66_n5 DC_2-66-66-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-66_n12 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n12 | 0.5 |
| DC_2-66_n25 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_2-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |
| DC_2-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-66_n71 DC_2_n66-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |
| DC_2-71_n66 DC_2-2-71_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-71_n78 DC_2-2-71_n78 | 2 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n1-n7 | 3 | 0 |
|  | n1 | 0 |
|  | n7 | 0 |
| DC_3_n1-n28 | 3 | 0 |
|  | n1 | 0 |
|  | n28 | 0.2 |
| DC_3_n1-n77 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n3-n77 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n3-n78 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n40 | 7 | 0.3 |
|  | n40 | 0.8 |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |
| DC_3-7_n8 | n8 | 0.2 |
| DC_3-7_n78 DC_3-7-7_n78 DC_3-3-7_n78 DC_3-3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n7-n78 | 3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_3-8_n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_3-8_n78 DC_3-3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-18-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n77 | 0.5 |
| DC_3-18-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n20-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-28_n5 | 28 | 0.1 |
|  | n5 | 0.1 |
| DC_3-28_n41 | n41 | 01/0.52 |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-32_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-38_n78 | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_3_n40-n41 | 3 | 0 |
|  | n40 | 0 |
|  | n41 | 04 |
|  |  | 0.53 |
| DC_3-41_n28 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
| DC_3-41_n41 | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| DC_3-(n)41 | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| DC_3-41-n77 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| DC_3-41_n78 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| DC_3-41-n79 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
| DC_3_SUL_n41-n80 | n41 | 0.53 |
| DC_3-42_n28 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_SUL_n77-n80 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_SUL_n77-n84 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-SUL_n78-n82 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_SUL_n78-n84 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n71 | n71 | 0.2 |
| DC_5-7_n78, DC_5-7-7_n78 , DC_5_n7-n78 | 5 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
| DC_5_(n)12 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |
| DC_5_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_5-66_n78 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n3-n78 | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n7-n78 | 7 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |
| DC_7-8_n1 DC_7-7-8_n1 | 8 | 0.2 |
| DC_7-8_n3 | 8 | 0.2 |
| DC_7-8_n77 |  |  |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_7-8_n78 DC_7-7-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_7-13_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7-28_n40 | n40 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_7-40_n1 | 7 | 0.3 |
|  | 40 | 0.8 |
| DC_7-46_n78 | n78 | 0.5 |
| DC_7-66_n38 | n38 | 0.2 |
| DC_7-66_n66 DC_7-7-66_n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n66-n78 DC_7-7_n66-n78 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.1 |
| DC_7_SUL_n78-n80 | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_8_n1-n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_8_n3-n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_8-11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |
| DC_8-11_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8-11_n78 | 8 | 0.2 |
|  | n78 | 0.2 |
| DC_8-20_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_8-42_n28 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
| DC_8-42_n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_8_SUL_n78-n80 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_8A-SUL_n78-n81 | 8 | 0.2 |
|  | n78 | 0.2 |
| DC_11-18_n77 | n77 | 0.5 |
| DC_11-18_n78 | n78 | 0.5 |
| DC_12_(n)5 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12_n7-n78 | 12 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |
| DC_12-30_n2 | 30 | 0.5 |
|  | n2 | 0.4 |
| DC_12-30_n66 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_12-66_n2 | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_12-66_n25 | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_13-48_n2 | 48 | 0.5 |
|  | n2 | 0.2 |
| DC_13-48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |
| DC_13-66_n2 DC_13-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_13-66_n48 DC_13-66-66_n48 | 66 | 0.2 |
|  | n48 | 0.5 |
| DC_14-66_n2 DC_14-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_18_n3-n78 | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_18-28_n77 | n77 | 0.5 |
| DC_18-28_n78 | n78 | 0.5 |
| DC_18-41_n3 | 41 | 01/0.52 |
| DC_18-41_n77 | n77 | 0.5 |
| DC_18-41_n78 | n78 | 0.5 |
| DC_18-42_n77 |  |  |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_18-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_18-42_n79 | 42 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | n77 | 0.5 |
| DC_19_n78-n79 | n78 | 0.5 |
| DC_20_n1-n28 |  |  |
|  | n1 | 0.2 |
|  | n28 | 0.2 |
| DC_20_n1-n78 | n78 | 0.5 |
| DC_20_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n28-n75 | n28 | 0.2 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20-32_n78 | n78 | 0.5 |
| DC_20-38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_20-(n)41 | 20 | 0.3 |
|  | 41 | 0.3 |
|  | n41 | 0.3 |
| DC_20_n75-n78 | n78 | 0.5 |
| DC_20_n76-n78 | n78 | 0.5 |
| DC_20_SUL_n78-n80 | n78 | 0.5 |
| DC_20-SUL_n78-n82 | n78 | 0.5 |
| DC_20-SUL_n78-n83 | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | n77 | 0.5 |
| DC_21_n78-n79 | n78 | 0.5 |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | 41 | 01 |
|  |  | 0.52 |
|  | n41 | 01 |
|  |  | 0.52 |
| DC_28-SUL_n78-n83 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_28_n3-n78 | 28 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_28_n7-n78 | n78 | 0.5 |
| DC_28-41_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28-41_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_28-41_n79 | n79 | 0.5 |
| DC_28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_30-66_n2 | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_30-66_n5 DC_30-66-66_n5 DC_30-66-66-66_n5 | 66 | 0.4 |
|  | n5 | 0.5 |
| DC_41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_41-42_n79 | 42 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_46-66_n41 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_48-66_n5 | 48 | 0.5 |
|  | 66 | 0.2 |
| DC_48-66_n12 | 48 | 0.5 |
|  | 66 | 0.2 |
| DC_48-66_n71 | 48 | 0.5 |
|  | 66 | 0.2 |
| DC_66_n7-n78 | 66 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_66_n25-n71 | 66 | 0.3 |
|  | n25 | 0.5 |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| DC_66_n66-n78 | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |
| DC_66-71_n38 | 66 | 0.5 |
|  | 71 | 0.5 |
|  | n38 | 0.5 |
| DC_66-71_n78 | 66 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |
| DC_66-SUL_n78-n86 | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz |  |  |

------------------------------ End of modified section ------------------------------