3GPP TSG-RAN WG4 Meeting #95-e	R4-2006607

Online, 25 May – 5 June, 2020

Source:	Huawei, HiSilicon, Bell Mobility, Telus

Title:	TP to TR 38.716-03-02 for CA_n7-n25-n66

Agenda item:	8.11.2

Document for:	Approval

# Background

This contribution provides text proposal on the NR CA band combination CA_n7-n25-n66 as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL [1].

# Text Proposal

##### ---Start of changes---

### 5.1.x	CA_n7-n25-n66

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: CA band combination of band n7+n25+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported bandwidths per CA band combination of band n7+n25+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n25A-n66A | CA_n7A-n25A, CA_n7A-n66A, CA_n25A-n66A | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

Based on co-existence studies of Band n7 + Band n25, Band n7 + Band n66 and Band n25 + Band n66 captured in TR 38.716-02-00, there is no own Rx impact on the 3rd band.

#### 5.1.x.4	∆TIB and ∆RIB values

For CA_n7-n25-n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66 | n7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66 | n7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

#### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

##### ---End of changes---

# Reference

[1]	R4-2004181, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 3 bands DL with 2 bands UL”, ZTE Corporation