3GPP TSG-RAN WG4 Meeting #95-e	R4-2006608

Online, 25 May – 5 June, 2020

Source:	Huawei, HiSilicon, Bell Mobility, Telus

Title:	TP to TR 38.716-03-02 for CA_n7-n66-n78

Agenda item:	8.11.2

Document for:	Approval

# Background

This contribution provides text proposal on the NR CA band combination CA_n7-n66-n78 as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL [1].

# Text Proposal

##### ---Start of changes---

### 5.1.x	CA_n7-n66-n78

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: CA band combination of band n7+n66+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported bandwidths per CA band combination of band n7+n66+n78

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n66A-n78A |  | CA_n7A-n66A, CA_n7A-n78A, CA_n66A-n78A | n7 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | n66 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n7A-n66A-n78(2A) |  | CA_n7A-n66A, CA_n7A-n78A, CA_n66A-n78A | n7 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | n66 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n78 |  | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

The coexistence study for DC_66_n7-n78 has been presented in TR 37.716-21-21 and the coexistence study for DC_7-66_n78 has been included in TR 37.716-21-11. It can be seen that:

- the IMD3 product of UL CA_n7A-n66A may fall into the Rx frequency of n78,

- the IMD4 product of UL CA_n7A-n78A may fall into the Rx frequency of n66.

#### 5.1.x.4	∆TIB and ∆RIB values

For CA_n7-n66-n78 , the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66-n78 | n7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66-n78 | n7 | 0.5 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.x.5	REFSENS requirements

The n78 DL is affected by the IMD3 of UL CA_n7A-n66A and the MSD value for DC_66_n7-n78 in TR 37.716-21-21 is reused. Moreover, the n66 DL is affected by the IMD4 of UL CA_n7A-n78A and the MSD value for DC_7-66_n78 in TR 37.716-21-11 is reused.

Table 5.1.x.5-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n7A-n66A-n78A, CA_n7A-n66A-n78(2A) | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 ¦ 2*fn7- fn66¦ |
| CA_n7A-n66A-n78A, CA_n7A-n66A-n78(2A) | n7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 ¦2*fn7 -2*fn78¦ |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |

##### ---End of changes---

# Reference

[1]	R4-2004181, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 3 bands DL with 2 bands UL”, ZTE Corporation