TSG-RAN Working Group 4 (Radio) meeting #95-E	R4-2008917

Electronic Meeting, May 25th – June 5th 2020

Source:	Ericsson

Title:	TP to TR 38.820: Addition of antenna parameter selection guideline in subclause 7.2

Agenda item:	9.2.6.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#94bis-E) array antenna parameters relevant for the frequency range close to 10 GHz was discussed. It was noticed that TR 38.820 describes some typical array antenna topologies to consider for 7 to 24 GHz. However, the current technical information in TR 38.820 does not describe fundamental relations between physical antenna characteristics such as element beamwidth and element separation. This information is essential when parameters is defined for a specific antenna topology and architecture and later mapped towards the array antenna model used by RAN4 and other bodies.

In this contribution we summarize the array antenna model used in RAN4 and add some example parameters to show some basic relations between parameters with the intension to simplify and stimulate work in coming WIs to define RF core requirements.

In this contribution a text proposal for TR 38.820, subclause 7.2 [1] with additional technical background for how to determine antenna parameters for different array geometries is prepared. The text proposal is attached at the end of the contribution and is presented for approval.

# Discussion

This is a revised version of R4-2006925. The changes are summarized as;

- The definition of number of elements is clarified, instead of N, now referring to MxNx2.

- Addition of relevant antenna model symbols in symbol section, subclause 3.2.

- Clarification on parameter selection when sub-arrays is modelled.

- Alignment with deployment scenarios used in section 5.

- In parameter selection procedure text not relevant for the antenna model is removed.

- Correction of “beam width product” spelling.

- The technical background for the model was changes according to feedback.

- Parameters proposed for ITU-R 5D is added as examples.

- Step 4 and 5 is updated to resolve circular procedure; First select beamwidths based on spacing, then calculate true gain.

- Table 7.2.4-2 is updated to now include both peak normalized element pattern and peak gain normalized element pattern.

- Eq. 7.2.4-3 and 7.2.4-4 is updated to consider peak normalized pattern given in Table 7.2.4-2.

- In step 1, the considered coexistence situation is added.

- Antenna parameter examples values are removed.

- A note is introduced in procedure on how to handle sub-arrays.

- Equation 7.2.4-2 is updated to support the case with symmetrical elements and the case where narrower sub-arrays is used.

- The reference to IEEE is changes to give more input for the background to 32400 and 52525.

- Added symbols for M and N in subclause 3.2.

- Definition of M and N is aligned, errors are fixed.

- Added word “radiating” in note.

- Error in element pattern is corrected

# Conclusion

In this contribution we have summarized the array antenna model and described how antenna model parameters can be determined. Also, some example parameters sets have been created to map towards different antenna topologies.

A text proposal has been created to TR 38.820 [1] with some additional technical background information about the antenna topologies relation to the array antenna model parameters. At the end on this contribution a text proposal for TR 38.820, subclause 7.2 is attached for approval.

# References

[1]	R4-2005740, “draft TR 38.820 v130”, Huawei

[2]	M.2101, “RecommendationITU-RM.2101-0; Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies”, ITU-R

[3]		TR 37.840, “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station”, 3GPP

[4]		W. L. Stutzman, "Estimating directivity and gain of antennas," in IEEE Antennas and Propagation Magazine, vol. 40, no. 4, pp. 7-11, Aug. 1998, doi: 10.1109/74.730532, URL: http://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=730532&isnumber=15753

TEXT PROPOSAL for clause 2

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

TEXT PROPOSAL for clause 3.2

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

N	Number of columns of the antenna array

AA	Composite antenna array pattern

AE	Array element pattern

Am	Front-to-back ratio

BWChannel	BS channel bandwidth

BWSSB				Bandwidth of the SSB

COff 	Isolation in OFF state

DE,max 	Array element peak directivity

dh	Horizontal elements separation in antenna array

dv	Vertical elements separation in antenna array

Fmax		Measure of the achievable power gain

Ft	Cut-off frequency

GE,max	Array element peak gain

IMF 	Industrial Margin

LE	Array element loss

M	Number of rows of the antenna array

Psat	Saturated output power

RON 	Losses in ON state

SLAv	Side-lobe suppression

xFR	Integer representing the BS frequency range

	Horizontal angle (defined between -180° and 180°).

3dB	Horizontal half power beam width in degrees

	Vertical angle of the signal direction (defined between -0° and 180°; 90° represents the direction perpendicular to the antenna array)

3dB	Vertical half power beam width in degrees

etilt	Electrical down-tilt angle

escan	Electrical scan angle

TEXT PROPOSAL for subclause 7.2:

### 7.2.3	Antenna topologies

The antenna array (AA) consists of M x N x 2 antenna elements (AE) placed is a certain lattice, arranged in M rows and N columns of cross-polarized elements. The signals from the AA are mapped in the RDN creating different antenna topologies, as presented on examples in figure 7.2.3-1. The RDN mapping is creating sub-arrays, where the radiating characteristics of a sub-array is different to single antenna elements.

Figure 7.2.3-1: Example RDN mappings

Depending on intended coverage scenarios different types of RDN mappings are foreseen for the frequency range 7 to 24 GHz. For BS type xFR-H and BS type xFR-O, the OTA RF characteristics defined for requirement set category H and requirement set category O is declared by the base station manufacturer in terms of full array capability as well as sub-array or element capability, see TS 38.141-2 [6], clause 4.6.

### 7.2.4	Array antenna model

In Table 7.2.4-1, the parameters used by the parameterized array antenna model are described. Based on AAS base station architecture in clause 7.2.1 (i.e. the BS type xFR-H and BS type xFR-O) and deployment scenarios envisioned for the 7 – 24 GHz range, different parameter sets are required to model an AAS base station.

Table 7.2.4-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Array element loss | LE | dB |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 7.2.4-2.

Table 7.2.4-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Element peak gain | GE,max=DE,max-LE , where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πn-1dvλcosθ+m-1dhλsinθsinφ wm,n=1MNexpj2πn-1dvλsinθetilt-m-1dhλcosθetiltsinφescan | dBi |

To conserve complexity the model is created so that the element is gain normalized, instead of the composite array pattern. As a consequence, parameters cannot be selected arbitrarily, since parameters are dependent on each other. The intension with the model is to model absolute gain patterns correctly without full pattern directivity normalization. To model absolute gain, parameters must be selected carefully, if not the model produces nonphysical and incorrect gain response.

When array antenna parameters are selected for the array antenna model it is preferable to consider physical aspects such as the gain/area relation and gain/beamwidth relations by checking following aspects in given order:

- The considered deployment scenario and coexistence situation will give the appropriate coverage range requirement for the horizontal and vertical domain.

NOTE 1: 	For analysis of the NR deployment scenarios considered for the 7 – 24 GHz range, refer to clause 5.6.

- From the coverage ranges and deployment scenario the required antenna gain can be determined, from which the array antenna geometry can be determined in terms of number of rows (M), the number of columns (N).

- From the coverage ranges the array antenna steering capability can be determined in terms element separations (dv, dh).

NOTE 2:	 The element separations dv and dh is the distance between radiating elements in the array antenna. The RDN can be used to create sub-arrays to optimize coverage. When sub-arrays are modelled, parameters can be selected to model the sub-array as a radiating element, in this case dv and dh is the distance between sub-arrays in the antenna array.

- From the given array lattice the element parameters can be considered with respect to the given area for a single element. The element peak gain (GE,max) and half power beamwidth product (3dB and 3dB) depend on each other and must be selected together to maintain accurate model gain response. The element loss (LE) needs to be included when the element peak gain is determined. Select parameter values for beamwidth based on the following two parameters checks:

  - Check the peak element directivity (DE,max) with the unit area available for a single element in the array lattice, as described in Eq. 7.2.4-1.

  - Check the peak element directivity (DE,max) with the half-power beam width product (3dB and 3dB), as described in Eq. 7.2.4-2.

- The model gain is guaranteed by a element peak directivity normalization directivity (DE,max) described in Eq. 7.2.4-3. The peak element gain GE,max is calculated based on Eq. 7.2.4-4.

The peak element directivity (assuming no losses for a given antenna aperture area) can be expressed as:

DE,max≤10log104πdhdvλ2		(Eq. 7.2.4-1)

Also, the peak element directivity for a given wide symmetrical beam can be approximated by:

DE,max≈10log10Kφ3dBθ3dB		(Eq. 7.2.4-2)

, where K is a factor that depends on the element properties. For single elements with symmetrical large beamwidths, K = 52525 is appropriate, while for sub-arrays with narrower beamwidth characteristics, K = 32400 is more appropriate. Depending on the element characteristics the relation between element peak gain and the half power beam width product is different as described in [65].

To be exact it is recommended to select element parameters, where the peak element gain is determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 7.2.4-1 in dBi as:

DE,max=10log104πAθ,φmax-ππ0πAθ,φsinθdθdφ		(Eq. 7.2.4-3)

, where A(,) is defined in linear scale as:

Aθ,φ=10Aθ,φ10		(Eq. 7.2.4-4)