3GPP TSG-RAN WG4 Meeting #95-e                                    R4-2008618

Electronic Meeting, 25th May – 5th June, 2020

Agenda item:				6.11.2

Source:				Samsung

Title:			WF on NR eMIMO RRM Requirement

Document for:			Approval

# 1	Background

Rel-16 NR eMIMO WI (i.e., Enhancements on MIMO for NR) is a RAN1 leading WI with below major enhancement in RAN1 area which has potential RRM requirement impact in RAN4:

- Enhancements on multi-beam operation

  - DL/UL beam indication with reduced latency and overhead

  - Beam failure recovery for SCell

  - L1-SINR measurement

This WF is used to capture further agreements from RAN#95.

# 2	Agreement from RAN4#95 Chairman Notes after 1st Round

Topic #1: L1-SINR Measurement Issue 1-1-1: For SSB/CSI-RS-based CMR+IMR, the sharing factor P - Agreement: - For SSB/CSI-RS-based CMR+IMR L1-SINR measurement: - No requirement when CMR or IMR is fully overlapped with MG. - The variable P used for defining L1-SINR measurement period could can be defined as the maximum value between PCMR and PIMR, where - PCMR is the scaling factor for CMR according to the principles of defining variable P for L1-RSRP measurement. - PIMR is the scaling factor for IMR according to the principles of defining variable P for L1-RSRP measurement. Issue 1-1-2: Extend single carrier requirement to CA - Agreement: - For extending single carrier requirement to CA for L1-SINR measurement: - Follow the conclusion from extending single carrier requirement to CA for L1-RSRP measurement in Rel-15 TEI. Issue 1-2-2: For SSB-based CMR+IMR, “repetition = ON” field of IMR - Agreement: For SSB-based CMR+IMR with NZP IMR configured with “repetition = ON”: - No measurement period requirement shall be applied. Topic #2: SCell Beam Failure Recovery Issue 2-1-1: BFD/CBD Sharing factor for FR1 inter-band CA - Agreement: - BFD/CBD Sharing factor for FR1 inter-band CA: - The sharing factor is proportional to the number of bands on which UE is performing BFD/CBD only for SCell. Issue 2-1-3: Sharing factor for BFD/CBD measurement on PCell/PSCell - Agreement - Sharing factor for BFD/CBD measurement on PCell/PSCell: - No scaling factor is introduced for BFD/CBD measurements on PCell/PSCell. Issue 2-2-1: How the requirement for beam failure recovery request should be defined - Agreement - The requirement for beam failure recovery request is defined as: - After detecting beam failure in an SCell, UE is required to transmit scheduling request on PUCCH configured for SR for BFR within a period T, where - T = T1 x Ceil((T2 + D) / T1), - T1 is equal to the periodicity of PUCCH configured with schedulingRequestForBFR. - T2 is the time to perform the candidate beam detection T2 = TEvaluate_CBD. - TEvaluate_CBD is the evaluation period for candidate beam detection specified in TS38.133 8.5.5 and 8.5.6. - D is the UE Processing time. Topic #3: DL/UL Beam Indication with Reduced Latency and Overhead Issue 3-1-1: The necessity of new RRM requirement for MAC-CE based spatial relation update for aperiodic-SRS - Agreement - The necessity of new RRM requirement for MAC-CE based spatial relation update for aperiodic-SRS: - No new RRM requirement is introduced for the feature of MAC-CE based spatial relation update for aperiodic SRS. Topic #4: Multi-TRxP Transmission Issue 4-2-1: For FR1 Intra-band CA, whether or not the same conclusion as intra-band EN-DC can be applied - Agreement - For FR1 Intra-band CA, RRM MRTD requirement impact due to enabling multi-TRxP transmission in Rel-16: - RAN4 apply the same conclusion as intra-band EN-DC.

# 3	Way Forward

3.1  L1-SINR Measurement

##### 3.1.1 “Repetition” field of NZP-IMR Not Present:

For L1-SINR measurement with dedicated configured NZP-IMR, the expected UE behavior if NZP-IMR is not configured with “repetition” field:

  - In RAN4 requirement, the FR2 scaling factor N is defined only based on CMR configuration.

  - In RAN4 requirement, there is no need to specify the expected UE behavior/requirement if NZP-IMR is not configured with “repetition” field, because the expected UE behavior is already specified in RAN1 and RAN2 specification.

##### 3.1.2 Scheduling Restriction

- Scheduling restriction due to L1-SINR measurement

  - The scheduling restriction due to L1-SINR shall be based on the framework of scheduling restriction due to Rel-15 L1-RSRP measurement.

3.2  SCell Beam failure recovery

##### 3.2.1 BFD and CBD on SCell

- BFD/CBD Sharing factor for FR2 inter-band CA and FR1-FR2 CA

  - Define sharing factor as:

    - In FR2 inter-band CA, the sharing factor is proportional to the number of bands on which UE is performing BFD/CBD only for SCell.

      - UE is required to perform BFD/CBD in only one band among a set of bands that it can receive with the common beam.”

    - In FR1+FR2 CA, sharing factor shall be introduced, and the sharing factors is the sum of the sharing factor of FR1 and the sharing factor of FR2.

##### 3.2.2 SCell Beam Failure Recovery Request

UE Processing time, D:

  - D = 2ms

    - In RAN4#96, companies may further discuss the conclusion of D = 2ms, if problem identified.

3.3 DL/UL Beam Indication with Reduced Latency and Overhead

##### 3.3.1 RRM requirement impact to enable multi-TRP transmission

- The necessity of new RRM requirement for MAC-CE based Pathloss RS activation

  - Option 1: The agreed UE behavior and requirements for MAC-CE based pathloss RS activation when an activated pathloss RS is not being maintained shall be specified in TS 38.133.

  - Option 2: Follow existing agreement from RAN4 chairman notes, as below:

- < RAN4#92bis Chairman Notes> - No RAN4 impact has been identified due to newly introduced - Mechanism of updating pathloss RS for PUSCH/SRS via MAC-CE. - Mechanism of simultaneous spatial relation update for multiple PUCCH resources with one MAC-CE. - Default spatial Relation for PUCCH/SRS in FR2.

- How to capture new RRM requirement for MAC-CE based Pathloss RS activation (if any)

  - If the necessity of new requirement is confirmed (in the discussion on the above listed issue), there are options proposed for how to capture new RRM requirement:

    - Option 1: in Rel-16 TEI.

    - Option 2: in Rel-17 scope.

    - Option 3: in Rel-16 eMIMO WI.

    - Option 4: No new requirement needed at all.

3.4 Multi-TRxP Transmission

##### 3.3.1 RRM requirement impact to enable multi-TRxP transmission

- “Co-located deployment assumption” and its impact on multi-TRxP transmission

  - FFS whether or not “co-located deployment” in intra-band EN-DC and intra-band FR1 CA MRTD requirement needs to be revised to enable multi-TRxP transmission on NR cells.

  - FFS the reference timing of each multi-TRxP enabled carrier to find MRTD/MTTD in EN-DC scenarios.

# 4	Reference

| R4-2006205 | Discussion on requirements for L1-SINR measurements | Apple | discussion |
| --- | --- | --- | --- |
| R4-2006370 | Discussion on L1-SINR Measurement Requirement | Samsung | discussion |
| R4-2006371 | CR to TS38.133 on introduction of L1-SINR Measurement Requirement (Section 3.3 and 9) | Samsung | CR |
| R4-2006864 | Discussion on RRM requirements for L1-SINR | MediaTek inc. | discussion |
| R4-2007483 | RRM requirements for L1-SINR estimation | Qualcomm | discussion |
| R4-2007767 | Discussion on L1-SINR measurement requirements for NR eMIMO | Huawei, HiSilicon | discussion |
| R4-2007768 | Discussion on L1-SINR measurement accuracy for NR eMIMO | Huawei, HiSilicon | discussion |
| R4-2007769 | DraftCR on L1-SINR measurement accuracy requirements | Huawei, HiSilicon | draftCR |
| R4-2008091 | Discussions on Rel-16 NR eMIMO L1-SINR measurements | Nokia, Nokia Shanghai Bell | other |
| R4-2006372 | Discussion on SCell Beam Failure Recovery RRM Requirement | Samsung | discussion |
| R4-2006373 | CR to TS38.133 on introduction of SCell BFD and CBD (Section 8.5) | Samsung | CR |
| R4-2006374 | CR to TS38.133 on introduction of SCell BFRQ Procedure (Section 8.5) | Samsung | CR |
| R4-2006865 | Discussion on RRM requirements for BFR on SCell | MediaTek inc. | discussion |
| R4-2007378 | BFRQ on SR-like PUCCH resource | Ericsson | discussion |
| R4-2007379 | Draft CR: Correction of SCell BFRQ Procedure (Section 8.5) | Ericsson | draftCR |
| R4-2007484 | SCell Beam Failure Detection and Recovery | Qualcomm | discussion |
| R4-2007770 | Discussion on SCell BFD and CBD requiremetns for NR eMIMO | Huawei, HiSilicon | discussion |
| R4-2007771 | CR on SCell BFD and CBD requirements | Huawei, HiSilicon | CR |
| R4-2006375 | Discussion on MAC-CE based spatial relation update for aperiodic SRS | Samsung | discussion |
| R4-2006065 | Discussion on applicable timing for the unknown PL RS activated by MAC-CE | ZTE Corporation | discussion |
| R4-2006206 | Discussion on RRM requirements for Multi-TRP | Apple | discussion |
| R4-2006376 | Discussion on MRTD/MTTD requirement to Enable Multi-TRP Transmission | Samsung | discussion |
| R4-2006377 | CR to TS38.133 on introduction of multi-TRP transmission (Section 7.5 and 7.6) | Samsung | CR |
| R4-2006866 | Discussion on MRTD for multiple TRPs scenario | MediaTek inc. | discussion |
| R4-2006867 | Discussion on PL RS activation requirement via MAC CE | MediaTek inc. | discussion |
| R4-2006868 | CR for introduction of pathloss reference signal switching delay | MediaTek inc. | CR |
| R4-2007380 | MRTD/MTTD requirements for Multi-TRP deployment for MIMO+CA and MIMO+DC | Ericsson | discussion |
| R4-2007485 | MRTD/MTTD in CA/DC with multiple TRPs | Qualcomm | discussion |
| R4-2007772 | Discussion on MRTD and MTTD requirements for multi-TRP transmissions | Huawei, HiSilicon | discussion |
| R4-2008092 | Discussions on Rel-16 NR eMIMO multi-TRP transmissions | Nokia, Nokia Shanghai Bell | other |