TSG-RAN Working Group 4 (Radio) meeting #95-E	R4-2008786

Electronic Meeting, May25th – June  5 th 2020

Source:	Ericsson

Title:	WF on IAB-MT In-band selectivity and blocking

Agenda item:	6.5.2.2.2

Document for:	Approval

# Introduction

At RAN4#94-E meeting a way-forward for IAB-MT In-Band Blocking and blocking was created based on the discussion [1].

# Way-Forward

There is strong dependency on this WF and the WF on REFSENS, so right now, there are three options :

Option 1: If only OTA sensitivity is defined for 1-O, only  EISminSENS will be used for ACS and IBB

Option 2: If only OTA REFSENS is defined for 1-O, , Only EIS_REFSENS will be used for ACS and IBB

Option 3: If both OTA REFSENS and OTA sensitivity are defined for 1-O, both  EIS_minSENS and EIS_REFSENS will be used for IBB, but only both  EIS_minSENS is used for ACS, this is the same as BS ACS type 1-O

This WF takes consideration of the WF on REFSENS and lists the options that could be make later when REFSENS is agreed on IAB-MT type 1-O. This will apply both WA IAB-MT and LA IAB-MT.

The Below WF options for IAB-MT Type 1-O will be decided according to above options once WF of REFSENS is agreed.

- ACS and IBB of FR1 WA IAB-MT Type 1-H:

| WA IAB-MT | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- |
| ACS(45dB) | REFSENS+6dB | -52 | CP-OFDM, others the same with BS spec |
| In-band blocking | REFSENS+6dB | -43 | CP-OFDM, others the same with BS spec |

- ACS of FR1 WA IAB-MT Type 1-O:

| WA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- | --- |
| ACS(45dB) | Option 1 | EISminSENS +6dB | -52 – ΔminSENS | CP-OFDM, others the same with BS spec |
|  | Option 2 | EISREFSENS +6dB | -52 - ΔOTAREFSENS |  |
|  | Option 3 | EISminSENS +6dB | -52 – ΔminSENS |  |

- IBB of FR1 WA IAB-MT Type 1-O:

| WA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- | --- |
| In-band blocking | Option 1 | EISREFSENS +6dB | -43 - ΔOTAREFSENS | CP-OFDM, others the same with BS spec |
|  | Option 2 | EISminSENS + 6 dB | -43 – ΔminSENS |  |
|  | Option 3 | EISREFSENS +6dB | -43 - ΔOTAREFSENS |  |
|  |  | EISminSENS + 6 dB | -43 – ΔminSENS |  |

- FR1  LA IAB-MT Type 1-H  with below candidate options

| LA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] |
| --- | --- | --- | --- |
| Option 1: ACS 33dB | ACS | REFSENS+14dB | Case: [-45.5] Necessity on case 2 is FFS since the dynamic range for FR1 IAB-MT is open too. |
|  | In-band blocking | REFSENS+6dB | Case 1:-56 Case 2:-44 |
| Option 2: ACS 45dB | ACS(45dB) | REFSENS+6dB | -44 |
|  | In-band blocking | REFSENS+6dB | -35 |

Note: in option 1 for LA IAB-MT is based on the NF of 9/10dB, if the NF is agreed as 13dB the interference level should be adjusted accordingly.

- ACS of FR1 LA IAB-MT Type 1-O:

| WA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- | --- |
| ACS(45dB) | Option 1 | EISminSENS +6dB | -44 – ΔminSENS | CP-OFDM, others the same with BS spec |
|  | Option 2 | EISREFSENS +6dB | -44 - ΔOTAREFSENS |  |
|  | Option 3 | EISminSENS +6dB | -44 – ΔminSENS |  |

- IBB of FR1 LA IAB-MT Type 1-O:

| WA IAB-MT |  | Wanted signal level[dBm] | Interference signal level [dBm] | Interference signal bandwidth and offset |
| --- | --- | --- | --- | --- |
| In-band blocking | Option 1 | EISREFSENS +6dB | -35 - ΔOTAREFSENS | CP-OFDM, others the same with BS spec |
|  | Option 2 | EISminSENS + 6 dB | -35 – ΔminSENS |  |
|  | Option 3 | EISREFSENS +6dB | -35 - ΔOTAREFSENS |  |
|  |  | EISminSENS + 6 dB | -35 – ΔminSENS |  |

# References

[1]	R4-2008699,	Email discussion summary for [95e][310] NR_IAB_RF_Part_3, Samsung