3GPP TSG-RAN WG4 Meeting #95-e                 	R4-2006061

Electronic Meeting, 25 May – 5 June, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 non-relaxed high reliability BS demodulation requirement simulation results

Agenda item:	6.9.1.1.2

Document for:	Discussion

Introduction

During the last RAN4#94-bis-e meeting, good progress was made on the topic of NR URLLC performance requirements with Ultra-Low BLER [1]. With the agreements captured in the WF [2], it is now possible to start simulation campaigns.

In this contribution we provide the results of our NR URLLC performance requirements with Ultra-Low BLER simulation campaigns.
Part of the results are used in our discussion paper to justify the value of “X”.

Simulation results non-relaxed high reliability

Parameters

Unless stated explicitly otherwise in any of the result tables, the simulation setup follows the baseline as captured in WF [2] and the previous ones. 
We summarize the settings as follows:

Table 1: Simulation parameters.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled (CP-OFDM only) |
| System | Mod/Demod branches | 1T2R |
|  | fc | 4 GHz (FR1) |
|  | SCS/CBW | 15 kHz with 10 MHz 30 kHz with 40 MHz |
|  | Uplink-downlink allocation for TDD | 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz |
|  | Channel model | AWGN |
|  | Cyclic Prefix | Normal |
| High reliability features | pusch-AggregationFactor | n1 (no slot aggregation) |
|  | mcs-Table | qam64LowSE |
|  | MCS | 5 |
|  | maxRank | 1 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | OFF |
|  | Code block group based PUSCH retransmission | Disabled |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 (i.e., DM-RS = 1+1) |
|  | Number of DM-RS CDM group(s) without data | [2] |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | [-3 dB] |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | [NID=0, nSCID =0] |
| Time domain resource assignment | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Others | Channel coding: Limited buffer rate matching, PT-RS | Disabled |
| Performance metric |  | SNR@1e-5 BLER point; SNR@1e-6 BLER point [SNR vs BLER curve] (Block in BLER is “TB”) |

Results

The simulations are still ongoing at the time of writing this contribution, hence the figure and numbers below are a preliminary snapshot and might be updated during the meeting.

Figure 1: PUSCH high reliability testing simulation results.

Table 2: PUSCH high reliability testing simulation results.

| Results | SCS/CBW | Mapping Type | SNR @ target BLER [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1e-5 | 1e-6 |
| Set 1 | 15 kHz/10 MHz | TypeA | -7.3 |  |
|  |  | TypeB | -7.3 |  |
|  | 30 kHz/40 MHz | TypeA | [-7.6] |  |
|  |  | TypeB | -7.7 |  |

Conclusion

In this contribution we provide the results of our NR URLLC performance requirements with Ultra-Low BLER simulation campaigns.

- References

- R4-2005589, Email discussion summary for [94e Bis][218]R_L1enh_URLLC_Demod_Test, Moderator (Ericsson), RAN4#94-e-bis.

- R4-2005526, Way forward on NR URLLC performance requirements with Ultra-Low BLER, Moderator (Ericsson), RAN4#94-e-bis.