3GPP TSG-RAN4 Meeting #95e	R4-2006211

# Online, May 25th 2020 – June 5th 2020

Agenda item:	6.13.2.1

Source:	Apple Inc.

Title:	On DL interruption of UL Tx Switching

WI:	NR_RF_FR1

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

As part of FR1 enhancement WI, a new objective was included

Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

  - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

  - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

  - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

  - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3:  The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

In RAN4#94bis, the follow agreements have been reached regarding the DL interruption.

- The DL interruption granularity is in the unit of OFDM symbols.

- Two options for interruption length is TBD

- The DL interruption location/Starting point

  - Option 1: the DL interruption are the OFDM symbols fully or partial overlapped with the UL switching period.

  - Option 2: the starting time is determined by TA+TA uncertainty+MRTD

# 2       DL interruption and related requirements

From NW perspective, it is hard to determined when exactly in symbol level the interruption will start since it depends on both MRTD(33us) and TA adjustment accuracy which is unknown at NW. More specifically, the interruption can happen anywhere within a window of UL switching period+2*MRTD+2*TA adjustment accuracy. It is noted that the exact interruption length should be less than this window length. However, due to the uncertainty at NW side, it is hard for NW to judge which DL symbols during this window will be interrupted.

Thanks to flexible NR scheduling mechanism in symbol level, if NW can avoid scheduling data during interruption window (UL switching period+2*MRTD+2*TA adjustment accuracy), the rest of symbols in the same slot can still be used and decodable. However, if data is transmitted during this window and some of symbols cannot be received due to the interruption, the whole slot may not be decodable. This is exactly why all the interruption time defined in LTE and NR is in slot level.

The proposed interruption time is summarized in the following table.

| u | NR slot length (ms) of victim cell | UL switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
|  |  | Uncertainty UL switching window |  |  |
|  |  | 101us | 206us | 276us |
| 0 | 1 | 3 | 5 | 5 |
| 1 | 0.5 | 4 | 7 | 9 |
| 2 | 0.25 | 7 | 13 | 17 |

Based on the analysis above, the starting point of DL interruption should be determined by TA+TA uncertainty+MRTD.

# 5 Conclusion

In this contribution, DL interruption length due to UL Tx switching is proposed as

Proposal 1: Length of DL interruption due to UL Tx switching is proposed as

| u | NR slot length (ms) of victim cell | UL switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
|  |  | Uncertainty UL switching window |  |  |
|  |  | 101us | 206us | 276us |
| 0 | 1 | 3 | 5 | 5 |
| 1 | 0.5 | 4 | 7 | 9 |
| 2 | 0.25 | 7 | 13 | 17 |

Based on the analysis above, the starting point of DL interruption should be determined by TA+TA uncertainty+MRTD.