3GPP TSG-RAN WG4 Meeting #95-e                                                              R4-2006268

Electronic Meeting, 25 May – 5 June, 2020

Source: 	CATT

Title: 	Discussion on the introduction of DFT-s-OFDM for NR HST PUSCH

Agenda item:	6.17.2.2.1

Document for:	Discussion

- Introduction

In RAN4#94-e-bis meeting, whether to introduce the performance requirement for DFT-s-OFDM for NR HST PUSCH was discussed based on [1] with a lot of remaining issue undetermined yet. This contribution will provide our views and analysis on the introduction of DFT-s-OFDM for PUCSH.

- Discussion

As per the WFs on NR HST demodulation requirements, the open issues of  DFT-s-OFDM for PSUCH in RAN4#94-e-bis meeting are summarized as follows [1]:

- Dft-s-OFDM waveform - Option 1: Introduce PUSCH HST requirements for DFT-s-OFDM. - Option 2: Do not introduce PUSCH HST requirements for DFT-s-OFDM - Option 3: Define DFT-s-OFDM only for 350km/h scenario, 1T2R and minimum channel bandwidth Proposed WF: Discuss in next meeting. Clarify how/if implicit test passing is still applicable. - Organisation of high-speed train requirement sections for PUSCH DFT-s-OFDM in specifications - Option 1: In HST PUSCH section currently used for 350kph (e.g., 38.104 section 8.2.4) - New test parameter table and minimum requirements tables(s) for “transform precoding = on”. - Option 2: TBD after DFT-s-OFDM agreement. Proposed WF TBD after DFT-s-OFDM introduction agreement. - If Dft-s-OFDM is introduced: Applicability rule - Option 1: Similar applicability rule for waveforms as existing PUSCH performance requirements will be used for HST - Option 2: TBD after DFT-s-OFDM decision. Proposed WF TBD after DFT-s-OFDM introduction agreement. - If Dft-s-OFDM is introduced: Configuration - Option 1 Configuration as follows. 500kph only, MCS2, RB allocation: 24 RB for 5MHz CBW/15KHz SCS, TDRA: type A, DMRS position: 1+1+1, L0: 3. - Option 2: Minimum channel bandwidth (i.e., 5MHz for 15 kHz SCS, 10MHz for 30 kHz SCS) - Option 3: Define HST requirements with DFT-s-OFDM for both 350 km/h and 500 km/h. - Option 4: TBD after Dft-s-OFDM decision. Proposed WF: TBD after DFT-s-OFDM introduction agreement.

In general, the practical use case of DFT-s-OFDM is rather rare in HST scenarios. Considering the performance requirement of DFT-s-OFDM is defined in Rel-15 normal demodulation, it is sufficient to only specify HST PUSCH requirement for CP-OFDM. Looking at the simulation results, the performance gap between DFT-s-OFDM and CP-OFDM is small. So we don’t see the demand to introduce the performance requirement for DFT-s-OFDM for NR HST.

Proposal 1: Do not introduce HST PUSCH requirements for DFT-s-OFDM (Option 2).

- Conclusion

In this contribution, whether to introduce the performance requirement for DFT-s-OFDM for NR HST PUSCH is analysed. The following proposal is derived as follows:

Proposal 1: Do not introduce PUSCH HST requirements for DFT-s-OFDM (Option 2).

- References

- R4-2005534, WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#94-e-bis