3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006273

Electronic Meeting, 25 May – 5 June, 2020

Title: 	TP for TS 38.174: Transmit ON/OFF power

Source: 	CATT

Agenda item:	6.5.2.1.3

Document for:	Approval

- Introduction

This contribution provides TP for TS 38.174, the discussion background is in the contribution [1].

- Reference

[1] R4-2006272, “TP for TR 38.809: Transmit ON/OFF power”, CATT

[2] R4-2004801, TS 38.174 v0.0.1

Annex: TP for IAB TS

<First part>

6.4	Transmit ON/OFF power

6.4.1 Transmitter OFF power

The requirements in clause 6.4.1.1 and 6.4.1.3 in TS 38.104 [TBD] apply to IAB-DU type 1-H and IAB-MT type 1-H.

6.4.2 Transmitter transient period

6.4.2.1 General

Transmitter transient period requirements apply only to TDD operation of NR IAB-DU and IAB-MT.

The transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1 for IAB-DU and 6.4.2.1-2 for IAB-MT.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.4.2.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period for IAB-DU

Transmitter output power

Time

Transmitter ON period

(UL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

DL transmission

GP or DL transmission

Figure 6.4.2.1-2: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period for IAB-MT

For IAB-DU and IAB-MT type 1-H, this requirement shall be applied at each TAB connector supporting transmission in the operating band.

6.4.2.2	Minimum requirement for IAB-DU type 1-H and IAB-MT type 1-H

The requirements in clause 6.4.2.2 in TS 38.104 [TBD] apply to IAB-DU type 1-H and IAB-MT type 1-H.

<Next part>

9.5	OTA transmit ON/OFF power

9.5.1	General

OTA transmit ON/OFF power requirements apply only to TDD operation of IAB-DU and IAB-MT.

9.5.2	OTA transmitter OFF power

9.5.2.1	General

The general part in clause 9.5.2.1 for BS type 1-O in TS 38.104 [TBD] apply to IAB-DU type 1-O and IAB-MT type 1-O.

The general part in clause 9.5.2.1 for BS type 2-O in TS 38.104 [TBD] apply to IAB-DU type 2-O and IAB-MT type 2-O.

9.5.2.2	Minimum requirement for IAB-DU type 1-O and IAB-MT type 1-O

The requirements in clause 9.5.2.2 in TS 38.104 [TBD] apply to IAB-DU type 1-O and IAB-MT type 1-O.

9.5.2.3	Minimum requirement for IAB-DU type 2-O and IAB-MT type 2-O

The requirements in clause 9.5.2.3 in TS 38.104 [TBD] apply for IAB-DU type 2-O and IAB-MT type 2-O.

9.5.3	OTA transient period

9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1 for IAB-DU and figure 6.4.2.1-2 for IAB-MT..

This requirement shall be applied at each RIB supporting transmission in the operating band.

9.5.3.2	Minimum requirement for IAB-DU type 1-O and IAB-MT type 1-O

The requirements in clause 9.5.3.2 in TS 38.104 [TBD] apply to IAB-DU type 1-O and IAB-MT type 1-O.

9.5.3.3	Minimum requirement for IAB-DU type 2-O and IAB-MT type 2-O

The requirements in clause 9.5.3.3 in TS 38.104 [TBD] apply to IAB-DU type 2-O and IAB-MT type 2-O.

<End of the TP>