3GPP TSG-WG RAN4 Meeting #95-e	R4-2006291

# Electronic Meeting, 25 May - 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0230 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for suppporting Tx switching between two uplink carriers in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Time mask needs to be specified for Tx switching between two uplink carriers for EN-DC scenario . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UE time mask requirements in new section 6.3B.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is not allowed to support Tx switching between two uplink carriers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.3B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change >

6.3B.4	Output power dynamics for switching between two uplink carriers

6.3B.4.1	Time mask for UE supporting RF chain switching between two uplink carriers

In addition to the requirements in 6.3B.0, the time mask specified in this sub-clause is applicable for UE supporting RF chain switching between two uplink carriers configured on E-UTRA band and NR bands respectively. And it is only applicable when Tx Chain is switching between E-UTRA carrier 1 with one transmit antenna connector and NR carrier 2 with two transmit antenna connectors.

The switching period X indicated by UE capability uplinkTxSwitchingPeriod is located in NR carrier as shown in Figure 6.3B.4.1-1.

Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

The requirements apply for the case of co-located and synchronized network deployment.

< Start of change >