3GPP RAN WG4 Meeting #95-e		R4-2006337

Electronic meeting, 25th May – 5th June 2020

Agenda item:	8.18.2

Source:	Apple Inc, Comcast

Title:	LTE/NR spectrum sharing in band 48/n48 frequency range (UL shift)

WI/SI:	NR_n48_LTE_48_coex-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

Dynamic spectrum sharing is an important feature that allows for sharing existing spectrum between the LTE and NR carriers, thus enabling smoother transition from LTE and faster adoption of NR. Amongst a number of prerequisites for running LTE and NR within the same frequency band, either FDD or TDD, an operator has to ensure that both LTE and NR raster are aligned and that the NR synchronization blocks do not overlap with LTE reference signals. As an example, the following WI was agreed to enable dynamic spectrum sharing in band 41/n41 frequency range [1].

After the RAN#86 meeting, a new WI was agreed [2] aiming to analyse and introduce, if needed, changes to support dynamic spectrum sharing in band 48/n48 frequency range. During the RAN4#94 meeting, a number of companies expressed their view on the required modifications [5]-[9], but no consensus was reached.

From the agreed WI perspective, the following issues have to be addressed: channel raster, UL shift, synchronisation pattern. In this discussion paper we elaborate further on the channel raster and required changes to enable alignment of the LTE and NR sub-carrier grids.  In this discussion paper we elaborate further on the UL shift for band the DSS feature in 48/n48 and its need for various network deployments.

# 2	UL shift for LTE/NR spectrum sharing in band 48/n48

## 2.1	Background

One of the prerequisites for dynamic spectrum sharing between the LTE and NR carriers is the system ability to align sub-carrier grids of the LTE and NR systems. To align UL sub-carrier grids, an additional higher layer parameter was added to the system information to indicate UL channel raster shift. As can be seen from Figure 2-1, having a common and aligned sub-carrier grid, the base station can decide flexibly how many RBs should be allocated for LTE or NR UEs. And if both LTE and NR use 15kHz SCS, then no inter-numerology guard-band is needed.

Figure 2-1: Exemplary sub-carrier grid with 15kHz LTE and 15kHz NR RBs.

However, as follows from TR 38.822 and TS 38.101-1 (see also Annex A), this parameter is mandatory only for the FDD and SUL bands. In other words, a UE camped on the TDD bands is not anticipated to act on the corresponding parameter if it is signalled by the network. The problem was firstly identified for bands 41/n41 for LTE/NR spectrum sharing WI [1], and RAN WG4 agreed to introduce a new band [2] to circumvent around the fact that channel raster shift is not a mandatory parameter for the NR TDD bands and to introduce new channel raster values. Referring to Table 5.2-1 from TS 38.101-1 (highlighted in yellow for the sake of clarity), band n90 was introduced which is effectively the same band as n41, but the network deploying LTE and NR on the same carrier will use band n90 to prevent legacy terminals from camping on that carrier. In turn, a UE supporting band n90 has to support UL channel raster shift parameter.

Referring to the agreements from the CBRS Alliance, 30kHz SCS is the mandatory supported configuration. In other words, dynamic spectrum sharing for the LTE and NR carriers should deal with the mixed numerology case. According to RAN WG4 specifications, there are no particular requirements for the mixed-numerology deployments, and as Figure 2-2 presents, a particular network implementation may follow different strategies. If the network uses the common sub-carrier grid spanning over LTE and NR allocations, then it would be quite easy to ensure that inter-numerology guard band is either 15kHz or 30kHz SCS RB, or even zero RBs. The latter case is also feasible because even though 15kHz and 30kHz numerologies are not orthogonal, it will just cause additional emission to the neighbouring RB. And since the scheduling is done by the base station, the latter will be aware of it. As an example, if LTE and NR transmissions do not take 100% of UL resources, then it is quite natural to make an inter-numerology guard-band as it comes for free. For the 100% load scenario, it can be up to the network implementation and scheduling whether to schedule all RBs catering for better spectrum utilization (e.g. best effort service) or avoid unnecessary emissions to the neighbouring RB.

Figure 2-2: Exemplary sub-carrier grid for 15kHz LTE and 30kHz NR RBs.

Thus, enabling UL shift on band n48 will allow sub-carrier grid alignment which in turn can be used by the network for better resource utilization. And even though it can be viewed as a marginal optimization for a large carrier size, it will be a more noticeable improvement for 5 or 10MHz carriers which can be allocated by the SAS entity, especially accounting for the fact that 30kHz SCS RB will provide worse spectrum utilization than 15kHz SCS.

Observation 1a:	Due to the dynamic nature of allocated spectrum on band n48, the SAS entity may allocate a small channel, e.g. as small as 5MHz, for which 15kHz SCS would most likely be used.

Observation 1b:	UL shift is needed for 15kHz SCS deployments to align sub-carrier grids between LTE and NR thus achieving better resource utilization.

Observation 1c:	UL shift can be considered as not essential feature for 30kHz SCS if some inter-numerology guard band is always used by the network.

## 2.2	Potential solutions

As a summary of our observations, our view is that it is possible to contemplate further two major approaches: a) we keep the UL shift as an optional functionality for the UE side; or b) we mandate UL shift functionality for the UE side (either only for 15kHz SCS or for both 15/30kHz SCS).

a)	UL shift is optional for UE. With this approach no further changes will be needed in RAN WG4. However, as quickly discussed during RAN4#94bis meeting, it is not entirely clear what the overall system behaviour will be in a situation when the network enables UL shift, but a UE does not support it. On the one hand, there is nothing that prevents a UE from camping on that cell; but it does not make much sense either because UL grid will be shifted. The most straightforward solution would be to prevent a UE from camping on the cell if its system information indicates UL shift and a UE does not support. However, this behaviour is not captured now in RAN WG2 specifications, i.e. there is no rule for the UE side to consider the cell as barred if a UE does not support UL shift, but the network activates it.

b)	UL shift is mandatory for UE. UL shift can be considered as a mandatory feature at least for 15kHz SCS. Following the same considerations as for other FDD bands and TDD band 41/n41, enabling UL shift will help DSS deployments for narrow channels that can be allocated by the SAS entity and for which an operator will apply 15kHz SCS. At the same time, we can keep UL shift as an optional feature for 30kHz SCS. This approach is in line with the understanding of the majority of companies that if some inter-numerology guard band is anyway required, then there is no strong need to align sub-carrier grids.

Referring back to the approaches a) and b), one can see that they have a common denominator. Irrespective of the fact whether UL shift remains purely optional or mandated only for 15kHz SCS, it is better to clarify UE behaviour for the case when a UE camps on the cell, configuration of which enables UL shift for the given SCS, but a UE does not support.

Proposal 1a:	Ask RAN WG2 to clarify UE behaviour when a UE camps on the cell with enabled UL shift.

Proposal 1b:	If a UE does not support UL shift, then it can consider the cell as barred if the cell configuration has enabled UL shift.

As for the optional versus mandatory support of the UL shift parameter, we encourage companies to explore the option of making it mandatory only for 15kHz SCS deployments. It will not impact 30kHz SCS deployments or UE implementations.

Proposal 2:	Consider introduction of UL shift as a mandatory UE feature only for 15kHz SCS (on band n48).

# 3	Conclusions

In this discussion paper we have presented our further views on the need of the UL shift parameter for dynamic spectrum sharing in band 48/n48. As a summary of our paper, our proposals are:

Proposal 1a:	Ask RAN WG2 to clarify UE behaviour when a UE camps on the cell with enabled UL shift.

Proposal 1b:	If a UE does not support UL shift, then it can consider the cell as barred if the cell configuration has enabled UL shift.

Proposal 2:	Consider introduction of UL shift as a mandatory UE feature only for 15kHz SCS (on band n48).

# 4	References

RP-182883, "New WI proposal: LTE/NR spectrum sharing in Band 41/n41", KDDI Corporation

RP-192427, "New WID: LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

R4-2003175, "DSS in LTE/NR band 48/n48", Samsung

R4-2003212, "On LTE/NR dynamic spectrum sharing for band 48/n48", Intel Corporation

R4-2003336, "LTE/NR spectrum sharing in band 48/n48", Ericsson

R4-2005035, "Study of DSS in band 48/n48 channel raster", CableLabs

R4-2004396, "Views on dynamic spectrum sharing between LTE band 48 and NR band n48", Google Inc.

R4-2004507, "Views on band 48/n48 spectrum sharing", Nokia, Nokia Shanghai Bell

R4-2004686, "LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

Annex A (excerpt from TS 38.101-1)

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n303 | 2305 Mhz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band [n90] is in accordance with that for Band n41; a UE supporting Band [n90] shall meet the requirements for Band n41. |  |  |  |

Annex B (excerpt from TS 38.101-1):

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| [n90] | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |

Annex C (excerpt from TS 38.101-1):

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For SUL bands, for the uplink of all FDD bands defined in Table 5.2-1 and for Band [n90]

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in Section 5.4.2.2. The applicable entries for each operating band are defined in Section 5.4.2.3

Annex D (excerpt from TS 38.331):

#### FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2).

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15]. Value in dBm.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise. |