3GPP TSG-RAN WG4 Meeting # 95-e    	                                         R4-2006559

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.8.2.1.5

Source:	Intel Corporation

Title:	Link-level simulation Assumptions for UE Rx-Tx time difference measurement

Document for:		Discussion

# Introduction

In the previous RAN4 meeting it was agreed that both system-level and link-level simulation studies were necessary for NR positioning requirements [1]. In this contribution, we propose a set of initial link-level simulation assumptions for UE Rx-Tx time difference measurements.

# Simulation Assumptions

In principle, UE RX-TX time difference depends on UE timing to receive DL reference signal and UE Tx timing to transmit UL subframe by itself as illustrated in the figure below. That is the estimation error for UE Rx-Tx time difference can be determined by UE Rx timing (ToA) only as the transmit timing is perfectly known by UE itself.

Figure 1. RSTD measurement in NR

Observation 1: When UE measuring UE Rx-Tx time difference, the transmission timing shall be perfectly known by UE.

Therefore, we can proposed the link level simulation assumption as.

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> (optional) Note: Async case is optional, and companies submitting results for async case are encouraged to provide Es/Noc conditions for the 3 cells. |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-C (300 ns delay spread, 100 Hz), TDL-A (30 ns delay spread, 5Hz), TDL-B (100 ns delay spread, 200Hz) | TDL-C (60 ns delay spread, 300 Hz) |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13) for FR1 and FR2 (-3, -13, -13) for FR2 only | (-6, -13, -13) for FR1 and FR2 (-3, -13, -13) for FR2 only |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| UE transmission timing | Perfectly known by UE |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: - 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: - 32(50MHz),64(100MHz), 128 (200MHz) |
| Comb | Comb-4, comb-6, comb-2 (with muting), comb-12 |  |
| DL-PRS-ResourceRepetitionFactor | 1, 2, 4, 16 |  |
| PRS-ResourceTimeGap | 1, 4 | 1, 16 |
| PRS muting | No muting (comb-4, comb-6, comb-12), muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-Uncertainty | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |

# Performance Metric

At least the following performance metric are to be provided for Rx-Tx time difference:

- UE Rx-Tx time difference error CDFs for 3 cells

Proposal 1: The following performance metric are proposed for link-level studies:

- CDF of UE Rx-Tx time difference estimation error

# Conclusion

In this contribution, the view on the link level simulation assumption for UE Rx-Tx time difference for NR positioning measurement is provided.

Observation 1: When UE measuring UE Rx-Tx time difference, the transmission timing of UL SRS shall be perfectly known by UE.

Proposal 1: The following performance metric are proposed for link-level studies:

- CDF of UE Rx-Tx time difference estimation error

# References

- R4-1912793, “ Ad-hoc minutes for NR Positioning RRM”

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”