3GPP TSG-RAN WG4 Meeting #95-e	R4-2007196

# Electronic Meeting, 25th May – 5th June, 2020

Title: 	Discussion and simulation results for URLLC BS PUSCH demodulation requirements for high reliability with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2.2

Document for:	Discussion

Background

RAN4 has agreed that the BLER target 10^-5 and the higher BLER target will be discussed separately when introduce URLLC BS PUSCH demodulation requirements for high reliability, the higher BLER target was discussed on #94 meeting and 10^-2 was agreed as the BLER target. In this paper, the PUSCH demodulation requirements for high reliability with target BLER 10^-2 are discussed. From #94bis e-meeting discussion results, some agreements for FR1 were made and there are still some open issues:

Agreements:

- PUSCH aggregation factor: n2 for TDD 30 kHz SCS with TDD pattern 7DS2U.

- Number of PRBs: Full bandwidth.

- SCS/CBW: 15 kHz for 10 MHz, 30 kHz for 40 MHz

- Test applicability for different SCS and channel bandwidth: Reuse the test applicability rules defined for NR Rel-15 PUSCH performance requirements

- Not define requirements with DFT-s-OFDM

- Whether to define requirements for FR2: Keep it open meanwhile prioritize discussion on introducing FR1 requirements in Q2; and interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios in FR2 with ultra-low BLER and/or higher BLER for high reliability and low latency

Open issues:

- PUSCH aggregation factor

  - 15 kHz TDD with pattern DDDSU:

    - Option 1: n2

    - Option 2: n8

- SCS/CBW (15 KHz/10 MHz, 30 KHz/40 MHz have been agreed)

  - Option 1: 15 KHz for 5/10 MHz, 30 KHz for 10/40 MHz

  - Option 2: Only 15 KHz/10 MHz, 30 KHz/40 MHz

- Whether to clarify the safety statement

  - Option 1: No

  - Option 2: Yes

    - Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence

- Whether to define requirements for FR2

  - Option 1: No

  - Option 2: Yes

- Test applicability rule for FR1 and FR2 (only if FR2 is defined)

  - Option 1: If BS supports both FR1 and FR2, the tests shall be done for either FR1 or FR2

  - Option 2: If BS supports both FR1 and FR2, the tests shall be done both

  - Option 3: Which tests related to FR1 and FR2 be tested shall be based on BS declaration: [FR1], [FR2], [FR1&FR2]

- SCS/BW for FR2 (only if FR2 is defined)

  - 60 KHz:

    - Option 1: 50 MHz

    - Option 2: 50 MHz and 100 MHz

  - 120 KHz

    - Option 1: 100 MHz

    - Option 2: 50 MHz and 100 MHz In this paper, the remained open issues are discussed and our views are provided.

In this paper, the simulation results are provided for PUSCH high reliability.

Simulation

Simulation assumption

According to the WF [1], the test parameters according to the agreements of #94 and #94bis e-meeting are concluded in table below:

Table 2-1: Parameters for BS PUSCH performance requirements for high reliability

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | n2 for 30 kHz SCS FFS 15 kHz SCS |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Propagation condition |  | TDLB100-400 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Maximum number of HARQ transmissions |  | 4 |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |

Simulation result

- TDD

Based on the parameters listed in Table 2-1, the BLER vs SNR curve for TDD pattern “7D1S2U” is shown in figure below.

Figure 2-1: BLER vs SNR for PUSCH TDD pattern “7D1S2U”

Figure 2-1 shows that the SNR value is around -8.5dB with target BLER=0.01.

Discussion

- PUSCH aggregation factor for 15 kHz TDD with pattern DDDSU:

Firstly, we do not think it is a practical scenario in real network to configure n8 for TDD pattern of DDDSU. We do not think this is a good idea to verify the repetition functionality regardless the practical use case. Secondly, if n8 is configured for “DDDSU”, the delay between two repetitions is too large. Although this test is for high reliability, but we still need to consider the latency for the real use cases.

Proposal 1: PUSCH aggregation factor of n2 for 15 kHz TDD with pattern “DDDSU”.

Proposals

By discussing the open issues for URLLC BS PUSCH performance requirements for high reliability, we have following proposal:

Proposal 1: PUSCH aggregation factor of n2 for 15 kHz TDD with pattern “DDDSU”.

Reference

[1] R4-2005528, Way forward on NR URLLC BS performance requirements, #94bis, Huawei, Hisilicon.