3GPP TSG-RAN WG4 Meeting #95-e	R4-2007228

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0051 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Clarification on EPRE ratio for DL channel signal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The definition of EPRE power ratio for downlink physical channels or reference signals are unclear due to the pre-coding or beamforming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification to make the definition of EPRE power ratio clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

Annex C (normative):
Downlink physical channels

# C.1	General

This annex specifies the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

# C.2	Setup (Conducted)

Table C.2-1 describes the downlink Physical Channels that are required for connection set up.

Table C.2-1: Downlink Physical Channels required for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

# C.3	Connection (Conducted)

The following clauses, describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

## C.3.1	Measurement of Performance requirements

<Editor's note: OCNG for DMRS is FFS in Annex A.>

Table C.3.1-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value (Note 2) | Mapping to antenna port(s) |
| --- | --- | --- | --- |
| SSS transmit power | W | Test specific | Single port 4000 |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | Single port 2000 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH to SSS | dB | -10*log(N) (Note 3) | Test specific for PDSCH demodulation requirement |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | -10*log(L) (Note 4) | Test specific for NZP-CSI-RS configuration |
| EPRE ratio of PDSCH OCNG to SSS | dB | -10*log(N) (Note 3) | Same as PDSCH |
| EPRE ratio of PDCCH OCNG to SSS | dB | 0 | Same as PDCCH |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on "Number of DM-RS CDM groups without data" and "DMRS Type" parameters specified for each test. Note 2: The value is the energy of per RE for a single antenna port before pre-coding. Note 3: N {1,…, Number of layers} is the number of antenna ports of PDSCH Note 4: is the CDM group size of NZP CSI-RS. |  |  |  |

or

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific (Note 2) |
| Ratio of PSS EPRE to SSS EPRE | dB | 0 |
| Ratio of PBCH EPRE to SSS EPRE | dB | 0 |
| Ratio of PBCH EPRE to PBCH DMRS EPRE | dB | 0 |
| Ratio of PDCCH EPRE to SSS EPRE | dB | 0 |
| Ratio of PDCCH EPRE to PDCCH DMRS EPRE | dB | 0 |
| Ratio of PDSCH EPRE to SSS EPRE | dB | 0 (Note 3) |
| Ratio of PDSCH EPRE to PDSCH DMRS EPRE | dB | Test specific (Note 1) |
| Ratio of NZP CSI-RS EPRE to SSS EPRE | dB | 0 (Note 4) |
| Ratio of PDSCH OCNG EPRE to SSS EPRE | dB | 0 (Note 3) |
| Ratio of PDCCH OCNG EPRE to SSS EPRE | dB | 0 |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on "Number of DM-RS CDM groups without data" and "DMRS Type" parameters specified for each test. Note 2: Value can be derived from ss-PBCH-BlockPower provided by higher layers. Note 3: PDSCH EPRE is the energy of total PDSCH ports multiplexing on one subcarrier of one OFDM symbol Note 4: NZP CSI-RS EPRE is the energy of all CSI-RS ports multiplexed on one subcarrier of one OFDM symbol. |  |  |

# C.4	Setup (Radiated)

Table C.4-1 describes the downlink Physical Channels that are required for connection set up.

Table C.4-1: Downlink Physical Channels required for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

PTRS

# C.5	Connection (Radiated)

The following clauses, describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

## C.5.1	Measurement of Receiver Characteristics

<Editor's note: OCNG for DMRS is FFS in Annex A.>

Table C.5.1-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table C.5.1-1: Downlink Physical Channels transmitted during a connection (TDD)

| Parameter |  | Unit |  | Value (Note 2) |  |  |  |  | Mapping to antenna port(s) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSS transmit power |  |  | W |  |  | Test specific |  | Single port 4000 |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | Single port 2000 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH to SSS | dB | -10*log(N) (Note 3) | Test specific for PDSCH demodulation requirement |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | -10*log(L) (Note 4) | Test specific for NZP-CSI-RS configuration |
| EPRE ratio of PTRS to PDSCH | dB | 0 | Test specific, the lowest indexed DM-RS antenna port assigned for the PDSCH |
| EPRE ratio of PDSCH OCNG to SSS |  | dB |  | -10*log(N) (Note 3) |  |  |  |  | Same as PDSCH |  |
| EPRE ratio of PDCCH OCNG to SSS | dB |  |  |  | 0 |  | Same as PDCCH |  |  |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on "Number of DM-RS CDM groups without data" and "DMRS Type" parameters specified for each test. Note 2: The value is the energy of per RE for a single antenna port before pre-coding. Note 3: N {1…Number of layers} is the number of antenna ports of PDSCH. Note 4: is the CDM group size of NZP CSI-RS. |  |  |  |

or

Table C.5.1-1: Downlink Physical Channels transmitted during a connection (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific (Note 2) |
| Ratio of PSS EPRE to SSS EPRE | dB | 0 |
| Ratio of PBCH EPRE to SSS EPRE | dB | 0 |
| Ratio of PBCH EPRE to PBCH DMRS EPRE | dB | 0 |
| Ratio of PDCCH EPRE to SSS EPRE | dB | 0 |
| Ratio of PDCCH EPRE to PDCCH DMRS EPRE | dB | 0 |
| Ratio of PDSCH EPRE to SSS EPRE | dB | 0 (Note 3) |
| Ratio of PDSCH EPRE to PDSCH DMRS EPRE | dB | Test specific (Note 1) |
| Ratio of NZP CSI-RS EPRE to SSS EPRE | dB | 0 (Note 4) |
| Ratio of PTRS EPRE to PDSCH EPRE | dB | Test specific (Note 5) |
| Ratio of PDSCH OCNG EPRE to SSS EPRE | dB | 0 (Note 3) |
| Ratio of PDCCH OCNG EPRE to SSS EPRE | dB | 0 |
| Note 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on "Number of DM-RS CDM groups without data" and "DMRS Type" parameters specified for each test. Note 2: Value can be derived from ss-PBCH-BlockPower provided by higher layers. Note 3: PDSCH EPRE is the energy of total PDSCH ports multiplexing on one subcarrier of one OFDM symbol. Note 4: NZP CSI-RS EPRE is the energy of all CSI-RS ports multiplexed on one subcarrier of one OFDM symbol. Note 5: Value is derived from Table 4.1-2 in TS 38.214 [12] based on the “epre-Ratio” and “The number of PDSCH layers” parameters specified for each test. |  |  |

<< End of change >>