3GPP TSG-RAN WG4 Meeting #95-e	R4-2007384

Electronic Meeting, 25 May – 5 June 2020

Agenda Item:	6.17.2.1

Source:	Ericsson

Title:	Release independence and applicability rule for NR HST demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#94-e-Bis discussed the UE demodulation requirements defined in Rel-16 NR HST are whether release independent from Rel-15 or not [1]. RAN4#94-e also discussed the applicability rule and capabilities. We discuss our view on these issues.

For information, Table 1 summarizes the maximum Doppler shift assumed for PDSCH demodulation requirements in RAN4.

Table 1	Maximum Doppler shift assumed for PDSCH demodulation requirements.

|  | Non-HST | HST single tap | HST fading | HST-SFN (joint transmission) |
| --- | --- | --- | --- | --- |
| Rel-15 PDSCH demodulation | 5/10/100 Hz for 15/30kHz | 700Hz for 15kHz 1000Hz for 30kHz | 400Hz with TDLB100 | N/A |
| Rel-16 PDSCH demodulation | 5/10/100Hz for 15/30kHz | 870/972/1250Hz for 15kHz 1667Hz for 30kHz | 600Hz for 15kHz 1200Hz for 30kHz (with TDLC300) | 851/870Hz for 15kHz 1667Hz for 30kHz |

# 2	Discussion

## 2.1	Applicability rule

Applicability for different channel models - FFS the applicability rule between HST-SFN, HST single tap and HST multi-path fading performance test cases - FFS whether to define to Applicability rule between different Doppler frequencies for the same channel model

RAN4#94-e-bis discussed the applicability rule between HST-SFN, HST single tap and HST multi-path fading performance requirements. In our understanding, the HST single tap verifies the high Doppler shift tracking and optional HST-SFN requirements verifies the high Doppler shift tracking per TRP. We could understand the argument Rel-16 HST single tap is not applicable for UE that passes HST-SFN. However RAN4 has already defined HST single tap with lower Doppler shift in Rel-15 and Rel-16 UE anyway need to pass this Rel-15 HST single tap requirement. As a compromise we propose the following applicability rule.

Proposal 1:

- Rel-16 HST-SFN test is applicable for Rel-16 UE capable of DemodulationEnhancementforNR

- Rel-16 HST single tap test is applicable for Rel-16 UE.

- Rel-15 HST single tap test is not applicable for UE that passes Rel-16 HST single tap test.

Multi-path fading channel test verifies UE demodulation performance under frequency selective fading channel, and we don’t not think it could be replaced by HST-SFN and/or HST single tap. On the other hand, since Rel-16 HST multi-path fading test reuse the same setup (e.g., TRS, DMRS configurations) as Rel-15 TDLB100-400 test case except MCS, we think UE can skip to lower Doppler scenarios defined in Rel-15 if UE pass the Rel-16 test cases.

Proposal 2:

- Rel-15 multi-path fading with TDLB100-400 is not applicable for UE that passes Rel-16 multi-path fading tests (TDLC300-600 for FDD and TDLC300-1200 for TDD).

## 2.3	UE capabilities/features

UE capabilities/features - For HST-SFN - Introduce per-UE capability to support enhanced demodulation performance for HST-SFN joint transmission scheme with velocity up to 500km/h. (Agreement in RAN4#93) - For HST fading channel requirements, take it as mandatory requirements for Rel-16 and no capability signaling will be introduced. - For HST single Tap channel demodulation requirements, no capability signaling will be introduced - FFS whether requirements will be mandatory or optional - Further discuss whether feature list will be introduced for HST fading channel, and HST single Tap

Rel-16 HST single tap test and HST multi-path fading test are based on the corresponding Rel-15 test setup, and we don’t think any new UE performance features are required as well as Rel-15 LTE HST UE demodulation requirements. Therefore Rel-16 HST single tap test and multi-path fading test should be also mandatory requirements for Rel-16 UEs.

Proposal 3: No RAN4 UE performance features are introduced for Rel-16 HST single tap and Rel-16 HST multi path fading tests.

Proposal 4: Rel-16 HST single tap tests and Rel-16 multi-path fading tests are mandatory for Rel-16 UEs.

## 2.1	Release independent

Release independent - Whether Rel.16 HST requirements can be release independent from Rel-15 - HST Multi-path fading tests can be release independent from Rel-15 - HST single tap tests can be release independent from Rel-15 if HST RRM signaling is not provided in the demodulation test - Further study whether HST-SFN tests can be release independent from Rel-15 - send LS to RAN2 to check ‘early implementation approach is also applicable for NR

3GPP defines some LTE performance requirements introduced in Rel-N spec as the release independent from Rel-M (M<N). One example is HST-SFN requirements introduced in Rel-14. It is defined release independent from Rel-13 if ‘early implemented’ UE supports the capability signaling.

RAN4#94-e-Bis discussed the same approach is applicable for NR HST-SFN and agreed to send LS to RAN2 [2]. RAN4 need to wait for LS response from RAN2 whether ‘early implementation’ approach can be applicable for NR HST-SFN.

Proposal 5: RAN4 wait for the LS response from RAN2 to decide whether Rel-16 HST-SFN requirement is released independent from Rel-15 or not.

For Rel-16 HST single tap and multi-path fading tests, this is based on Rel-15 features and it should not assume any signaling or features introduced in Rel-16. If RAN4 will define these cases are release independent from Rel-15, we think these tests should be optional for earlier release UEs otherwise some UEs in the market may not pass the test.

Proposal 6: If Rel-16 HST single tap test is defined as release independent from Rel-15, this requirement should be optional for Rel-15 UEs.

Proposal 7: If Rel-16 HST multi-path fading test is defined as release independent from Rel-15, this requirement should be optional for Rel-15 UEs.

# 3	Summary

Proposal 1:

- Rel-16 HST-SFN test is applicable for Rel-16 UE capable of DemodulationEnhancementforNR

- Rel-16 HST single tap test is applicable for Rel-16 UE.

- Rel-15 HST single tap test is not applicable for UE that passes Rel-16 HST single tap test.

Proposal 2:

- Rel-15 multi-path fading with TDLB100-400 is not applicable for UE that passes Rel-16 multi-path fading tests (TDLC300-600 for FDD and TDLC300-1200 for TDD).

Proposal 3: No RAN4 UE performance features are introduced for Rel-16 HST single tap and Rel-16 HST multi path fading tests.

Proposal 4: Rel-16 HST single tap tests and Rel-16 multi-path fading tests are mandatory for Rel-16 UEs.

Proposal 5: RAN4 wait for the LS response from RAN2 to decide whether Rel-16 HST-SFN requirement is released independent from Rel-15 or not.

Proposal 6: If Rel-16 HST single tap test is defined as release independent from Rel-15, this requirement should be optional for Rel-15 UEs.

Proposal 7: If Rel-16 HST multi-path fading test is defined as release independent from Rel-15, this requirement should be optional for Rel-15 UEs.

# References

- R4-2005532, “WF on UE demodulation for NR HST”, CMCC.

- R4-2005533, “LS on supporting Rel-16 NR HST from Rel-15 UEs”, CMCC.