3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007617

Electronic Meeting, 25 May – 5 June, 2020

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-21-11: Including band combinations 46-66_n5

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_46E-66A_n5A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_46-66_n5

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 46-66_n5 | CA_46-66 | n5 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A | DC_66A_n5A | CA_46A-66A CA_46C-66A CA_46D-66A CA_46E-66A | n5A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_46-66_n5, the TIB,c and RIB,c values are reused from LTE combination CA_5-46-66 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n5 | 66 | 0 |
|  | n5 | 0 |

### 5.1.x.4	REFSENS requirements

There is IMD4 impact from DC_66_n5 UL affecting band 46 DL. MSD value is derived from CA_1-5-40 which have IMD4 impact to band 40 from UL_1-5, and from DC_12_n7-n78 which have IMD4 impact to band n78 from UL_12_n7.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-200271, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon