3GPP TSG-RAN WG4 Meeting # 95-e 	                             R4-2007683

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:	6.5.3.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on BFD and CBD requirement for IAB-MT

Document for:	Discussion

- Introduction

In the previous RAN4 meetings, the RRM measurement for IAB-MT were discussed. There are some remaining issues about BFD and CBD. We further investigate these issues and present views in this paper.

- Discussion

In theRAN4 94e and RAN4 94-e-bis meetings, the requirements for BFD and CBD were discussed with following agreements:

RAN4 94e: Agreement: BFD/CBD requirements for IAB-MTs are defined for no-DRX mode only. Rel-15 BFD OOS and IS BLER levels are reused for IAB-MTs. RAN4 94-e-bis Agreement: The CSI-RS requirements shall apply only for CSI-RS density 3 over the bandwidth ≥ 24 PRBs. Agreement: The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based CBD and CSI-RS based CBD - Scheduling availability during CBD

As discussed in RLM in RAN4 94-e-meeting, the sharing factor P is also necessary since measurement gap can be configured for IAB-MTs. So it is suggested to adopt the same principle for BFD and CBD to keep the sharing factor P unchanged,

Proposal 1: There is no need to remove the sharing factor P in BFD and CBD requirements for IAB-MT.

For the evaluation period requirements for BFD and CBD, as discussed in RAN4 94e meeting, it is reasonable to reuse the similar requirements as Rel-15 UE provided that the sharing factor is kept unchanged.

Proposal 2: The requirements of evaluation period of BFD/CBD for Rel-15 UE can apply for IAB-MT.

Another issues is about the beam sweeping factor for FR2. It can be observed that the beam sweeping factor N = 8 in the existing TS 38.133 for RLM and Link recovery procedure regardless of the UE type. The same sweeping factor also applies to UE PC1, which is the also fixed node. Thus for fixed IAB node, it is suggested to adopt the same beam sweeping factor which is 8 for RLM and Link recovery in TS 38.174.

Proposal 3: Adopt the same beam sweeping factor which is 8 for RLM and Link recovery for FR2 in TS 38.174.

- Conclusion

Proposal 1: There is no need to remove the sharing factor P in BFD and CBD requirements for IAB-MT.

Proposal 2: The requirements of evaluation period of BFD/CBD for Rel-15 UE can apply for IAB-MT.

Proposal 3: Adopt the same beam sweeping factor which is 8 for FR2 in TS 38.174.

References

[1] R4-2005322, WF on RLM requirements and sharing factor in RLM/BFD/CBD evaluation for IAB-MTs