3GPP TSG-RAN WG4 Meeting #95-e		R4-2007922

Electronic meeting, 25th May – 5th June, 2020

Agenda item:	6.17.2.1.4

Source:	Ericsson

Title:	Simulation results for HST Multipath fading channels

Document for:	Discussion

# Introduction

In this paper we provide simulation results based on the assumptions found in WF [1].

# Simulations

## Test setup

The simulation assumptions from [1].

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13; based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

Based on the simulation assumptions, we simulate FDD according to it and for TDD only transmitting in the 7 downlink slots. Simulation results shown in Figure 2-1, and 2-2 for FDD, and TDD respectively.

Figure 2-1: FDD simulations of Multi-path channel according to simulation assumptions

Figure 2-2: TDD simulations of Multi-path channel with special slot transmission turned off for PDSCH

# Results

In Table 3-1, and 3-2 the results are presented for FDD, and TDD respectively.

Table 3-1 SNR at 70% throughput for FDD test cases

| Test | BW / SCS Max Doppler | Antennas | SNR @ 70% maximum Throughput |
| --- | --- | --- | --- |
| 1 | 10MHz / 15kHz 600Hz | 2Tx2Rx | 6.22 |
| 2 |  | 2Tx4Rx | 2.70 |

Table 3-2 SNR at 70% throughput for TDD test cases

| Test | BW / SCS Max Doppler | Antennas | SNR @ 70% maximum Throughput |
| --- | --- | --- | --- |
| 1 | 40MHz / 30kHz 1200Hz | 2Tx2Rx | 7.03 |
| 2 |  | 2Tx4Rx | 3.5 |

# Conclusions

In this contribution we have evaluated HST Multi-path fading demodulation performance according to the agreed parameters found in

# References

- R4-2005532 “WF on UE demodulation for NR HST”, CMCC, RAN4 #94-e-Bis, April 2020.