3GPP TSG-RAN WG4 Meeting # 95-e 															R4-2008069

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei, HiSilicon, CMCC

Title: 	TP to TR 37.716-00-00: SUL_n41A-n95A

Agenda Item:	8.8.2

Document for:	Approval

- 1 Introduction

The WID for SUL band combinations was updated in RAN #87 meeting. This contribution provides a TP for SUL_n41A-n95A for TR 37.716-00-00.

- 2 Discussion

In last meeting, companies proposed to check the MSD issue which is caused by cross band isolation for SUL_n41A-n95A. In this paper, we’d like to discuss and address this issue. We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1.

Figure 1 main/diversity antenna topology

Figure 2 Frequency—Insertion loss for n95 post PA filter

The absolute attenuation of n95 post PA filter can be shown in figure 2. We can find the minimum attenuation value for band n41 frequency range is about 28dB. Thus, table 1 summarize the MSD analysis results for SUL_n41-n95.

Table 1 MSD calculation for victim Rx of band n41

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n95 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n95 PA output noise power at n41 freq | dBm/Hz | （15MHz UL） | -130 | -130 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| B41 Rx bandwidth | MHz | 10 |  |  |
| n95 PA output noise power |  |  | -60 | -60 |
| n95 SAW TX to COM port on n41 RX freq | dB | 28 |  |  |
| Diplexer loss, dB |  | 1.5 |  |  |
| diplexer out of band isolation | dB | 10 |  |  |
| n95 PA noise power at SAW- COM port |  |  | -98 | -109.5 |
| front end loss |  | 4 |  |  |
| n95 PA noise power at ant port |  |  | -94 | -105.5 |
| noise figure dB | dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -165 |  |  |
| Thermal noise, dBm |  |  | -95 | -95 |
| TX IM2 noise dBm |  |  | -94 | -116 |
| totle noise level at ANT port, dBm |  |  | -89.53723 | -94.597663 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  |  | -90.53723 | -95.597663 |
| combined REFSENS(10MHz BW), dBm |  |  | -96.77653 |  |
| MSD | dB |  | -1.67653 |  |

Based on the analysis above, there is no need MSD which is caused by cross band isolation for SUL_n41-n95.

- References

[1]	RP-200273, “New WID on Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, RAN #87e..

- Text Proposal

<TP for TR 37.716-00-00>

- Scope

The present document is a technical report for SA NR Supplementary uplink (SUL), NSA NR SUL and NSA NR SUL with UL sharing from the UE perspective (ULSUP) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address NR SUL for the Rel-16 bands/band combinations in Table 1-1 to Table 1-4.

Table 1-1: Release 16 SUL bands

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n89 | 824 – 849 MHz | N/A | SUL |
| n95 | 2010 – 2015 MHz | N/A | SUL |

Table 1-2: Release 16 SA SUL band combinations

| SA SUL band combination | REL independent from |
| --- | --- |
| SUL_n79-n84 | Rel-15 |
| SUL_n41-n80 | Rel-15 |
| SUL_n41-n81 | Rel-15 |
| SUL_n41-n95 | Rel-15 |
| SUL_n77_n80 | Rel-15 |
| SUL_n77_n84 | Rel-15 |
| SUL_n79-n95 | Rel-15 |

Table 1-3: Release 16 NSA SUL band combinations

| NSA SUL band combination | REL independent from |
| --- | --- |
| DC_1_SUL_n78-n80 | Rel-15 |
| DC_7_SUL_n78-n80 | Rel-15 |
| DC_20_SUL_n78-n80 | Rel-15 |
| DC_8_SUL_n78-n80 | Rel-15 |
| DC_1_SUL_n77-n80 | Rel-15 |
| DC_3_SUL_n77-n84 | Rel-15 |
| DC_3_SUL_n78-n84 | Rel-15 |

Table 1-4: Release 16 NSA SUL with ULSUP band combinations

| NSA SUL with ULSUP band combination | REL independent from |
| --- | --- |
| DC_1_SUL_n79-n84 | Rel-15 |
| DC_3_SUL_n41-n80 | Rel-15 |
| DC_8_SUL_n41-n81 | Rel-15 |
| DC_3_SUL_n78-n80 | Rel-15 |
| DC_1-3_SUL_n78-n80 | Rel-15 |
| DC_3-7_SUL_n78-n80 | Rel-15 |
| DC_3-8_SUL_n78-n80 | Rel-15 |
| DC_3_20_SUL_n78-n80 | Rel-15 |
| DC_3_SUL_n77-n80 | Rel-15 |
| DC_1_SUL_n77-n84 | Rel-15 |

This TR contains a general part and specific band combination part. The actual requirements are added to the corresponding technical specifications.

< Unchanged sections omitted >

## 5.X	SUL_n41A-n95A

### 5.X.1	Operating bands

Table 5.X.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1 from 38.101-1) |
| --- | --- |
| SUL_n41-n95 | n41, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier |  |

### 5.X.2	Channel bandwidths per operating band

Table 5.X.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n95A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

### 5.X.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.X.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

### 5.X.5	MSD

For SUL_n41-n95, there is no harmonic or harmonic mixing product generated by Band n95 that may fall into the RX band of Band n41. Therefore, MSD due to harmonic or harmonic mixing is not needed.

Based on the cross band isolation analysis in R4-200xxxx, there is no need MSD for SUL_n41-n95.

### 5.X.6	∆TIB and ∆RIB values

For SUL_n41-n95, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n95 | n41 | 0 |
|  | n95 | 0 |

Table 5.X.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n95 | n41 | 0.2 |

<End of TP >